T-VER-P-METH-01-08

Green Hydrogen Production

Version 01

Scope: 01 - Energy industries

Scope: 05 - Chemical Industry

Effective from 24 September 2026

| 1. Methodology | Green Hydrogen Production |
| --- | --- |
| 2. Project Type | Renewable energy or fossil fuel replacement |
| 3. Scope | 01 - Energy industries 05 - Chemical industry |
| 4. Project Outline | This project is intended to reduce emissions through the production of green hydrogen as a substitute for hydrogen derived from fossil fuels. |
| 5. Applicability | 1. Green hydrogen production to replace conventional methods, such as hydrogen derived from fossil fuels, in the following forms: - Installation of new green hydrogen production systems (Greenfield), or - Production of green hydrogen to substitute existing systems (Replacement). 2. Eligible green hydrogen production systems include: - Electrolysis of water powered by renewable energy sources such as solar and/or wind energy. - Hydrogen production from biomethane, for example through reforming processes. |
| 6. Project Conditions | - In the case where the project developer produces green hydrogen from electrolysis, electricity consumed in the hydrogen production facility is only produced by renewable-energy sources. The ratio between electricity consumption from non-renewable sources and total energy consumption in the hydrogen production facility remains below 0.1 monitored annually. If this ratio exceeds the threshold, the project developer shall not be eligible to claim greenhouse gas emission reductions for the relevant crediting period. - In the case where the project developer produces green hydrogen from biomethane, renewable energy shall be the main energy source of the process. The ratio between electricity consumption from non-renewable sources and total energy consumption in the hydrogen production facility must remain below 0.1 monitored annually. If this ratio exceeds the threshold, the project developer shall not be eligible to claim greenhouse gas emission reductions for the relevant crediting period. - The project developer shall demonstrate that the biomethane used for green hydrogen production originates from sustainable waste sources, such as anaerobic wastewater treatment systems or municipal solid waste landfills with biogas capture. - In cases where the project activity involves the installation of a biogas upgrading system, the project developer shall account for project emissions and leakage emissions. - The project developer shall ensure that water use does not significantly affect community drinking water consumption, thereby guaranteeing that water used in the electrolysis process does not displace other essential uses. Verification shall be conducted during validation and at each crediting period renewal, based on project activity data and official sources. - Hydrogen consumption demands planned to be replaced or displaced by green hydrogen from the project activity shall be stated in the Project Design Document (PDD) prior to project implementation. - In cases of replacement installations, the baseline hydrogen production system shall not be relocated or reused outside the project boundary. |
| 7. Project Starting Date | The date on which the project developer (employer) and the contractor jointly signed the purchase and installation contract for the green hydrogen production system to be developed as a T-VER project. |
| 8. Definition | Green Hydrogen refers to low-carbon hydrogen production, generated either through electrolysis of water using electricity from renewable energy sources, or from biomethane. |
|  | Electrolysis means the process of passing direct current through water to separate water molecules into hydrogen gas and oxygen gas, based on electrochemical principles applied via electrodes. |
|  | Reforming refers to a chemical process that uses catalysts under heat and pressure to restructure hydrocarbons, such as methane, to produce syngas or subsequent fuels, including hydrogen gas. |
|  | Biomethane refers to biogas produced from the anaerobic digestion of organic matter (e.g., food waste, animal manure, wastewater, or municipal solid waste), which is then upgraded through a biogas upgrading process to achieve properties comparable to natural gas. |
|  | Renewable Energy refers to a type of alternative energy derived from sources that can be naturally replenished and reused. |
| Note | 1) In the event that a project developer installs a new green hydrogen production system (Greenfield) or implements green hydrogen production to replace an existing system (Replacement) and also utilizes the produced green hydrogen for purposes such as electricity or heat generation, the project developer may apply for both the green hydrogen production component and the utilization component. 2) Non-renewable energy power plants include those utilizing fossil fuel combustion and hydrogen produced from fossil fuels. |

Details of the voluntary emission reduction methodology for green hydrogen production

- Emission reduction activities

Table 1: Sources and Types of Greenhouse Gases

| Greenhouse Gas Emissions | Sources of Greenhouse Gases | Types of Greenhouse Gases | Description of Activities Associated with Greenhouse Gas Emissions |
| --- | --- | --- | --- |
| Baseline Emissions | Use of fossil fuels | CO2 | Hydrogen production from fossil fuels (natural gas) replaced by green hydrogen |
| Project Emissions | Use of electricity | CO2 | Purchase of electricity from the grid for use in the supporting systems of green hydrogen production |
|  | Use of fossil fuels | CO2 | Combustion of fossil fuels, including: - Heat generation for use in green hydrogen production processes - Electricity generation from backup generators |
|  | Hydrogen transportation | CO2 | Transportation of hydrogen to end‑users, considering only additional transportation, categorized as follows: - Use of fossil fuels for road transportation - Use of electricity for pipeline transportation |
|  | Physical leakage of hydrogen | H2 | Hydrogen leakage during transportation to end-users, which triggers indirect chain reactions and contributes to global warming. |
| Leakage Emissions | Biomethane production or biogas production | CO2 and CH4 | Purchasing or carrying biomethane/ biogas from producer located outside boundary |

- Applicability and Scope of Project

Project activity involving the installation of new green hydrogen production systems (Greenfield) or the replacement of existing systems (Replacement), utilizing either electrolysis powered by renewable energy or hydrogen production from biomethane (e.g., through reforming processes), to substitute hydrogen derived from fossil fuels.

The project boundary covers the site of the green hydrogen production system and all related activities associated with hydrogen production, including biogas upgrading systems (if applicable) and hydrogen transportation for utilization.

- Additionality

The project must demonstrate additionality beyond common practice by applying the ‘Criteria for Demonstration of Additionality under Premium Thailand Voluntary Emission Reduction (Premium T-VER) Program’ as prescribed by TGO.

- Baseline Scenario

When considering the determination of baseline data below business as usual (Below BAU), in accordance with the Guideline for the Development of Premium T‑VER Methodology (Version 02), the baseline shall take into account greenhouse gas emissions from hydrogen production using fossil fuels, which are replaced by green hydrogen generated under the project. For year 1 baseline data, existing actual or historical emissions shall be applied. From year 2 onwards during the crediting period, the downward adjusted baseline approach shall be adopted. Details are as follows.

4.1	Installation of new green hydrogen production systems (Greenfield),

For year 1 baseline data, emissions from hydrogen production using fossil fuels, which are replaced by green hydrogen, shall be determined using existing actual data with downward adjusted. For year 2 onwards during the crediting period, the downward adjusted baseline approach shall be applied.

4.2	Production of green hydrogen to substitute existing systems (Replacement).

For year 1 baseline data, greenhouse gas emissions from hydrogen production using fossil fuels, which are replaced by green hydrogen under the project, shall be determined using historical emissions together with the Adjusted Downward principle. From year 2 onwards during the crediting period, the downward adjusted baseline method shall be applied.

- Baseline Emission

Baseline greenhouse gas emissions comprise the methodology for calculating baseline emissions, the calculation of greenhouse gas emissions reduced from the baseline, and the calculation of greenhouse gas emissions under business-as-usual operations, with details as follows.

5.1	Baseline Emission Equation

Baseline emissions are calculated based on the amount of green hydrogen produced under the project and the emission factor of hydrogen produced from fossil fuels. The calculation can be expressed as follows:

| BEy | = | MH2,PJ,y x EFH2,i,y | Equation (1) |
| --- | --- | --- | --- |

Where

| BEy | = | Baseline emission in year y. (tCO2/year) |
| --- | --- | --- |
| MH2,PJ,y | = | The amount of green hydrogen produced and transported for utilization under the project in year y. (tH2) |
| EFH2,i,y | = | Emission factor of hydrogen produced from fossil fuel type i. (tCO2e/H2) |

The amount of green hydrogen produced and transported for utilization under the project activity shall be calculated according to the following equation:

y

| MH2,PJ,y | = | ∑ V0,t x vH2,t x (2/22.4) x 10-3 | Equation (2) |
| --- | --- | --- | --- |

t=1

Where

| MH2,PJ,y | = | The amount of green hydrogen produced and transported for utilization under the project in year y. (tH2) |
| --- | --- | --- |
| V0,t | = | Volumetric flow rate of gas under standard temperature and pressure (STP) during time interval t. (Sm3) |
| vH2,t | = | Volumetric fraction of hydrogen gas during time interval t. (m3 H2/m3 gas) |
| 2 | = | Molar mass of hydrogen (kg/kmol) |
| 22.4 | = | Gas volume under standard conditions (Sm3/kmol) |

The volume of hydrogen under STP conditions, the project developer shall convert the values using the following equation:

| V0,t | = | VH2,t x PH2,t x 273.15 101,325 x (273.15 + TH2,t) | Equation (3) |
| --- | --- | --- | --- |

Where

| V0,t | = | Volumetric flow rate of gas under standard temperature and pressure (STP) during time interval t (Sm3) |
| --- | --- | --- |
| VH2,t | = | Volumetric flow rate of hydrogen under operational conditions during time interval t (m³) |
| PH2,t | = | Pressure of compressed hydrogen during time interval t (Pa) |
| TH2,t | = | Temperature of compressed hydrogen during time interval t (C) |

5.2	Baseline emissions for year 1

5.2.1	Installation of new green hydrogen production systems (Greenfield)

The project developer shall calculate baseline emissions from the installation of new green hydrogen production systems (Greenfield) according to the following steps:

- For year 1 of the crediting period, the calculation shall apply existing actual data (Existing Actual) using Equation (1), together with the adjusted downwards approach specified in Section 5.2.2.

- From year 2 onwards during the crediting period, the calculation shall apply the adjusted downwards approach specified in Section 5.3.

The application of the equation shall follow the guidelines as outlined below:

- Emission factor of hydrogen produced from fossil fuel (EFH2,i,y)

Emission factor of hydrogen produced from fossil fuels for the installation of new green hydrogen production systems (Greenfield) shall be determined based on the emission factor of hydrogen produced from natural gas in the existing production system, using the Ambitious Benchmark Approach. The calculation can be expressed as follows:

EFH2,i,y = EFH2,NG Equation (4)

Where

| EFH2,i,y | = | Emission factor of hydrogen produced from fossil fuel type i (tCO2e/ tH2) |
| --- | --- | --- |
| EFH2,NG | = | Emission factor of hydrogen produced from natural gas (tCO2e/ tH2) |

5.2.2	Production of green hydrogen to substitute existing systems (Replacement).

The project developer shall calculate baseline emissions for the case of replacing the existing system with green hydrogen production, using historical emissions together with equation (1) and the Adjusted Downward principle. From year 2 onwards during the crediting period, the downward adjusted baseline method specified in Section 5.3 shall be applied. Under the adjusted downward approach, the calculation shall consider the minimum value between the downward-adjusted baseline and the baseline adjusted for uncertainty, following the steps below:

| BEadj,y1 = min (BEadj,min,y1 : BEadj,UNC,y1) | Equation (5) |
| --- | --- |

Where

| BEadj,min,y1 = BEy1 - 0.1 × (BEy1 - PEy1) | Equation (6) |
| --- | --- |
| BEadj,UNC,y1 = BEy1 × (1 - UNCBE,y1) | Equation (7) |

| BEadj,y1 | = | Adjusted baseline emissions in year 1 of the crediting period (tCO2e/year) |
| --- | --- | --- |
| BEadj,min,y1 | = | Minimum adjusted baseline emissions in year 1 of the crediting period (tCO2e/year) |
| BEadj,UNC,y1 | = | Adjusted baseline emissions for uncertainty in year 1 of the crediting period (tCO2e/year) |
| BEy1 | = | Baseline emissions in year 1 of the crediting period (tCO2e/year) |
| PEy1 | = | Project emissions in year 1 of the crediting period (tCO2e/year) |
| UNCBE,y1 | = | Uncertainty value of greenhouse gas emissions at the lower bound of the 95% confidence interval compared to the mean baseline emissions (unadjusted) in year 1 of the crediting period (% represented as a value between 0–1). |

- Baseline emissions in year 1 of the crediting period (BEy1)

For the calculation of baseline emissions in year 1 of the crediting period, project developers are required to apply equation (1). The application of the equation shall follow these guidelines:

1.1)  Emission factor of hydrogen produced from fossil fuel (EFH2,i,y)

For the replacement of the existing system with green hydrogen production, the emission factor of hydrogen produced from fossil fuels shall be determined based on historical data from the past three years, including electricity consumption, fossil fuel consumption, and hydrogen output. The calculation shall be carried out using the following equation:

t-1

EFH2,i,y = EFH2,BL = ∑ (ECH2,BL,t x EFEF,BL,t) + (∑i FCi,BL,t x NCVi x EFCO2,i) Equation (8) t-3 MH2,BL,t

Where

| EFH2,i,y | = | Emission factor of hydrogen produced from fossil fuel type i. (tCO2e/ tH2) |
| --- | --- | --- |
| EFH2,BL | = | Emission factor of the baseline hydrogen produced from fossil fuels (tCO2e/ tH2) |
| ECH2,BL,t | = | Electricity consumption of the baseline hydrogen production system in year t (MWh) |
| EFEF,BL,t | = | Emission factor of the electricity source supplying the baseline hydrogen production system in year t (tCO2e/ MWh) |
| FCi,BL,t | = | Consumption of fossil fuel type i in the baseline hydrogen production system in year t (Unit) |
| NCVi | = | Net Calorific Value of fossil fuel type i (GJ/Unit) |
| EFCO2,i | = | Emission factor of fossil fuel combustion type i (tCO2e/ GJ) |
| MH2,BL,t | = | Hydrogen output from the baseline hydrogen production system in year t (tH2) |

2)	Uncertainty value of greenhouse gas emissions at the lower bound of the 95% confidence interval compared to the mean baseline emissions (unadjusted) in year 1 of the crediting period (UNCBEy1)

Uncertainty value of greenhouse gas emissions at the lower bound of the 95% confidence interval compared to the mean baseline emissions (unadjusted) in year 1 of the crediting period, shall be calculated using the latest version of T-VER-P-TOOL-02-06 ‘Tool to Uncertainty Analysis for the Equation Calculating Greenhouse Gas Emission Reductions,’ with details as follows.

Calculation of combined uncertainty from multiplication or division

The calculation of uncertainty from multiplication or division can be performed as follows:

Equation (9)

UNCtotal=UNC12+...+ UNCi2+...+UNCn2

Where

| UNCtotal | = | The percentage uncertainty in the product of the quantities |
| --- | --- | --- |
| UNCi | = | The percentage uncertainties associated with each of the quantities |

Calculation of combined uncertainty from addition or subtraction

Calculation of combined uncertainty from addition or subtraction can be performed as follows:

Equation (10)

UNCtotal=(UNC1* x1)2+...+(UNCi* xi)2+...+ (UNCn* xn)2x1+ xi +... + xn

Where

| UNCtotal | = | The percentage uncertainty in the sum of the quantities |
| --- | --- | --- |
| UNCi | = | The percentage uncertainties associated with them. |
| xi | = | The uncertain quantities associated with them. |

5.3	Adjusted Downwards for baseline emissions from year 2 onwards

5.3.1	Adjusted Downwards for baseline emissions: DAy1

The calculation of the adjusted downwards value to be applied for determining the baseline from year 2 onwards shall consider the maximum value between the two results, as expressed in the following equation.

| 0.1 × (BEy1 - PEy1) |  |
| --- | --- |
| DAy1 = MAX | Equation (11) |
| BEy1 × UNCBE,y1 |  |

Where

| DAy1 | = | Adjusted downwards for baseline emissions in year 1 (tCO2e/year) |
| --- | --- | --- |
| BEy1 | = | Baseline emissions in year 1 of the crediting period (tCO2e/year) |
| PEy1 | = | Project emissions in year 1 of the crediting period (tCO2e/year) |
| UNCBE,y1 | = | Uncertainty value of greenhouse gas emissions at the lower bound of the 95% confidence interval compared to the mean baseline emissions. (Unadjusted) in year 1 of the crediting period (% represented as a value between 0–1) |

5.3.2	Emissions under business-as-usual operations and the principle of conservativeness. (BAUcons,y)

Emissions under business-as-usual operations and the principle of conservativeness shall be determined by considering the minimum value between the lowest adjusted baseline and the uncertainty-adjusted baseline, according to the following steps.

| BAUcons,y = min (BAUcons,min,y : BAUcons,UNC,y) | Equation (12) |
| --- | --- |

Where

BAUcons,min,y  =  BAUy - 0.1 × (BAUy - PEy) 				Equation (13)

BAUcons,UNC,y  =  BAUy × (1 - UNCBAU,y)					Equation (14)

| BAUcons,min,y | = | Emissions under business-as-usual operations and the principle of conservativeness in year y (tCO2e/year) |
| --- | --- | --- |
| BAUcons,UNC,y | = | Emissions under business-as-usual operations, incorporating the principle of conservativeness and uncertainty, in year y. (tCO2e/year) |
| BAUy | = | Emissions under business-as-usual operations in year y. (tCO2e/year) |
| PEy | = | Project emissions in year y of the crediting period (tCO2e/year) |
| UNCBAU,y | = | Uncertainty value of greenhouse gas emissions at the lower bound of the 95% confidence interval, compared to the mean business-as-usual (unadjusted) emissions in year y of the crediting period. (% represented as a value between 0–1). |

- Emissions under business-as-usual operations (BAUy)

Emissions under business-as-usual operations can be calculated as follows:

| BAUy | = | MH2,PJ,y x EFH2,BAU,y | Equation (15) |
| --- | --- | --- | --- |

Where

| BAUy | = | Emissions under business-as-usual operations in year y. (tCO2e/year) |
| --- | --- | --- |
| MH2,PJ,y | = | The amount of green hydrogen produced and transported for utilization under the project in year y. (tH2) |
| EFH2,BAU,y | = | Emission factor of hydrogen produced from fossil fuels under business-as-usual operations in year y (tCO2e/tH2) |

- Uncertainty value of greenhouse gas emissions at the lower bound of the 95% confidence interval, compared to the mean business-as-usual (Unadjusted) emissions in year y of the crediting period. (UNCBAU,cp1,y)

Uncertainty value of greenhouse gas emissions at the lower bound of the 95% confidence interval, compared to the mean business-as-usual (unadjusted) emissions in year y of the crediting period, shall be calculated using the latest version of T-VER-P-TOOL-02-06 ‘Tool to Uncertainty Analysis for the Equation Calculating Greenhouse Gas Emission Reductions,’ with details provided in Equation (9) and Equation (10).

5.3.3	Adjusted baseline value for year y (from year 2 onwards), BEadj,y

The calculation of the adjusted baseline value for year y (from year 2 onwards) can be performed as follows:

| BEadj,y = BEy - DAy1 + BEy1 x 0.01 x (y - y1) | Equation (16) |
| --- | --- |

Where

| BEadj,y | = | Adjusted baseline emissions in year y of the crediting period (tCO2e/year) |
| --- | --- | --- |
| BEy1 | = | Baseline emissions in year 1 of the crediting period (tCO2e/year) |
| DAy1 | = | Adjusted downwards value for baseline emissions in year 1 (tCO2e/year) |
| y | = | Year y |
| y1 | = | Year 1 |

Consider comparing the values of BEadj,y and BAUcons,y calculated from Section 5.3.2, and proceed according to the following conditions:

- If BEadj,y < BAUcons,y , BEadj,y can be readily applied

- If BEadj,y > BAUcons,y recalculate BEadj,y according to Section 5.3.3 by adding a constant adjustment so that BEadj,y becomes lower than BAUcons,y

Accordingly, repeat the procedures in Sections 5.3.2 and 5.3.3 for the calculation of baseline values for years 3, 4, and 5, until the completion of the carbon crediting period.

- Project Emission

Project emissions shall be based on emissions from electricity consumption, fossil fuel consumption, hydrogen transportation for utilization, and physical leakage of hydrogen, with details as follows:

| PEy | = | PEEC,y + PEFF,y + PEtransport,y + PEH2-leaks,y Equation (17) |
| --- | --- | --- |

Where

| PEy | = | Project emission in year y (tCO2eq/year) |
| --- | --- | --- |
| PEEC,y | = | Project emission from electricity consumption in year y (tCO2/year) |
| PEFF,y | = | Project emission from fossil fuel consumption in year y (tCO2/year) |
| PEtransport,y | = | Project emission from hydrogen transportation for additional utilization in year y (tCO2/year) |
| PEH2-leaks,y | = | Project emission from physical leakage of hydrogen in year y (tCO2eq/year) |

6.1	Project emission from electricity consumption (PEEC,y)

Project emission from electricity consumption can be calculated based on electricity consumption, the emission factor of electricity generation, and transmission losses in the grid, as follows:

| PEEC,y = ECPJ,y × EFElec,y × (1 + TDLy) | Equation (18) |
| --- | --- |

Where

| PEEC,y | = | Project emission from electricity consumption in year y (tCO2/year) |
| --- | --- | --- |
| ECPJ,y | = | Electricity consumption of the project activity in year y (MWh/year) |
| EFElec,y | = | Emission factor for electricity generation/use in year y (tCO2/MWh) |
| TDLy | = | Proportion of electricity transmission losses in the grid for electricity supply in year y |

6.2	Project emission from fossil fuel consumption (PEFF,y)

Project emission from fossil fuel consumption shall be calculated using the latest version of T-VER-P-TOOL-02-01 ‘Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion’

6.3	Project emission from hydrogen transportation for additional utilization (PEtransport,y)

Project emissions from hydrogen transportation for additional utilization shall be determined based on the sum of emissions from road transportation of hydrogen and emissions from pipeline transportation of hydrogen. The calculation shall be applied only in cases where hydrogen is transported to new consumers or where the transportation distance is increased and can be calculated as follows.

| PEtransport,y | = | PERoad,y + PEPipeline,y Equation (19) |
| --- | --- | --- |

Where

| PEtransport,y | = | Project emission from hydrogen transportation for additional utilization in year y (tCO2/year) |
| --- | --- | --- |
| PERoad,y | = | Project emission from road transport of hydrogen in year y (tCO2/year) |
| PEPipeline,y | = | Project emission from pipeline transport of hydrogen in year y (tCO2/year) |

6.3.1	Project emission from road transport of hydrogen (PERoad,y)

Project emission from road transport of hydrogen shall be calculated based on the round-trip distance between origin and destination of hydrogen transport, the quantity of hydrogen transport, and the emission factor of fossil fuel consumption for hydrogen transport, as follows:

| PERoad,y = Dy × Qy × EFCO2,TR,i × 10-6 | Equation (20) |
| --- | --- |

Where

| PERoad,y | = | Project emission from road transport of hydrogen in year y (tCO2/year) |
| --- | --- | --- |
| Dy | = | Round-trip distance between origin and destination of hydrogen road transport activity for utilization of the project activity in year y (km) |
| Qy | = | Quantity of hydrogen transported in year y (t H2) |
| EFCO2,TR,i | = | Emission factor of fossil fuel type i used for hydrogen transport for utilization (gCO2/t-km) |
| i | = | Type of fossil fuel |

6.3.2	Project emission from pipeline transport of hydrogen (PEPipeline,y)

Project emission from pipeline transport of hydrogen shall be calculated based on electricity consumption for pipeline transport, the emission factor of electricity generation/use, and the proportion of grid transmission losses, as follows:

| PEPipeline,y = ECH2,Pipeline,y × EFElec,y × (1 + TDLy) | Equation (21) |
| --- | --- |

Where

| PEPipeline,y | = | Project emission from pipeline transport of hydrogen in year y (tCO2/year) |
| --- | --- | --- |
| ECH2,Pipeline,y | = | Electricity consumption for pipeline transport of hydrogen of the project activity in year y (MWh/year) |
| EFElec,y | = | Emission factor for electricity generation/consumption in year y (tCO2/MWh) |
| TDLy | = | Proportion of electricity transmission losses in the grid for electricity supply in year y |

6.4	Project emission from physical leakage of hydrogen (PEH2-leaks,y)

Project emission from physical leakage of hydrogen shall be calculated based on the quantity of green hydrogen produced and the proportion of physical hydrogen leakage from the project activity, as follows:

PEH2-leaks,y = MH2,PJ,y × PLH2 × GWPH2					   Equation (22)

Where

| PEH2-leaks,y | = | Project emission from physical leakage of hydrogen in year y (tCO2eq/year) |
| --- | --- | --- |
| MH2,PJ,y | = | The amount of green hydrogen produced and transported for utilization under the project in year y. (tH2) |
| PLH2 | = | Proportion of physical hydrogen leakage from the project activity (%) |
| GWPH2 | = | Global Warming Potential (GWP) of hydrogen (tCO2eq/tH2) |

- Leakage Emission

None

- Emission Reduction

Emission reductions can be calculated as follows.

| ERy | = | BEy – PEy– LEy Equation (23) |
| --- | --- | --- |

Where

| ERy | = | Emission reductions in year y (tCO2e/year) |
| --- | --- | --- |
| BEy | = | Baseline emissions in year y (tCO2e/year) |
| PEy | = | Project emissions in year y (tCO2e/year) |
| LEy | = | Leakage emissions in year y (tCO2e/year) |

8.1	The calculation of emission reductions from the adjusted baseline during the crediting period.

The calculation of emission reductions from the adjusted baseline during the crediting period can be calculated as follows.

| ERy | = | BEadj,y – PEy– LEy Equation (24) |
| --- | --- | --- |

Where

| ERy | = | Emission reductions in year y (tCO2e/year) |
| --- | --- | --- |
| BEadj,y | = | Adjusted baseline emissions in year y of the crediting period (tCO2e/year) |
| PEy | = | Project emissions in year y (tCO2e/year) |
| LEy | = | Leakage emissions in year y (tCO2e/year) |

- Monitoring Plan

9.1	Monitoring procedure

- The project developer shall describe and specify the monitoring steps for project activity data or verification of all measurement results in the project proposal document, including the type of measuring instruments used, the responsible personnel for monitoring and data verification, instrument calibration (if applicable), and the procedures for quality assurance and quality control. In cases where alternative methods are available, such as using default values or on-site measurements, the project developer must indicate which option will be applied. Furthermore, the installation, maintenance, and calibration of measuring instruments shall be carried out in accordance with the manufacturer’s instructions and in compliance with national or international standards, such as IEC and ISO, and as specified in Table 9.2.

- All data collected as part of the monitoring of greenhouse gas emission reductions shall be stored in electronic file format and retained for a period consistent with the guidelines specified by TGO or in accordance with the organization’s quality system, but not less than the period required by TGO. The data shall also be verified for accuracy in line with the monitoring methods specified for the data/parameters to be monitored, as indicated in Table 9.2.

9.2 	Data and parameters monitored

| Data/parameter | MH2,PJ,y |  |
| --- | --- | --- |
| Description | The amount of green hydrogen produced and transported for utilization under the project in year y |  |
| Unit | tH2 |  |
| Equations referred | (1) (2) (15) and (22) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Continuous monitoring and recording of data at least monthly. |  |
| Measurement methods and procedures | Measurements shall be taken using the hydrogen flow meter. |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | Flow meter |
|  | Accuracy class | Flow meters under supervision shall comply with the specifications of the measuring instrument supplier. |
|  | Calibration requirements | Flow meters shall comply with either the specifications of the measuring instrument supplier or national requirements. If no applicable standard exists and the supplier does not specify requirements, the meter shall be calibrated every three years. |
|  | Location | Flow meters must comply with national standards or the requirements specified by the measuring instrument supplier. |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | V0,t |  |
| --- | --- | --- |
| Description | Volumetric flow rate of gas under standard temperature and pressure (STP) during time interval t |  |
| Unit | Sm3 |  |
| Equations referred | (2) and (3) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Measurement methods and procedures | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | vH2,t |  |
| --- | --- | --- |
| Description | Volumetric fraction of hydrogen gas during time interval t. |  |
| Unit | m3 H2/m3 gas |  |
| Equations referred | (2) and (3) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Measurement methods and procedures | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | VH2,t |  |
| --- | --- | --- |
| Description | Volumetric flow rate of hydrogen under operational conditions during time interval t. |  |
| Unit | m³ |  |
| Equations referred | (2) and (3) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Measurement methods and procedures | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | PH2,t |  |
| --- | --- | --- |
| Description | Pressure of compressed hydrogen during time interval t. |  |
| Unit | Pa |  |
| Equations referred | (3) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Measurement methods and procedures | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | TH2,t |  |
| --- | --- | --- |
| Description | Temperature of compressed hydrogen during time interval t. |  |
| Unit | C |  |
| Equations referred | (3) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Measurement methods and procedures | Reference: T-VER-P-TOOL-02-05 Tool to calculate the mass flow of a greenhouse gas in a gaseous stream |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | EGH2,y |  |
| --- | --- | --- |
| Description | Electricity supplied to the green hydrogen production system in year y |  |
| Unit | MWh/year |  |
| Equations referred | - |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Daily data recording |  |
| Measurement methods and procedures | Measurements shall be taken using a bidirectional kWh meter |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | Bidirectional kWh meter |
|  | Accuracy class | - Electricity meters under regulatory supervision shall comply with the requirements of the meter supplier and/or the requirements specified by the transmission system operator, or national requirements. - Electricity meters not under regulatory supervision shall comply with the requirements of the meter supplier or national requirements. In cases where no applicable standard exists and the meter supplier has not specified requirements, the meter shall be calibrated every three years and shall have an accuracy class of at least 0.5. |
|  | Calibration requirements | - Electricity meters under regulatory supervision shall comply with national standards, or the requirements specified by the meter supplier, or those specified by the transmission system operator. - Electricity meters not under regulatory supervision shall comply with national standards or the requirements specified by the meter supplier. |
|  | Location | Install measuring instruments at the connection point with the green hydrogen production system. |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | EFH2,BAU,y |  |
| --- | --- | --- |
| Description | Emission factor of hydrogen produced from fossil fuels under business-as-usual operations in year y |  |
| Unit | tCO2e/ tH2 |  |
| Equations referred | (15) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Once per year |  |
| Measurement methods and procedures | For the preparation of the project proposal document The latest EFH2,BAU,y value announced by TGO shall be used. For the monitoring of greenhouse gas emission reductions The EFH2,BAU,y value announced by TGO for the calendar year corresponding to the crediting period shall be used. In cases where the EFH2,BAU,y value for that calendar year has not yet been announced by TGO, the latest EFH2,BAU,y value announced by TGO shall be applied for that year. |  |
| Entity/person responsible for the measurement | TGO |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | - |  |
| Additional comment | - |  |

| Data/parameter | EFElec,y |  |
| --- | --- | --- |
| Description | Emission factor for electricity generation/use in year y |  |
| Unit | tCO2/MWh |  |
| Equations referred | (18) and (21) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Once per year |  |
| Measurement methods and procedures | For the preparation of the project proposal document The latest EFElec,y value announced by TGO shall be used. For the monitoring of greenhouse gas emission reductions The EFElec,y value announced by TGO for the calendar year corresponding to the crediting period shall be used. In cases where the EFElec,y value for that calendar year has not yet been announced by TGO, the latest EFElec,y value announced by TGO shall be applied for that year. |  |
| Entity/person responsible for the measurement | TGO |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | - |  |

| Data/parameter | ECPJ,y |  |
| --- | --- | --- |
| Description | Electricity consumption of the project activity in year y |  |
| Unit | MWh/year |  |
| Equations referred | (18) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Electricity consumption shall be recorded at least on a monthly basis. |  |
| Measurement methods and procedures | Measurements shall be taken using a bidirectional kWh meter |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | Bidirectional kWh meter |
|  | Accuracy class | - Electricity meters under regulatory supervision shall comply with the requirements of the meter supplier and/or the requirements specified by the transmission system operator, or national requirements. - Electricity meters not under regulatory supervision shall comply with the requirements of the meter supplier or national requirements. In cases where no applicable standard exists and the meter supplier has not specified requirements, the meter shall be calibrated every three years and shall have an accuracy class of at least 0.5. |
|  | Calibration requirements | - Electricity meters under regulatory supervision shall comply with national standards, or the requirements specified by the meter supplier, or those specified by the transmission system operator. - Electricity meters not under regulatory supervision shall comply with national standards or the requirements specified by the meter supplier. |
|  | Location | - In the case of electricity from renewable energy, measuring instruments shall be installed at the connection point with the renewable energy generation system. - In the case of electricity from the grid, measuring instruments shall be installed at the connection point with the electricity grid. |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | TDLy |  |
| --- | --- | --- |
| Description | Proportion of electricity transmission losses in the grid for electricity supply in year y |  |
| Data unit | - |  |
| Equations referred | (18) and (21) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | - If Option 1 is chosen, the project developer shall monitor the value annually throughout the monitoring of greenhouse gas emission reductions. - If Option 2 is chosen, the project developer shall apply this value consistently throughout the monitoring of greenhouse gas emission reductions. |  |
| Measurement methods and procedures | Option 1: Report measurement results in cases where data on the amount of electricity supplied by the producer and the amount of electricity received by the consumer are available. Option 2: Use the latest value announced by TGO (equal to 0.0596), which is referenced from the Thailand Energy Balance Report 2023 by the Department of Alternative Energy Development and Efficiency. For rural low-voltage grids, a value of 0.2 shall be used. |  |
| Entity/person responsible for the measurement | In the case of Option 1, the project developer shall be the reporting entity. In the case of Option 2, TGO shall be the reporting entity. |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | - |  |
| Additional comment | In cases where the measurement results differ significantly from previous measurements or other relevant data sources, additional measurements shall be conducted. |  |

| Data/parameter | Dy |  |
| --- | --- | --- |
| Description | Round-trip distance between origin and destination of hydrogen road transport activity for utilization of the project activity in year y |  |
| Unit | km |  |
| Equations referred | (21) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Round-trip distance shall be recorded at least monthly. |  |
| Measurement methods and procedures | Round-trip distance shall be recorded from vehicles transporting hydrogen by road. |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | - |
|  | Accuracy class | - |
|  | Calibration requirements | - |
|  | Location | - |
| QA/QC procedures | - |  |
| Treatment of uncertainties | - |  |
| Additional comment | - |  |

| Data/parameter | ECH2,Pipeline,y |  |
| --- | --- | --- |
| Description | Electricity consumption for pipeline transport of hydrogen of the project activity in year y |  |
| Unit | MWh/year |  |
| Equations referred | (22) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | Electricity consumption shall be recorded at least on a monthly basis. |  |
| Measurement methods and procedures | Measurements shall be taken using a bidirectional kWh meter |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | Bidirectional kWh meter |
|  | Accuracy class | - Electricity meters under regulatory supervision shall comply with the requirements of the meter supplier and/or the requirements specified by the transmission system operator, or national requirements. - Electricity meters not under regulatory supervision shall comply with the requirements of the meter supplier or national requirements. In cases where no applicable standard exists and the meter supplier has not specified requirements, the meter shall be calibrated every three years and shall have an accuracy class of at least 0.5. |
|  | Calibration requirements | - Electricity meters under regulatory supervision shall comply with national standards, or the requirements specified by the meter supplier, or those specified by the transmission system operator. - Electricity meters not under regulatory supervision shall comply with national standards or the requirements specified by the meter supplier. |
|  | Location | Install measuring instruments at the hydrogen pipeline inlet. |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

| Data/parameter | Qy |  |
| --- | --- | --- |
| Description | Quantity of hydrogen transported in year y |  |
| Unit | tH2 |  |
| Equations referred | (21) |  |
| Purpose of data | Baseline emission Project emission Leakage emission |  |
| Monitoring frequency | The quantity of hydrogen transported shall be recorded at least monthly. |  |
| Measurement methods and procedures | Measurements shall be taken using a flow meter |  |
| Entity/person responsible for the measurement | Project developer |  |
| Measuring instrument | Type of instrument | Flow meter |
|  | Accuracy class | Flow meters under supervision shall comply with the specifications of the measuring instrument supplier. |
|  | Calibration requirements | Flow meters shall comply with either the specifications of the measuring instrument supplier or national requirements. If no applicable standard exists and the supplier does not specify requirements, the meter shall be calibrated every three years. |
|  | Location | Install measuring instruments within the project area. |
| QA/QC procedures | - |  |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |  |
| Additional comment | - |  |

9.3 	Data and parameters not monitored

| Data/parameter | EFH2,NG |
| --- | --- |
| Description | Emission factor of hydrogen produced from natural gas |
| Unit | tCO2e/ ton H2 |
| Equations referred | (4) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | 9 tCO2e/tH2 |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | Reference: CDM AM0124 Hydrogen production from electrolysis of water |
| Treatment of uncertainties | Reference from value(s) applied |
| Additional comment | - |

| Data/parameter | FCi,BL,t |
| --- | --- |
| Description | Consumption of fossil fuel type i in the baseline hydrogen production system in year t |
| Unit | unit |
| Equations referred | (8) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | - |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | Record the quantity of fossil fuel consumed by the baseline hydrogen production. |
| Treatment of uncertainties | Reference from value(s) applied |
| Additional comment |  |

| Data/parameter | NCVi |
| --- | --- |
| Description | Net Calorific Value of fossil fuel type i. |
| Unit | GJ/Unit |
| Equations referred | (8) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | - |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | Option 1 Data from the fuel supplier: The density of the fuel shall be recorded for each fuel delivery, and the values obtained shall then be used to calculate the annual weighted average. Option 2 Measurements by the project developer: In cases where the fuel supplier does not provide the fuel values, measurements shall be conducted in accordance with national or international fuel standards. The NCV of the fuel shall be determined for each fuel delivery, and the annual weighted average shall be calculated from the obtained values. Option 3 Regional or national standard values (applicable only to liquid fuels, in cases where data from the fuel supplier is not available). Such values shall be reviewed annually. Option 4 Reference to Table 1.2 in Chapter 1 of Volume 2 of the 2019 Refinement to the 2006 IPCC Guidelines for National GHG Inventories: The upper bound of the 95% confidence interval shall be used as the default value in cases where data from the fuel supplier is not available. |
| Treatment of uncertainties | Verify whether the uncertainty values specified under Option 1, Option 2, and Option 3 fall within the uncertainty ranges of the IPCC default values as indicated in Table 1.2 of Volume 2 of the 2019 Refinement to the 2006 IPCC Guidelines. In cases where the values are lower than this range, additional data shall be collected from the relevant laboratories to explain the cause of the results or further measurements shall be conducted. Laboratories for Option 1, Option 2, and Option 3 must be accredited under ISO standards or other equivalent standards. |
| Additional comment | - |

| Data/parameter | ECH2,BL,t |
| --- | --- |
| Description | Electricity consumption of the baseline hydrogen production system in year t |
| Unit | MWh |
| Equations referred | (8) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | - |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | Record the quantity of fossil fuel consumed by the baseline hydrogen production. |
| Treatment of uncertainties | Reference from value(s) applied |
| Additional comment |  |

| Data/parameter | EFEF,BL,t |
| --- | --- |
| Description | Emission factor of the electricity source supplying the baseline hydrogen production system in year t |
| Unit | tCO2e/MWh |
| Equations referred | (8) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | - |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | Reference shall be made to the emission factor announcement for electricity generation/use (Emission factor) for projects and greenhouse gas reduction activities issued by TGO. |
| Treatment of uncertainties | Reference from value(s) applied |
| Additional comment |  |

| Data/parameter | EFCO2,i |
| --- | --- |
| Description | Emission factor of fossil fuel combustion type i |
| Unit | tCO2e/ GJ |
| Equations referred | (8) and (19) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | - |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | Table 1.4 of Volume 2, Chapter 1 of the IPCC (2006) |
| Treatment of uncertainties | Reference from value(s) applied |
| Additional comment |  |

| Data/parameter | EFCO2,TR,i |
| --- | --- |
| Description | Emission factor of fossil fuel type i used for hydrogen transport for utilization |
| Unit | gCO2/t-km |
| Equations referred | (21) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | - |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | In cases where alternative data sources are selected, the following values shall be applied: - For transportation by small vehicles, use 245 (gCO2/t km). - For transportation by large vehicles, use 129 (gCO2/t km). |
| Treatment of uncertainties | - |
| Additional comment | - |

| Data/parameter | MH2,BL,t |
| --- | --- |
| Description | Hydrogen output from the baseline hydrogen production system in year t |
| Unit | tH2 |
| Equations referred | (8) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | - |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | - |
| Treatment of uncertainties | Uncertainty values shall be referenced from the measuring instruments. |
| Additional comment | - |

| Data/parameter | PLH2 |
| --- | --- |
| Description | Proportion of physical hydrogen leakage from the project activity |
| Unit | % |
| Equations referred | (23) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | 5 |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | Reference: CDM AM0124 Hydrogen production from electrolysis of water |
| Treatment of uncertainties | - |
| Additional comment | - |

| Data/parameter | GWPH2 |
| --- | --- |
| Description | Global Warming Potential (GWP) of hydrogen |
| Unit | tCO2eq/tH2 |
| Equations referred | (23) |
| Purpose of data | Baseline emission Project emission Leakage emission |
| Value(s) applied | 5.8 |
| Source of Data | Measured Other sources |
| Measurement methods and procedures | IPCC AR4 WG1 as under chapter 2.10.3.6 |
| Treatment of uncertainties | - |
| Additional comment | - |

10.  References

1) CDM: AM0124 Hydrogen production from electrolysis of water version 01.1

2) Article 6.4 Mechanism: A6.4-AMM-001 Flaring or use of landfill gas.

Document information T-VER-P-METH-01-08

| Version | Amendment | Entry into force | Description |
| --- | --- | --- | --- |
| 01 | - | 24 September 2026 | Initial issue |