T-VER-P-METH-14-02

CO2 Capture for Utilization for Concrete Production

Version 01

Scope: 05 – Chemical industry

Scope: 06 – Construction

Entry into force on 24 September 2025

| - Methodology | CO2 Capture and Utilization for Concrete Production |
| --- | --- |
| - Project Type | Capture, storage, and/or utilization of greenhouse gas |
| - Scope | Scope: 05 – Chemical industry Scope: 06 – Construction |
| - Project Outline | The project activity must aim to capture carbon dioxide (CO2) either from a defined point source or through Direct Air Capture (DAC) to be utilized in concrete production process. |
| - Applicability | The project activity must include a process for capturing gas from the emission source and separating CO2 (Capture), as well as utilizing the CO2 (Utilization) in concrete production for permanent CO2 storage. The proportion of cement used in the concrete product must not increase compared to the baseline scenario |
| - Project Conditions | 1) The concrete produced from the project activity must have a compressive strength equivalent to that of conventional concrete (baseline scenario) and must comply with engineering standards applicable to its intended use, as well as with the Thai Industrial Standard TIS 213-2520. 2) The concrete produced from the project activity must be utilized in commercial applications. 3) The project activity may include processes for CO2 liquefaction, CO2 evaporation, and the transportation of CO2 via vehicles or pipelines. 4) The project activity may involve the use of additives in cement or concrete. 5) The project activity must not involve concrete recycling. 6) The utilization of CO2 under the concrete production project activity requires project developers to specify a permanent serial number on the concrete to enable traceability. 7) The project developer must monitor and verify the total amount of CO2 captured through the capture process, as well as the amount of CO2 utilized in concrete production, to ensure accuracy and completeness. |
| - Project Starting Date | The date on which the project owner (employer) and the contractor jointly signed the construction or installation agreement for the greenhouse gas reduction project to be developed as a T-VER project. |
| - Definition | Concrete refers to a mixture of cement, sand, gravel, and water commonly used in various types of construction work, such as fresh concrete, concrete for road construction, structural concrete, and concrete for bridge building. Cement refers to a binding material used in concrete production, primarily composed of limestone, clay, silica, alumina, and gypsum. During the cement manufacturing process, carbon dioxide is released through a chemical reaction known as calcination. Calcination refers to the process of converting limestone (CaCO3), whose primary component is calcium carbonate, into calcium oxide (CaO), during which carbon dioxide (CO2) is released as a result of a chemical reaction. Carbonation plant refers to the facility or site where concrete mixing processes are carried out to enable carbonation during project implementation. Independent Power Supply (IPS) refers to a private power producer that generates electricity for self-consumption without supplying it to the grid or selling it directly to end-users. Direct Mineral Carbonation refers to A chemical reaction occurs between CO2 and metal oxides (alkali or alkaline earth metals), such as calcium oxide (CaO) and magnesium oxide (MgO) which are components of cement and water, resulting in the formation of calcium carbonate (CaCO3) or magnesium carbonate (MgCO3). These compounds are chemically stable minerals capable of permanently storing CO2. Indirect Mineral Carbonation refers to the carbon sequestration process of CO2 during concrete production involves two key reactions: first, the interaction between CO2, silica oxide (SiO2) in cement, and water; followed by the extraction of calcium from calcium silicate hydrate compounds and a direct carbonation reaction that leads to the crystallization of calcium carbonate (CaCO3), enabling permanent CO2 storage. |
| - Note |  |

Details of T-VER Methodology for CO2 Capture and Utilization for Concrete Production

- Emission reductions activities used in the calculations

Table 1: Sources and Types of Greenhouse Gases

| Greenhouse gas emission | Source | Greenhouse Gas | Details of activities that emit greenhouse gas emissions |
| --- | --- | --- | --- |
| Baseline Emission | A defined industrial stack emission | CO2 | - Fossil fuel combustion - Calcination of limestone |
|  | Carbon dioxide in the atmosphere | CO2 | Anthropogenic and natural activities |
|  | Baseline concrete | CO2 | Mass of carbon contained in baseline concrete |
|  | Baseline emission from cement | CO2 | Emission from cement production process |
| Project Emission | Concrete used in project activity | CO2 | Mass of carbon in concrete during project implementation |
|  | Cement used in project activity | CO2 | Emission from cement production process |
|  | Carbon capture and separation |  |  |
|  | - Leakage | CO2 | Machinery/Equipment/Pipeline system |
|  | - On-site electricity and heat generation for process use | CO2 | Combustion of fossil fuels in power plants within the project boundary |
|  | - Use of externally purchased electricity and thermal energy in the process | CO2 | - Electricity generation by grid-connected power plants - Electricity and/or steam generation by IPS-type power plants |
|  | Preparation of CO2 in liquid form |  |  |
|  | - On-site electricity generation for process use | CO2 | Combustion of fossil fuels in power plants within the project boundary |
|  | - Electricity consumption (purchased from external sources) in the process | CO2 | - Electricity generation by grid-connected power plants - Electricity and/or steam generation by IPS-type power plants |
|  | CO2 transportation |  |  |
|  | - Release or leakage | CO2 | Machinery/Equipment/Pipeline system |
|  | - Use of fossil fuels | CO2 | Fuel combustion from transportation vehicles |
|  | - Electricity consumption (purchased from external sources) in the process | CO2 | - Electricity generation by grid-connected power plants - Electricity and/or steam generation by IPS-type power plants |
|  | Preparation of CO2 in vapor phase |  |  |
|  | - Use of fossil fuels | CO2 | Fossil fuel combustion within the project boundary |
|  | - On-site electricity generation for process use | CO2 | Fossil fuel combustion in power plants within the project boundary |
|  | - Electricity consumption (purchased from external sources) in the process | CO2 | - Electricity generation by grid-connected power plants - Electricity and/or steam generation by IPS-type power plants |
|  | Utilization of CO2 in concrete production |  |  |
|  | - Release or leakage | CO2 | Machinery/Equipment/Pipeline system |
|  | - Use of fossil fuels | CO2 | Fossil fuel combustion within the project boundary |
|  | - On-site electricity generation for use in the process | CO2 | Combustion of fossil fuels in power plants within the project boundary |
|  | - Electricity consumption (purchased from external sources) in the process | CO2 | - Electricity generation by grid-connected power plants - Electricity generation by IPS-type power plants |
|  | Use of solvents in the CO2 utilization process | CO2 | Solvent production process |
| Leakage Emission | Transportation of gravel, sand, water, and cement |  |  |
|  | - Use of fossil fuels | CO2 | Fuel combustion from vehicles used for transportation |
|  | - Electricity consumption (purchased from external sources) in the process | CO2 | - Electricity generation by grid-connected power plants - Electricity generation by IPS-type power plants |

- Scope of Project

The project activity must aim to capture carbon dioxide either from a point-source emission or directly from the atmosphere, and store within concrete products.

- Carbon storage in concrete must ensure that the performance characteristics of the resulting concrete, when used in various applications, are equivalent to those of baseline concrete or meet the relevant standards for that specific type of concrete.

- The transportation of cement, aggregates (gravel and sand), and water from outside the project boundary refers to the transport of materials used for the production of concrete that exceeds the baseline production volume.

Figure 1: Carbon Dioxide Capture and Storage Process in Concrete Production

The project activity consists of five distinct components, as illustrated in Figure 1

- CO2 capture and separation from point sources can be carried out at various stages, as outlined below

  - Pre-combustion

  - Post-combustion

  - Oxy-fuel combustion

  - Direct Air Capture

Examples of Technologies Used for CO2 Capture

  - Adsorption

  - Absorption

  - Membrane separation

  - Cryogenic distillation

  - Chemical Looping

- CO2 Preparation (Liquefaction): Converting CO2 into its liquid state to optimize transportation efficiency

- Transportation The separated CO2 can be transported to the designated area for utilization in concrete production through various modes of transportation, including

  - Pipeline

  - Land transportation

In cases where the CO2 capture and separation process (Capture) and the CO2 collection (Collection) occur at the same location as the carbonation plant, allowing direct CO2 feed into concrete production, greenhouse gas emissions from transportation activities shall not be considered or shall be deemed zero.

- CO2 Evaporation: Converting CO2 into its gaseous state to optimize its suitability as a feedstock for mineral carbonation in the carbonation plant

- Utilization of CO2 within the carbonation plant occurs through the mineral carbonation process to produce various types of concrete, such as fresh concrete, pavement concrete, structural concrete (architectural concrete), and bridge concrete. This process constitutes the storage of CO2 within the concrete, whereby CO2 is chemically transformed into calcium carbonate (CaCO3), becoming an integral part of the concrete matrix.

Ca(OH)2 + CO2  ⇋ CaCO3 + H2O

The project activity may not include CO2 liquefaction, CO2 transportation, or CO2 evaporation processes as illustrated in Figure 1, depending on the actual implementation of the project.

- Additionality

The project must undergo further proof of operation from normal operations. (Additionality) using the “Proof of Operations Guidelines in addition to normal operations (Additionality) under the Thailand Voluntary Emission Reduction Program (T-VER)" standard equivalent to the international standards prescribed by the TGO.

- Baseline Scenario

The project activity involving carbon dioxide capture for utilization in concrete production represents the first of its kind in Thailand. Therefore, the baseline information refers to the amount of CO2 that is utilized and subsequently stored within the concrete during the production process

Note:	If Thailand sees an increase in CCU projects for concrete production, TGO (Thailand Greenhouse Gas Management Organization) will consider revising the baseline data in accordance with the 'Below Business as Usual (Below BAU) approach

If the utilization of CO2 in concrete production results in a reduction in cement consumption, Below Business as Usual (Below BAU) baseline approach may be applied. In such cases, the baseline concrete production shall be considered to use hydraulic cement with a greenhouse gas emission factor of 871 kgCO2 per ton of cement.

- Baseline Emission

Baseline emission is considered only from carbon dioxide (CO2) from a specific designated source. The calculation is based on the amount of carbon in concrete that increases under conventional operations, combined with the use of cement, which releases CO2 during its production process.

Baseline emission can be calculated as followed:

BEy = n (BECO2,Store,n,y) + BECement,y	Equation (1)

Where

| BEy | = | Baseline emissions in year y (tCO2e/year) |
| --- | --- | --- |
| BECO2,Store,n,y | = | Quantity of carbon dioxide calculated based on the difference between the amount of carbon stored in concrete type n under project implementation and that in the baseline case in year y (tCO2/year) |
| BECement,y | = | Baseline emissions from concrete production based on the amount of hydraulic cement used in year y (tCO2/year) |

5.1	Quantity of carbon dioxide calculated based on the difference between the amount of carbon stored in concrete under project implementation and that in the baseline case (BECO2,Store,n,y)

Quantity of carbon dioxide calculated based on the difference between the amount of carbon stored in concrete under project implementation and that in the baseline case may be calculated using one of the following three approaches.

Option 1: Direct Measurement of Carbon Content in Baseline Concrete

The amount of carbon dioxide sequestered in baseline concrete may be calculated based on the measured carbon content in concrete samples combined with total concrete production data. The calculation may be performed using the following equation.

BECO2,Store,n,y = Σi QConcrete,PJ,i,y x ((QC,PJ,Test,i - QC,BL,Test,i) x 106 x 4412) 	      Equation (2)

Where

| BECO2,Store,i,y | = | Quantity of carbon dioxide calculated based on the difference between the amount of carbon stored in concrete type i under project implementation and that in the baseline case in year y (tCO2/year) |
| --- | --- | --- |
| QConcrete,PJ,i,y | = | Quantity of concrete type i produced by the project activity in year y (t concrete/year) |
| QC,PJ,Test,i,y | = | Average carbon (C) content in samples of concrete type i from the project activity in year y (gram of C/gram of tested concrete) |
| QC,BL,Test,i | = | Average carbon (C) content in samples of concrete type i from the baseline case (gram of C/gram of tested concrete) |
| i | = | Type of concrete |

Option 2: Assessment via Measured Volume of CO2 Supplied to the Carbonation Plant

The amount of carbon dioxide sequestered in baseline concrete may be calculated based on the measured volume of CO2 gas entering the carbonation plant and the residual volume of CO2 remaining within the system. The residual CO2 may be considered as project emissions in cases where the carbonation process operates under a closed system. Alternatively, it may be excluded from the calculation if the CO2 is sourced from ambient air or would otherwise have been released under normal conditions. The assessment shall be conducted per concrete production batch, with energy consumption during the process maintained at a constant level.

The amount of carbon dioxide sequestered in baseline concrete may be calculated as followed. The carbonation process must operate under a closed system, with no leakage of gas during the reaction phase.

BECO2,Store,i,y = b (QCO2,PJ,Feed,i,b,y - QCO2,PJ,Non-Store,i,b,y)			    Equation (3)

Where

| BECO2,Store,i,y | = | Quantity of carbon dioxide calculated based on the difference between the amount of carbon stored in concrete type i under project implementation and that in the baseline case in year y (tCO2/year) |
| --- | --- | --- |
| QCO2,PJ,Feed,i,b,y | = | Amount of CO2 used for injection into the carbonation plant for producing concrete type i in production batch b under the project activity in year y (tCO2/year) |
| QCO2,PJ,Non-Store,i,b,y | = | Amount of residual CO2 remaining in the carbonation plant (not stored in concrete) for concrete type i in production batch b under the project activity in year y (tCO2/year) |
| i | = | Type of concrete |
| b | = | Concrete production batch |

The amount of CO2 gas injected into the carbonation plant per concrete production cycle (b) may be calculated based on the gas density, the proportion of CO2 in the gas mixture, and the total volume of gas supplied to the carbonation plant. The calculation may be performed using the following equation.

QCO2,PJ,Feed,i,b,y = t,b ρCO2,b x (%V/VCO2,Feed,i,b,y) x VFeed,b x 103		    Equation (4)

Where

| QCO2,PJ,Feed,i,b,y | = | Amount of CO2 used for injection into the carbonation plant for producing concrete type i in production batch b under the project activity in year y (tCO2/year) |
| --- | --- | --- |
| ρCO2,b | = | Density of CO2 under operating conditions in concrete production batch b (kg/m³) |
| %V/VCO2,Feed,i,b,y | = | Volumetric proportion of CO2 to total gas volume at the injection point of the carbonation plant, measured during concrete production batch b under operational conditions in year y (%) |
| VFeed,b | = | Volume of CO2 injected into the carbonation plant during concrete production batch b (m³) |
| i | = | Type of concrete |
| b | = | Concrete production batch |

The amount of CO2 gas not sequestered as carbonate in concrete may be calculated based on the free volume, pressure, and temperature within the reaction chamber, combined with the CO2 concentration and gas density. This assessment shall be conducted per concrete production cycle (b), using the following equation.

QCO2,PJ,Non-Store,i,b,y=  bPAfter,b VChamber,b R TAfter,b -PBefore,b VChamber,b R TBefore,b   x (%V/VCO2,Feed,i,b,y) x 44.01 x 10-6

Equation (5)

Where

| QCO2,PJ,Non-Store,i,b,y | = | Amount of residual CO2 remaining in the carbonation plant (not stored in concrete) for concrete type i in production batch b under the project activity in year y (tCO2/year) |
| --- | --- | --- |
| PAfter,b | = | Pressure within the chamber after the completion of the concrete production batch b (bar), following the mineral carbonation reaction inside the chamber |
| PBefore,b | = | Pressure within the chamber prior to operation during concrete production batch b (bar), before CO2 is fed into the chamber |
| VChamber,b | = | Void volume within the chamber during concrete production batch b (m³) |
| R | = | Gas constant = 8.314472 x 10-5 (m3 bar K-1 mol-1) |
| TAfter,b | = | Temperature within the chamber after operation during concrete production batch b (K), following completion of the mineral carbonation reaction in the chamber |
| TBefore,b | = | Temperature within the chamber prior to operation during concrete production batch b (K), before CO2 is injected into the chamber |
| %V/VCO2,Feed,i,b,y | = | Volumetric ratio of CO2 to total gas volume at the injection point of the carbonation plant, measured during concrete production batch b under operational conditions in year y (%) |
| i | = | Type of concrete |
| b | = | Concrete production batch |

Option 3: Assessment Using System Efficiency Estimates

The amount of carbon dioxide sequestered in baseline concrete may be calculated using an estimated system efficiency value of 0.6, based on a conservative approach of experimental results published by Monkman, S. (2018)

In this case, the amount of carbon dioxide sequestered in baseline concrete may be calculated using the following equation.

BECO2,Store,n,y = Σb (QCO2,PJ,Feed,i,b,y x 0.6)					    Equation (6)

Where

| BECO2,Store,i,y | = | Quantity of carbon dioxide calculated based on the difference between the amount of carbon stored in concrete type i under project implementation and that in the baseline case in year y (tCO2/year) |
| --- | --- | --- |
| QCO2,PJ,Feed,i,b,y | = | Amount of CO2 used for injection into the carbonation plant for producing concrete type i in production batch b under the project activity in year y (tCO2/year) |
| 0.6 | = | Amount of carbon dioxide transformed into mineral forms in concrete based on the conservative calculation approach, as referenced from the principle of conservative estimation for mineralization potential during concrete production referred from the academic publication by Monkman, S. (2018)[6]. |
| n | = | Type of concrete |
| b | = | Concrete production batch |

5.2	Baseline emissions from concrete production based on the amount of hydraulic cement used (BECement)

Baseline emissions from concrete production based on the amount of hydraulic cement used can be calculated as followed:

BECement,y = i QCement,BL,i,y  EFHC				                  Equation (7)

Where

| BECement,y | = | Baseline emissions from concrete production based on the amount of hydraulic cement used in year y (tCO2/year) |
| --- | --- | --- |
| QCement,BL,i,y | = | Quantity of hydraulic cement used in the production of concrete type i from the baseline case in year y (t cement) |
| EFHC | = | Emission factor from hydraulic cement production (tCO2/t cement) |
| i | = | Type of concrete |

5.2.1	Quantity of hydraulic cement used in the production of concrete from the baseline case (QCement,BL,i,y)

Quantity of hydraulic cement used in the production of concrete from the baseline case shall be assessed by comparing the quantity of hydraulic cement contained in baseline concrete sample type i with that in the corresponding project concrete sample

The amount of cement used in baseline concrete production may be calculated as followed:

QCement,BL,i,y = i (QCement,PJ,i,y x (QCement,BL,Test,iQCement,PJ,Test,i ))		      Equation (8)

Where

| QCement,BL,i,y | = | Quantity of hydraulic cement used in the production of concrete type i from the baseline case in year y (t cement) |
| --- | --- | --- |
| QCement,PJ,i,y | = | The quantity of cement used in the production of concrete type i in year y (t cement) |
| QCement,BL,Test,i | = | Quantity of hydraulic cement utilized in the preparation of concrete type i samples under the baseline scenario (gram of cement) |
| QCement,PJ,Test,i | = | Quantity of hydraulic cement utilized in the preparation of concrete type i samples under the project activity (gram of cement) |
| i | = | Type of concrete |

- Project Emission

Project emission from the project activity is considered based on seven key components of the project operations, namely: Cement consumption during project implementation CO2 capture process CO2 separation and collection process CO2 transportation process CO2 liquefaction process CO2 evaporation process and Mineral carbonation process. Additionally, the acquisition of solvents used to accelerate the mineral carbonation reaction is also considered. Only components that are implemented within the project scope shall be considered.

Project emission can be calculated as followed:

PEy =  PECement,y + PECapt,y + PELiq,y + PETrans,y + PEEvap,y+ PEMineralC,y+ PESolvent,y	Equation (9)

Where

| PEy | = | Project emissions in year y (tCO2e/year) |
| --- | --- | --- |
| PECement,y | = | Project emissions from concrete production based on the amount of hydraulic cement used in year y (tCO2/year) |
| PECapt,y | = | Project emissions from CO2 capture, separation, and collection processes in year y (tCO2/year) |
| PELiq,y | = | Project emissions from liquefying CO2 in year y (tCO2/year) |
| PETrans,y | = | Project emissions from the CO2 transportation process in year y (tCO2/year) |
| PEEvap,y | = | Project emissions from the vaporization of CO2 in year y (tCO2/year) |
| PEMineralC,y | = | Project emissions from conditioning the chamber (carbonation plant) to achieve suitable temperature and pressure for mineral carbonation in year y (tCO2/year) |
| PESolvent,y | = | Project emissions from the use of concrete admixture in year y (tCO2/year) |

6.1	Project emissions from concrete production based on the amount of hydraulic cement used (PECement,y)

Project emissions from concrete production based on the amount of hydraulic cement used can be calculated as followed:

PECement,y = i (QCement,PJ,j,y  EFCement,j)					        Equation (10)

Where

| PECement,y | = | Project emissions from concrete production based on the amount of hydraulic cement used in year y (tCO2/year) |
| --- | --- | --- |
| QCement,PJ,j,y | = | Quantity of cement type j used for concrete production under the project activity in year y (t cement) |
| EFCement,j | = | Emission factor of cement type j under the project activity (tCO2/t cement) |
| i | = | Type of concrete |

6.2	Project emissions from CO2 capture, separation, and collection processes (PECapt,y)

Project emissions from CO2 capture, separation, and collection processes can be calculated as followed:

PECapt,y = PECapt,FF,y + PECapt,Elec,y + PECapt,IPS,y	Equation (11)

Where

| PECapt,y | = | Project emissions from CO2 capture, separation, and collection processes in year y (tCO2/year) |
| --- | --- | --- |
| PECapt,FF,y | = | Project emissions from fossil fuel combustion during the capture, separation, and collection of CO2 in year y (tCO2/year) |
| PECapt,Elec,y | = | Project emissions from electricity consumption from the grid during the capture, separation of CO2 from other gases, and collection processes in year y (tCO2/year) |
| PECapt,IPS,y | = | Project emissions from the use of electricity and/or steam supplied by IPS-type power plants for the CO2 capture, separation from other gases, and collection processes in year y (tCO2e/year) |

6.2.1	Project emissions from fossil fuel combustion during the capture, separation, and collection of CO2 (PECapt,FF,y)

Project emissions from fossil fuel combustion during the capture, separation, and collection of CO2 such as heat generation for the regeneration of absorbents or adsorbents, and electricity and heat generation for CO2 capture, separation, and collection (excluding externally purchased energy)  shall be calculated using the latest version of the calculation tool T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion

6.2.2	Project emissions from electricity consumption from the grid during the capture, separation of CO2 from other gases, and collection processes (PECapt,Elec,y)

Project emissions from electricity consumption from the grid during the capture, separation of CO2 from other gases, and collection processes can be calculated based on the amount of electricity consumed, the emission factor associated with electricity generation/use, and the proportion of transmission and distribution losses in the grid, as expressed in the following equation.

PECapt,Elec,y = ECCapt,PJ,y × EFElec,y × (1 + TDLy) × 10-3	Equation (12)

Where

| PECapt,Elec,y | = | Project emissions from electricity consumption from the grid during the capture, separation of CO2 from other gases, and collection processes in year y (tCO2e/year) |
| --- | --- | --- |
| ECCapt,PJ,y | = | Electricity consumption for the capture, separation of CO2 from other gases, and collection processes in year y (kWh/year) |
| EFElec,y | = | Emission factor for electricity generation/consumption in year y (tCO2/MWh) |
| TDLy | = | Average technical transmission and distribution losses for providing electricity in year y (%) |

6.2.3	Project emissions from the use of electricity and/or steam supplied by IPS-type power plants for the CO2 capture, separation from other gases, and collection processes (PECapt,IPS,y)

Project emissions from the use of electricity and/or steam supplied by IPS-type power plants for the CO2 capture, separation from other gases, and collection processes shall be assessed based on fossil fuel combustion using an allocation approach. The resulting energy-related emissions shall then be calculated using the latest version of the calculation tool T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion.

FCCapt,IPS,y = 1/IPS,y  TFCIPS,y ×HCCapt,PJ,y+(3.6×ECCapt,PJ,y)HGIPS,y+(3.6×EGIPS,y)	Equation (13)

Where

| FCCapt,IPS,y | = | Fossil fuel consumption for purchased electricity and/or heat generation used in the CO2 capture, separation from other gases, and collection processes in year y (Unit/year) |
| --- | --- | --- |
| TFCIPS,y | = | Total fossil fuel consumption by the power plant(s) supplying electricity and/or steam for use in year y (Unit/year) |
| HCCapt,PJ,y | = | Heat consumption for the CO2 capture, separation from other gases, and collection processes in year y (MJ/year) |
| HGIPS,y | = | Total heat output generated by power plant(s) supplying electricity and/or steam used in year y (MJ/year) |
| EGIPS,y | = | Total electricity output generated by power plant(s) supplying electricity and/or steam used in year y (kWh/year) |
| IPS,y | = | Average annual efficiency of the power plant in year y (value between 0 and 1) |

6.3 	Project emissions from liquefying CO2 (PELiq,y)

Project emissions from liquefying CO2 can be calculated as followed:

PELiq,y = PELiq,Elec,y + PELiq,IPS,y	Equation (14)

Where

| PELiq,y | = | Project emissions from liquefying CO2 in year y (tCO2/year) |
| --- | --- | --- |
| PELiq,Elec,y | = | Project emissions from grid electricity consumption during the liquefaction process of CO2 in year y (tCO2/year) |
| PELiq,IPS,y | = | Project emissions from the use of electricity and/or steam from IPS-type power plants for CO2 liquefaction processes in year y (tCO2/year) |

6.3.1	Project emissions from grid electricity consumption during the liquefaction process of CO2 (PELiq,Elec,y)

Project emissions from grid electricity consumption during the liquefaction process of CO2 can be calculated based on the amount of electricity consumed, the emission factor associated with electricity generation/use, and the proportion of transmission and distribution losses in the grid, as expressed in the following equation.

PELiq,Elec,y = ECLiq,PJ,y × EFElec,y × (1 + TDLy) × 10-3	Equation (15)

Where

| PELiq,Elec,y | = | Project emissions from grid electricity consumption during the liquefaction process of CO2 in year y (tCO2e/year) |
| --- | --- | --- |
| ECLiq,PJ,y | = | Electricity consumption for the CO2 liquefaction process in year y (kWh/year) |
| EFElec,y | = | Emission factor for electricity generation/consumption in year y (tCO2/MWh) |
| TDLy | = | Average technical transmission and distribution losses for providing electricity in year y (%) |

6.3.2	Project emissions from the use of electricity and/or steam from IPS-type power plants for CO2 liquefaction processes (PELiq,IPS,y)

Project emissions from the use of electricity and/or steam from IPS-type power plants for CO2 liquefaction processes shall be assessed based on fossil fuel combustion using an allocation approach. The resulting emissions shall then be calculated using the latest version of the calculation tool T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion.

FCLiq,IPS,y = 1/IPS,y  TFCIPS,y ×3.6×ECLiq,PJ,yHGIPS,y+(3.6×EGIPS,y)	Equation (16)

Where

| FCLiq,IPS,y | = | Fossil fuel consumption for electricity generation associated with purchased electricity used in the CO2 liquefaction process in year y (Unit/year) |
| --- | --- | --- |
| TFCIPS,y | = | Total fossil fuel consumption by the power plant(s) supplying electricity and/or steam for use in year y (Unit/year) |
| HGIPS,y | = | Total heat output generated by power plant(s) supplying electricity and/or steam used in year y (MJ/year) |
| EGIPS,y | = | Total electricity output generated by power plant(s) supplying electricity and/or steam used in year y (kWh/year) |
| IPS,y | = | Average annual efficiency of the power plant in year y (value between 0 and 1) |

6.4	Project emissions from the CO2 transportation process (PETrans,y)

Project emissions from the transportation of CO2 captured, separated, and collected from other gases to the utilization site for mineral carbonation in concrete under the project activity can be calculated as followed:

PETrans,y = PETrans,FF,y + PETrans,Elec,y + PETrans,IPS,y	Equation (17)

Where

| PETrans,y | = | Project emissions from the CO2 transportation process in year y (t CO2/year), whereby if the location of CO2 capture, separation and collection activities and the carbonation plant are within the same area, PETrans,y = 0 |
| --- | --- | --- |
| PETrans,FF,y | = | Project emissions from the combustion of fossil fuels in machinery, equipment, or vehicles used for CO2 transportation in year y (tCO2/year) |
| PETrans,Elec,y | = | Project emissions from grid electricity consumption for CO2 transportation via pipeline in year y (tCO2/year) |
| PETrans,IPS,y | = | Project emissions from electricity and/or steam consumption supplied by an IPS-type power plant for CO2 pipeline transportation in year y (tCO2/year) |

6.4.1	Project emissions from the combustion of fossil fuels in machinery, equipment, or vehicles used for CO2 transportation (PETrans,FF,y)

Project emissions from fossil fuel combustion in vehicles used for transporting CO2 captured, separated, and collected from other gases to the utilization site for mineral carbonation in concrete under the project activity shall be calculated using the latest version of the calculation tool T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion

6.4.2	Project emissions from electricity and/or steam consumption supplied by an IPS-type power plant for CO2 pipeline transportation (PETrans,Elec,y)

Project emissions from electricity and/or steam consumption supplied by an IPS-type power plant for CO2 pipeline transportation can be calculated as followed:

PETrans,Elec,y = ECTrans,PJ,y × EFElec,y × (1 + TDLy) × 10-3	Equation (18)

Where

| PETrans,Elec,y | = | Project emissions from grid electricity consumption for CO2 transportation via pipeline under project operations in year y (tCO2e/year) |  |
| --- | --- | --- | --- |
| ECTrans,PJ,y | = | Electricity consumption for CO2 transportation under project operations in year y (kWh/year) |  |
| EFElec,y | = | Emission factor for electricity generation/consumption in year y (tCO2/MWh) |  |
| TDLy | = | Average technical transmission and distribution losses for providing electricity in year y (%) |  |

6.4.3.	Project emissions from electricity and/or steam consumption supplied by an IPS-type power plant for CO2 pipeline transportation (PETrans,IPS,y)

Project emissions from electricity and/or steam consumption supplied by an IPS-type power plant for CO2 pipeline transportation shall be assessed based on fossil fuel combustion using an allocation approach. The resulting emissions shall then be calculated using the latest version of the calculation tool T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion.

FCTrans,IPS,y = 1/IPS,y  TFCIPS,y ×3.6×ECTrans,PJ,yHGIPS,y+3.6×EGIPS,y	Equation (19)

Where

| FCTrans,IPS,y | = | Fossil fuel consumption for electricity generation associated with purchased electricity used in the CO2 transportation process in year y (Unit/year) |
| --- | --- | --- |
| TFCIPS,y | = | Total fossil fuel consumption by the power plant(s) supplying electricity and/or steam for use in year y (Unit/year) |
| HGIPS,y | = | Total heat output generated by power plant(s) supplying electricity and/or steam used in year y (MJ/year) |
| EGIPS,y | = | Total electricity output generated by power plant(s) supplying electricity and/or steam used in year y (kWh/year) |
| IPS,y | = | Average annual efficiency of the power plant in year y (value between 0 and 1) |

6.5	Project emissions from the vaporization of CO2 (PEEvap,y)

Project emissions from the vaporization of CO2 can be calculated as followed:

PEEvap,y = PEEvap,Elec,y + PEEvap,IPS,y	Equation (20)

Where

| PEEvap,y | = | Project emissions from the vaporization of CO2 in year y (tCO2e/year) |
| --- | --- | --- |
| PEEvap,Elec,y | = | Project emissions from grid electricity consumption in the CO2 vapor-phase preparation process in year y (tCO2e/year) |
| PEEvap,IPS,y | = | Project emissions from the use of electricity and/or steam supplied by IPS-type power plants for CO2 vapor-phase preparation processes in year y (tCO2e/year) |

6.5.1	Project emissions from grid electricity consumption in the CO2 vapor-phase preparation process (PEEvap,Elec,y)

Project emissions from grid electricity consumption in the CO2 vapor-phase preparation process can be calculated based on the amount of electricity consumed, the emission factor associated with electricity generation/use, and the proportion of transmission and distribution losses in the grid, as expressed in the following equation.

PEEvap,Elec,y = ECEvap,PJ,y × EFElec,y × (1 + TDLy) × 10-3	Equation (21)

Where

| ECEvap,PJ,y | = | Electricity consumption for the preparation of CO2 in vapor phase under project operations in year y (kWh/year) |
| --- | --- | --- |
| EFElec,y | = | Emission factor for electricity generation/consumption in year y (tCO2/MWh) |
| TDLy | = | Average technical transmission and distribution losses for providing electricity in year y (%) |

6.5.2	Project emissions from the use of electricity and/or steam supplied by IPS-type power plants for CO2 vapor-phase preparation processes (PEEvap,IPS,y)

Project emissions from the use of electricity and/or steam supplied by IPS-type power plants for CO2 vapor-phase preparation processes shall be assessed based on fossil fuel combustion using an allocation approach. The resulting emissions shall then be calculated using the latest version of the calculation tool T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion.

FCEV,IPS,y = 1/IPS,y  TFCIPS,y ×HCEvap,PJ,y+(3.6×ECEvap,PJ,y)HGIPS,y+(3.6×EGIPS,y)	Equation (22)

Where

| FCEvap,IPS,y | = | Fossil fuel consumption for electricity and/or heat generation associated with purchased energy used in the CO2 vapor-phase preparation process under project operations in year y (Unit/year) |
| --- | --- | --- |
| TFCIPS,y | = | Total fossil fuel consumption by the power plant(s) supplying electricity and/or steam for use in year y (Unit/year) |
| HCEvap,PJ,y | = | Heat consumption for the preparation of CO2 in vapor phase in year y (MJ/year) |
| HGIPS,y | = | Total heat output generated by power plant(s) supplying electricity and/or steam used in year y (MJ/year) |
| EGIPS,y | = | Total electricity output generated by power plant(s) supplying electricity and/or steam used in year y (kWh/year) |
| IPS,y | = | Average annual efficiency of the power plant in year y (value between 0 and 1) |

6.6	Project emissions from conditioning the chamber (carbonation plant) to achieve suitable temperature and pressure for mineral carbonation (PEMineralC,y)

Project emissions from conditioning the chamber (carbonation plant) to achieve suitable temperature and pressure for mineral carbonation can be calculated as followed:

PEMineralC,y = PEMineralC,Elec,y + PEMineralC,IPS,y	Equation (23)

Where

| PEMineralC,y | = | Project emissions from conditioning the chamber (carbonation plant) to achieve suitable temperature and pressure for mineral carbonation in year y (tCO2e/year) |
| --- | --- | --- |
| PEMineralC,Elec,y | = | Project emissions from electricity consumption from the power grid in the mineral carbonation production process in year y (tCO2e/year) |
| PEMineralC,IPS,y | = | Project emissions from the use of electricity and/or steam supplied by an IPS-type power plant in the mineral carbonation production process in year y (tCO2e/year) |

6.6.1	Project emissions from electricity consumption from the power grid

in the mineral carbonation production process (PEMineralC,Elec,y)

Project emissions from electricity consumption from the power grid in the mineral carbonation production process can be calculated based on the amount of electricity consumed, the emission factor associated with electricity generation/use, and the proportion of transmission and distribution losses in the grid, as expressed in the following equation.

PEMineralC,Elec,y = ECMineralC,PJ,y × EFElec,y × (1 + TDLy) × 10-3	Equation (24)

Where

| PEMineralC,Elec,y | = | Project emissions from electricity consumption from the power grid in the mineral carbonation production process in year y (tCO2e/year) |
| --- | --- | --- |
| ECMineralC,PJ,y | = | Electricity consumption in the mineral carbonation production process in year y (kWh/year) |
| EFElec,y | = | Emission factor for electricity generation/consumption in year y (tCO2/MWh) |
| TDLy | = | Average technical transmission and distribution losses for providing electricity in year y (%) |

6.6.2	Project emissions from the use of electricity and/or steam supplied by an IPS-type power plant in the mineral carbonation production process (PEMineralC,IPS,y)

Project emissions from the use of electricity and/or steam supplied by an IPS-type power plant in the mineral carbonation production process shall be assessed based on fossil fuel combustion using an allocation approach. The resulting emissions shall then be calculated using the latest version of the calculation tool T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion.

FCMineralC,IPS,y = 1/IPS,y  TFCIPS,y ×3.6×ECMineralC,PJ,yHGIPS,y+(3.6×EGIPS,y)	Equation (25)

Where

| FCMineralC,IPS,y | = | Fossil fuel consumption for electricity generation in the purchased portion used in the mineral carbonation production process in year y (Unit/year) |
| --- | --- | --- |
| TFCIPS,y | = | Total fossil fuel consumption by the power plant(s) supplying electricity and/or steam for use in year y (Unit/year) |
| HGIPS,y | = | Total heat output generated by power plant(s) supplying electricity and/or steam used in year y (MJ/year) |
| EGIPS,y | = | Total electricity output generated by power plant(s) supplying electricity and/or steam used in year y (kWh/year) |
| IPS,y | = | Average annual efficiency of the power plant in year y (value between 0 and 1) |

6.7	Project emissions from the use of concrete admixture (PESolvent,y)

Project emissions from the use of concrete admixture can be calculated as followed:

PESolvent,y = Σs(QC,Solvent,PJ,s,y x 4412) + Σs((FCSolvent,s,y x EFSolvent,s,y) 	Equation (26)

Where

| PESovent,y | = | Project emissions from the use of concrete admixture in year y (tCO2e/year) |
| --- | --- | --- |
| QC,Solvent,PJ,s,y | = | Carbon (C) content in solution type s for concrete mixing in year y (ton of C) |
| FCSolvent,s,y | = | Consumption of solvent type s in year y (ton of solvent/year) |
| EFSolvent,s,y | = | Emission factor of solvent type s (tCO2e/ton of Solvent) |
| s | = | Type of solvent |

6.7.1	Carbon (C) content in solution type s for concrete mixing (QC,Solvent,PJ,s,y)

Carbon (C) content in solution type s for concrete mixing can be calculated as followed:

QC,Solvent,PJ,s,y =   QSolvent,PJ,s,y  x Molar mass of C in solventsTotal molar mass of solvents) x ρSolvent,S	Equation (27)

Where

| QC,Solvent,PJ,s,y | = | Carbon (C) content in solution type s for concrete mixing in year y (ton of C) |
| --- | --- | --- |
| QSolvent,PJ,s,y | = | Volume of solution type s for concrete mixing from project implementation in year y (m³) |
| Molar mass of C in solvents | = | Mass of carbon in 1 mole (mol) of solution type s (grams) |
| Total molar mass of solvents | = | Mass of 1 mole of solution type s (grams) |
| ρSolvent,S | = | Density of solution type s (kg/Liter, kg/dm³) |

6.7.2	Emission factor of solvent (EFSOLVENT,s,y)

The emission factor for solvent types may be referenced from the following data sources.

Option 1: Emission Factor Based on Chemical Properties

  - In cases where the chemical is not listed in available databases, the emission factor may be determined using a conservative approach. This involves applying the highest emission factor among chemicals within the same group, based on data from the Carbon Footprint of Products (CFP) published by the Thailand Greenhouse Gas Management Organization (TGO).

  - In cases where the chemical is not listed within the same chemical group, the emission factor may be referenced using the calculation method outlined in the Carbon Footprint of Products (CFP) assessment guidelines established by the Thailand Greenhouse Gas Management Organization (TGO).

Option 2: Emission Factor from Online Databases suchas https://carboncloud.com/

Option 3: The emission factor may be obtained from calculation software tools such as SimaPro

- Leakage Emission

Leakage emission shall be considered from the transportation of concrete mix materials used in the additional concrete production beyond the baseline scenario. These emissions may be calculated as followed:

LEy = LETrans,OB,FF,y + LETrans,OB,Elec,y + LETrans,OB,IPS,y			     Equation (28)

Where

| LEy | = | Leakage emission in year y (tCO2e/year) |
| --- | --- | --- |
| LETrans,OB,FF,y | = | Leakage emission from fossil fuel consumption for transporting concrete mix materials used in the production of concrete exceeding the baseline production level in year y (tCO2e/year) |
| LETrans,OB,Elec,y | = | Leakage emission from the use of electricity from the power grid for transporting concrete mix materials used in additional concrete production beyond the baseline level in year y (tCO2e/year) |
| LETrans,OB,IPS,y | = | Leakage emission from electricity consumption supplied by IPS-type power plants for the transportation of concrete mix materials used in the production of concrete beyond the baseline level in year y (tCO2e/year) |

7.1	Leakage emission from fossil fuel consumption for transporting concrete mix materials used in the production of concrete exceeding the baseline production (LETrans,OB,FF,y)

Leakage emission from fossil fuel consumption for transporting concrete mix materials used in the production of concrete exceeding the baseline production shall be calculated using the latest version of the TGO calculation tool: T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion.

  - Leakage emission from the use of electricity from the power grid for transporting concrete mix materials used in additional concrete production beyond the baseline level (LETrans,OB,Elec,y)

Leakage emission from the use of electricity from the power grid for transporting concrete mix materials used in additional concrete production beyond the baseline level shall be calculated based on the amount of electricity consumed, the grid emission factor, and the transmission and distribution loss rate, as shown in the following equation

LETrans,OB,Elec,y = ECTrans,OB,PJ,y × EFElec,y × (1 + TDLy) × 10-3	Equation (29)

Where

| ECTrans,OB,PJ,y | = | Electricity consumption for transporting concrete mix materials used in the production of concrete exceeding the baseline level in year y (tCO2e/year) |
| --- | --- | --- |
| EFElec,y | = | Emission factor for electricity generation/consumption in year y (tCO2/MWh) |
| TDLy | = | Average technical transmission and distribution losses for providing electricity in year y (%) |

7.3	Leakage emission from electricity consumption supplied by IPS-type power plants for the transportation of concrete mix materials used in the production of concrete beyond the baseline level (LETrans,OB,IPS,y)

Leakage emission from electricity consumption supplied by IPS-type power plants for the transportation of concrete mix materials used in the production of concrete beyond the baseline level shall be assessed based on fossil fuel combustion. The allocation principle shall be applied to determine the proportion of fuel use attributable to the project activity. The resulting fuel consumption shall then be used to calculate greenhouse gas emissions using the latest version of the TGO calculation tool: T-VER-P-TOOL-02-01: Tool to Calculate Project or Leakage CO2 Emissions from Fossil Fuel Combustion.

FCTrans,OB,IPS,y = 1/IPS,y  TFCIPS,y ×3.6×ECtrans,OB,PJ,yHGIPS,y+(3.6×EGIPS,y)	Equation (30)

Where

| FCTrans,OB,IPS,y | = | Fossil fuel consumption for electricity generation in the purchased portion used for transporting concrete mix materials associated with increased concrete production beyond the baseline level in year y (tCO2e/year) |
| --- | --- | --- |
| TFCIPS,y | = | Total fossil fuel consumption by the power plant(s) supplying electricity and/or steam for use in year y (Unit/year) |
| HGIPS,y | = | Total heat output generated by power plant(s) supplying electricity and/or steam used in year y (MJ/year) |
| EGIPS,y | = | Total electricity output generated by power plant(s) supplying electricity and/or steam used in year y (kWh/year) |
| IPS,y | = | Average annual efficiency of the power plant in year y (value between 0 and 1) |

- Emission Reduction

Emission reduction can be calculated as followed:

| ERy | = | BEy – PEy– LEy Equation (31) |
| --- | --- | --- |

Where

| ERy | = | Emission reductions in year y (tCO2e/year) |
| --- | --- | --- |
| BEy | = | Baseline emissions in year y (tCO2e/year) |
| PEy | = | Project emissions in year y (tCO2e/year) |
| LEy | = | Leakage emission in year y (tCO2e/year) |

- Monitoring Plan

9.1 Monitoring approach

1) 	The project developer explains and specify the steps for monitoring the project activity data (Activity data) or verify all measurement results in the project proposal document. including the type of measuring instruments used Person responsible for monitoring results and verifying information Calibration of measuring instruments (if any) and procedures for warranty and quality control Where methods have different options, such as using default values or on-site measurements The project developer must specify which option to use. In addition, the installation, maintenance and calibration of measuring instruments should be carried out in accordance with the instructions of the equipment manufacturer and in accordance with national standards. or international standards such as IEC, ISO.

2)	All data collected as part of the greenhouse gas reduction monitoring. The data should be stored in electronic file format and the retention period is in accordance with the guidelines set by the Administrative Organization or the organization's quality system, but the period of time is not less than that specified by the TGO. Must follow the follow-up methods specified in the follow-up parameters specified in Table 9.2.

9.2 Data and parameters monitored

| Parameter | QConcrete,PJ,i,y |
| --- | --- |
| Unit | t concrete/year |
| Description | Quantity of concrete type i produced by the project activity in year y |
| Source of data | Record of the amount of concrete used from concrete manufacturers |
| Measurement procedures | Weighing of concrete load |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | QCement,PJ,j,y |
| --- | --- |
| Unit | t concrete/year |
| Description | Quantity of cement type j used for concrete production under the project activity in year y |
| Source of data | Record of the amount of concrete used from concrete manufacturers |
| Measurement procedures | Weighing of concrete load |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | QC,PJ,Test,i,y |
| --- | --- |
| Unit | gram of C/gram of project tested concrete |
| Description | Average carbon (C) content in samples of concrete type i from the project activity in year y |
| Source of data | The results of the carbon content analysis in type I concrete samples under baseline conditions, conducted using reliable scientific instruments or analytical techniques |
| Measurement procedures | Carbon content analysis in type i concrete samples conducted during project implementation, using reliable scientific instruments or analytical techniques |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | ρCO2,b |
| --- | --- |
| Unit | kg/m3 |
| Description | Density of CO2 under operating conditions in concrete production batch b |
| Source of data | Table showing the density of CO2 under various operational conditions. |
| Measurement procedures | Use the values from the table that correspond to the operational conditions of CO2 used in delivery to the carbonation plant |
| Monitoring frequency | Each time concrete type n is produced in concrete production batch b |

| Parameter | %V/VCO2,Feed,i,b,y |
| --- | --- |
| Unit | - |
| Description | Volumetric proportion of CO2 to total gas volume at the injection point of the carbonation plant, measured during concrete production batch b under operational conditions in year y |
| Source of data | Project data record |
| Measurement procedures | Continuous volumetric analysis, in conjunction with pressure and temperature measurements of the gas, is conducted to calculate the composition under standard conditions. The use of Gas Chromatography must be approved by the relevant authorities |
| Monitoring frequency | Continuous monitoring and at least monthly recording are required for the production of type n concrete during production cycle b |

| Parameter | VFeed,b |
| --- | --- |
| Unit | m3 |
| Description | Volume of CO2 injected into the carbonation plant during concrete production batch b |
| Source of data | Project data record |
| Measurement procedures | Continuous gas volume measurement, together with pressure and temperature monitoring, shall be conducted to calculate the volume under standard conditions. The flow meter used must be approved by the relevant authority |
| Monitoring frequency | Continuous monitoring and recording, at minimum on a monthly basis, are required for the production of type n concrete during production cycle b |

| Parameter | VChamber,b |
| --- | --- |
| Unit | m3 |
| Description | Void volume within the chamber during concrete production batch b |
| Source of data | Project data record |
| Measurement procedures | Measurements from the project |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | PAfter,b and Pbefore,b |
| --- | --- |
| Unit | bar |
| Description | PAfter,b refer to internal pressure within the chamber after the completion of the concrete production batch b (bar), following the mineral carbonation reaction inside the chamber Pbefore,b refer to internal pressure within the chamber prior to operation during concrete production batch b (bar), before CO2 is fed into the chamber |
| Source of data | Project data record |
| Measurement procedures | Continuous monitoring of pressure, along with measurement of gas volume and temperature, for the purpose of calculating the volume under standard conditions, with the pressure gauge subject to approval by the relevant authority |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | TAfter,b and Tbefore,b |
| --- | --- |
| Unit | K |
| Description | TAfter,b refer to internal temperature within the chamber after operation during concrete production batch b (K), following completion of the mineral carbonation reaction in the chamber Tbefore,b refer to internal temperature within the chamber prior to operation during concrete production batch b (K), before CO2 is injected into the chamber |
| Source of data | Project data record |
| Measurement procedures | Continuous temperature monitoring, along with gas volume and pressure measurements, for calculating standardized volume, whereby the industrial thermometer must be approved by the relevant authorities. |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | QCement,PJ,i,y |
| --- | --- |
| Unit | t cement |
| Description | The quantity of cement used in the production of concrete type i in year y |
| Source of data | Record the amount of cement used in concrete production from project activities |
| Measurement procedures | Verify the correlation between recorded cement consumption and the amount of concrete produced |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | QCement,PJ,Test,i |
| --- | --- |
| Unit | gram of cement |
| Description | Quantity of hydraulic cement utilized in the preparation of concrete type i samples under the project activity |
| Source of data | Record the amount of cement used in concrete production from project activities |
| Measurement procedures |  |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | ECCapt,PJ,y , ECLQ,PJ,y , ECTrans,PJ,y , ECEvap,PJ,y , ECMineralC,PJ,y และ ECTrans,OB,PJ,y |
| --- | --- |
| Unit | kWh/year |
| Description | ECCapt,PJ,y refer to electricity consumption for the capture, separation of CO2 from other gases, and collection processes in year y ECLiq,PJ,y refer to electricity consumption for the CO2 liquefaction process in year y ECTrans,PJ,y refer to electricity consumption for CO2 transportation under project operations in year y ECEvap,PJ,y refer to electricity consumption for the preparation of CO2 in vapor phase under project operations in year y ECMineralC,PJ,y refer to electricity consumption in the mineral carbonation production process in year y ECTrans,OB,PJ,y refer to electricity consumption for transporting concrete mix materials used in the production of concrete exceeding the baseline level in year y |
| Source of data | Project data record |
| Measurement procedures | Measure electricity consumption for relevant processes using instruments such as a kWh meter or record data from monthly electricity purchase documents such as receipts. Measuring instruments must be calibrated by an accredited entity according to the manufacturer's specifications |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | EFElec,y |
| --- | --- |
| unit | tCO2/MWh |
| Description | Emission factor for electricity generation/consumption in year y |
| Source of data | Report on greenhouse gas emissions (Emission Factor) from electricity generation/consumption for projects and activities of greenhouse gas reduction published by TGO. |
| Measurement procedures | For the preparation of project proposal documents Use the latest EFElec,y announced by TGO For monitoring the results of reducing greenhouse gas emissions Use the EFElec,y values announced by TGO according to the year of the crediting period. However, in the case that the year of the crediting period does not have EFElec,y values announced by TGO, use the latest EFElec,y values announced by TGO in that year instead. |
| Monitoring frequency | - |

| Parameter | TDLy |
| --- | --- |
| unit | - |
| Description | Average technical transmission and distribution losses for providing electricity in year y |
| Source of data | Option 1: Report based on monitoring data in cases where information is available on the amount of electricity supplied by the producer and the amount of electricity received by the user Option 2: Use the most recent value announced by TGO (value = 0.0596), which is based on data from the Thailand Energy Balance Report for the year 2023, by the Department of Alternative Energy Development and Efficiency |
| Measurement procedures | If Option 1 is chosen, the project developer must monitor the specified value annually throughout the monitoring period of greenhouse gas emission reductions If Option 2 is chosen, the project developer must use this value consistently throughout the monitoring period of greenhouse gas emission reduction |
| Monitoring frequency | To be determined once in the first year of the crediting period |

| Parameter | IPS,y |
| --- | --- |
| Unit | - |
| Description | Average annual efficiency of the power plant in year y (value between 0 and 1) |
| Source of data | Report on energy measurements and energy balance of the power plant |
| Measurement procedures | Measurements shall be conducted using the power plant’s monitoring instruments, including: • Amount of electricity • Quantity and pressure of steam • Quantity and temperature of flue gas • Mass or volume of fossil fuels used |
| Monitoring frequency | - |

| Parameter | TFCIPS,y |
| --- | --- |
| Unit | unit/year |
| Description | Total fossil fuel consumption by the power plant(s) supplying electricity and/or steam for use in year y |
| Source of data | Monitoring report and record logs of the power plant |
| Measurement procedures | Measure the mass or volume of fuel using the power plant’s monitoring instruments |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | HCCapt,PJ,y and HCEvap,PJ,y |
| --- | --- |
| Unit | MJ/year |
| Description | HCCapt,PJ,y refer to heat consumption for the CO2 capture, separation from other gases, and collection processes in year y HCEvap,PJ,y refer to heat consumption for the preparation of CO2 in vapor phase in year y |
| Source of data | Monitoring report and record logs |
| Measurement procedures | Measure the quantity and pressure of steam used in the process of CO2 capture and separation from gas using monitoring instruments, and calculate the thermal energy value, or obtain records from monthly steam purchase documents such as receipts. All monitoring instruments must be calibrated by accredited entities in accordance with the manufacturer's specifications |
| Monitoring frequency | Continuous monitoring and at least monthly recording |

| Parameter | HGIPS,y |
| --- | --- |
| Unit | MJ/year |
| Description | Total heat output generated by power plant(s) supplying electricity and/or steam used in year y (MJ/year) |
| Source of data | Monitoring report and record logs of the power plant |
| Measurement procedures | Measure the quantity and pressure of steam using the power plant’s monitoring instruments |
| Monitoring frequency | - |

| Parameter | EGIPS,y |
| --- | --- |
| Unit | kWh/year |
| Description | Total electricity output generated by power plant(s) supplying electricity and/or steam used in year y |
| Source of data | Monitoring report and record logs of the power plant. |
| Measurement procedures | Measure electricity consumption using the power plant’s monitoring instruments. |
| Monitoring frequency | - |

| Parameter | Qc,Solvent,PJ,s,y |
| --- | --- |
| Unit | ton of C |
| Description | Carbon (C) content in solution type s for concrete mixing in year y |
| Source of data | Record data from the project |
| Measurement procedures | Results of carbon content analysis in baseline samples of concrete type n, conducted using reliable scientific instruments or analytical techniques |
| Monitoring frequency | Conduct sampling inspections at least once per year for each type of concrete |

| Parameter | FCSolvent,s,y |
| --- | --- |
| Unit | ton of Solvent/year |
| Description | Consumption of solvent type s in year y |
| Source of data | Record the quantity of admixture used for concrete production |
| Measurement procedures | Verify the correlation between admixture volume used and the quantity of concrete produced |
| Monitoring frequency | Random sampling inspections shall be conducted at least once per year for each type of concrete |

9.3 Data and parameters not monitored

| Parameter | QC,BL,Test,i |
| --- | --- |
| Unit | gram of C/gram of project tested concrete |
| Description | Average carbon (C) content in samples of concrete type i from the baseline case |
| Source of data | Results of carbon content analysis in baseline samples of concrete type i, conducted using credible scientific instruments or analytical techniques |
| Applicable value | - |

| Parameter | R |
| --- | --- |
| Unit | m3 bar K-1 mol-1 |
| Description | Gas constant |
| Source of data | Theoretical calculation value |
| Applicable value | 8.314472 x 10-5 |

| Parameter | QCement,BL,test,i |
| --- | --- |
| Unit | gram of cement |
| Description | Quantity of hydraulic cement utilized in the preparation of concrete type i samples under the baseline scenario |
| Source of data | Option 1: Reference value derived from recorded cement usage in concrete sample production under the facility’s baseline scenario. Option 2: Reference value based on standard ratios for concrete production in project implementation, as defined by the quality specifications set by the responsible authority. |
| Applicable value | - |

| Parameter | EFHC |
| --- | --- |
| Unit | tCO2e/ton of cement |
| Description | Emission factor from hydraulic cement production |
| Source of data | Thai Cement Manufacturers Association |
| Applicable value | 0.871 tCO2/t cement |

| Parameter | EFCement,j |
| --- | --- |
| Unit | tCO2e/ton of cement |
| Description | Emission factor of cement type j under the project activity |
| Source of data | Thai Cement Manufacturers Association |
| Applicable value | Case 1: Using Portland cement (910 kgCO2e per ton of cement) Case 2: Using hydraulic cement (871 kgCO2e per ton of cement) |

| Parameter | Molar mass of C in solvents |
| --- | --- |
| Unit | gram |
| Description | Mass of carbon in 1 mole (mol) of solution type s |
| Source of data | Molecular mass calculation through chemical equations and the periodic table, considering only carbon |
| Applicable value | - |

| Parameter | Total molar mass of solvents |
| --- | --- |
| Unit | gram |
| Description | Mass of 1 mole of solution type s |
| Source of data | Molecular mass calculation through chemical equations and the periodic table |
| Applicable value | - |

| Parameter | ρSolvent,S |
| --- | --- |
| Unit | kg/Liter, kg/dm3 |
| Description | Density of solution type s |
| Source of data | Chemical product labeling or description |
| Applicable value | - |

| Parameter | EFSOLVENT,s,y |
| --- | --- |
| Unit | tCO2e/ton of Solvent |
| Description | Emission factor of solvent type s |
| Source of data | Carbon Footprint of Product (CFP) data for the solution |
| Applicable value | - |

- Reference

- Verified Carbon Standard

VCS Methodology: VM0032 CO2 Utilization in Concrete Production, Version 1.0, 5 April 2021

- Gold Standard

Methodology: Carbon sequestration through accelerated carbonation of concrete aggregate, Version 1.0, 7-03-2022

- Al‐Mamoori, A., Krishnamurthy, A., Rownaghi, A. A., & Rezaei, F. (2017). Carbon capture and utilization update. Energy Technology, 5(6), 834-849.

- Hunt AJ, Sin EHK, Marriott R, Clark JH. Generation, capture, and utilization of

industrial carbon dioxide. ChemSusChem 2010;3:306–22. https://doi.org/

10.1002/cssc.200900169.

- Ravikumar, D., Zhang, D., Keoleian, G., Miller, S., Sick, V., & Li, V. (2021). Carbon dioxide utilization in concrete curing or mixing might not produce a net climate benefit. Nature communications, 12(1), 855.

- Monkman, S. (2018). Sustainable ready mixed concrete production using waste CO2: A case study. Am. Concr. Ins, 330, 163-174

- Initiative, C. S. (2009). Cement industry energy and CO2 performance: getting the numbers right. World Business Council for Sustainable Development.

Document information T-VER-P-METH-14-02

| Version | Amendment | Entry into force | Description |
| --- | --- | --- | --- |
| 01 | - | 24 September 2025 | Initial adoption |