Project Design Document

The Siam Cement (Ta Luang) Company Limited

| Project Details |  |
| --- | --- |
| Project title | SCG Ta Luang Plant Cement LC3 Project Thailand |
| Project participant | The Siam Cement (Ta Luang) Company Limited |
| Co-project participant | Carbon Coordinating Managing Entity Limited (CCME) |
| Project owner | The Siam Cement (Ta Luang) Company Limited |
| Project location | Ta Luang District, Saraburi, Thailand |
| Coordinates of project location | 14°34'10.362" North latitude, 100°45'52.772" East longitude 14°39'55.8" North latitude, 100°51'12.2" East longitude |
| Project type | ☐ Renewable energy of fossil fuel replacement. ☐ Improvement of the efficiency of electricity and heat generation. ☐ Use of public transportation system. ☐ Use of electric vehicle. ☐ Improvement of the efficiency of engine. ☐ Improvement of the efficiency of energy consumption in building and factory and in household. ☐ Use of natural refrigerant. ☒ Use of clinker substitute. ☐ Solid waste management. ☐ Domestic wastewater management. ☐ Methane recovery and utilization. ☐ Industrial wastewater management. ☐ Reduction, absorption and removal of greenhouse gases from the forestry and agriculture sectors. ☐ Capture, storage, and/or utilization of greenhouse gas. ☐ Other ........................................................................................ |
| Project model | ☒ Single project ☐ Bundled project |
| Project size | ☐ Micro scale ☐ Small scale ☒ Large scale |
| T-VER methodology and tools | T-VER-P-METH-08-01 |
| Project activity | The SCG Ta Luang Plant Cement LC3 Project in Thailand aims to significantly reduce industrial carbon emissions by transitioning from traditional blended hydraulic cement to Limestone Calcined Clay Cement (LC3). This technology calcines raw clay at lower temperatures than traditional clinker to produce approximately 2 million tons of low-carbon cement annually. Managed by Siam Cement (Ta Luang) Company Limited (STL) with investment from The Siam Cement Public Company Limited (SCG) and carbon finance from the KliK Foundation, the project involves key stakeholders such as Thailand's Department of Climate Change and Environment (DCCE) and Switzerland's Federal Office for the Environment (FOEN). The production and monitoring commencing is schedule to begin in November 2027, measured against a conservative reference scenario of blended hydraulic cement with an 84% clinker factor. The project demonstrates additionality by overcoming technical and market barriers, addressing a critical financial gap where the 2.12% internal rate of return (IRR) without carbon finance falls well below the 6.63% benchmark. Monitoring is conducted under the Premium T-VER methodology (T-VER-P-METH-08-01), utilizing an automated SCADA system to track production volumes and energy consumption. |
| Project investment cost | 1,048.27 MTHB ( USD 31,970,210) |
| Estimated Greenhouse Gas Emission Reductions/Removals | 102,488 tCO2eq/y |
| Crediting period | ☒ 5 years ☐ 15 years ☐ others years XXX November 2027 - 31 December 2030 |

| Details of report preparation |  |  |
| --- | --- | --- |
| Finish date | 19 May 2026 |  |
| Version | 1.0 |  |
| Name of reporter | Name | Ms. Chananda Homklinchan |
|  | Position | Regional Manager, Climate Projects - SEA |
|  | Organization | Southpole Group (Thailand) Co.,Ltd |
|  | Telephone | +66 (0)2 219 3791 |
| Validation and Verification Body (VVB) |  |  |
| Name of VVB | # TEAM Consulting Engineering and Management Public Company Limited. |  |

| Details of the project participant (In case there is more than 1 project participant, please add the list of names) |  |
| --- | --- |
| Project participant | The Siam Cement (Ta Luang) Company Limited |
| Contact person | Weerakorn Saitep |
| Position | Chief Financial Officer - SCG Cement and Green Solutions |
| Address | 1516 Pracharat 1 Road, Wong Sawang Sub-district, Bang Sue District, Bangkok 10800, Thailand |
| Telephone | +66 36 240 888 |
| Fax | - |
| E-mail | weeranks@scg.com |

| Details of the project participant (In case there is more than 1 project participant, please add the list of names) |  |
| --- | --- |
| Project participant | Carbon Coordinating Managing Entity (Co) Ltd., Thailand (100% owned by South Pole Group) |
| Contact person | Ladaporn Kunikakorn |
| Position | Regional Director - APAC |
| Address | Unit 3A, Evergreen Place 318 Phayathai Rd, Khwaeng Thanon Phetchaburi, Ratchathewi, Bangkok 10400 |
| Telephone | +66 (0)2 219 3791 |
| Fax | - |
| E-mail | registries@southpole.com |

Contents

TEAM Consulting Engineering and Management Public Company Limited.	4

Part 1: Project Information	5

Part 2 T-VER Methodology	32

Part 3 Calculation of greenhouse gas reduction	36

Part 4 Monitoring of project	61

Part 5 Local Stakeholder Consultation	79

Appendix 1 Supporting documents/evidence	83

# Part 1: Project Information

  - Project Information

1.1.1 Objectives of the project activities

The project is the manufacturing of LC3 cement in replacement of Ordinary Portland Cement (OPC) and blended hydraulic cement which contributes to the reduction of CO2 emissions mainly by substituting energy intensive clinker content with calcined clay. During the clinker production process, limestone is heated in a kiln and results in the breakdown of calcium carbonate into calcium oxide and carbon dioxide. Therefore, the lower usage of clinker in cement products reduces the CO2 emissions released from the clinker burning and also requires lesser fossil fuel in the process. Significant energy savings are achieved because the calcined clays are more malleable and are heated at approximately 700-800 °C, whereas the manufacture of clinker requires a temperature of 1400-1500 °C.

1.1.2 Brief description of the organization’s activities related to the project

The project's boundary is in Saraburi Province, where implementation activities are carried out at two existing plants owned by Siam Cement (Ta Luang) Company Limited: Ta Luang Plant (TL) and Khao Wong Plant (KW), as shown in Figure 1. In TL plant, there are Kiln No.5 and No.6 currently under the operation.

Due to technical and operational feasibility, Kiln No. 6 at the TL plant will undergo modifications to facilitate clay calcination, thereby serving as a clinker substitute. The TL plant's clinker production capacity is insufficient to produce 2 million tons of LC3 cement which is the maximum capacity of LC3 production. Consequently, the calcined clay will be transferred to the KW plant for final blending into cement products.

TL plant

Clay mine

KW plant

Figure 1 - Locations of the manufacturing activities of the project

1.1.3 Operation prior to the implementation of the T-VER project.

Under the present operational framework, TL and KW plants manufacture limestone-based cement, encompassing blended hydraulic cement and Ordinary Portland Cement (OPC). The blended hydraulic cement means the cement which forms and hardens from reacting with water . In order to control the quality standard and to build up confidence among users of blended hydraulic cement, TIS 2594  defines that cement manufacturers must control  the quality of strength as according to the specified age and to control the expansion of cement at the age of 14 days, which is the advantage of hydraulic cement TIS 2594 which its effectiveness can be developed to respond to the need of each type of construction as according to its objective. The production operation is detailed in Figure 2.

Ta Luang Plant (TL)

The TL plant conducts the operation of two kilns, TL-5 and TL-6, with a daily clinker production capacity of 4,000 tons per kiln. Currently, the TL-5 production line manufactures an annual output of 1.0 to 1.2 million tons of cement, including OPC, blended hydraulic cement, and other cements. Conversely, the TL-6 production line which possesses an equivalent capacity was closed in 2023 and presently has no ongoing production.

Khao Wong Plant (KW)

The KW plant presently operates a single production line utilizing KW-1 kiln, which has a clinker production capacity of 11,000 tons per day.  The KW plant produces a range of cement products, including OPC, blended hydraulic cement, and mortar, with an annual output of 3.4 - 3.7 million tons.

To reduce carbon emissions and leverage carbon finance, the project will retrofit the inactive TL-6 kiln to produce calcined clay. This calcined clay will be transported to the KW plant and blended with a portion of the clinker from the KW-1 line to produce LC3 cement. All other existing production remains unchanged, ensuring the transition is a technical substitution within the current operational framework.

Figure 2 - The diagrams representing initial activities of the project

1.1.4 Greenhouse gas reduction activities, sources and quantities of raw materials, and number of the operation days of the project.

The project involves the production of LC3 cement as a replacement for OPC and other blended hydraulic cements. This process utilizes calcined clay as the primary replacement material.

The raw clay is sourced from the Pang Asok Clay Mine, which operates under concession licenses 28820/16248, 28821/16249, 28822/16250, and 28823/16251. The mine provides a robust supply chain capacity of 328,300 tons per year, ensuring the project meets the blending requirements for large-scale cement production without raw material shortages. The plants operate continuously for 356 days annually.

Figure 3 - The activities within the project boundary

Table 1 The project timeline for each implementation stage

| Stages | Descriptions | Indicative timeline |
| --- | --- | --- |
| 1. Technical readiness development | The development of LC3 technology began with comprehensive research, including literature reviews and laboratory work. Following this, SCG’s project team developed the specific formulation and pilot-scale production processes. The final stages involved pilot plant operations, prototype testing, and market readiness assessments. | 2013 - 2024 |
| 2. Feasibility study | The feasibility of LC3 cement in Thailand was conducted based on the technical and financial aspects, based on Thai standard cement industry assumptions and average costs in the Thai cement market. | August 2024 |
| 3. Local stakeholder consultation | Local stakeholder consultation was conducted with the relevant stakeholders, in accordance with Premium T-VER guidelines. | May 2025 |
| 4. Environmental Impact Assessment (EIA) study | EIA of the modification of the kiln burner and the production of LC3 cement is being conducted. It will be approved before the project construction starts, according to Thai regulations. | June 2025 - March 2026 |
| 5. Expected first financial expenditure | The first payment for the kiln burner modification. | End of May 2026 |
| 6. Expected equipment fabrication process | Commence fabrication of kiln burner for project operation. | End of May 2026 |
| 7. Expected application submission to FOEN and DCCE to request a Letter of Authorization (LoA) | The validated MADD and PDD, final validation report and required documents will be submitted to FOEN and DCCE for authorization. | July 2026 |
| 9. Expected start of the monitoring | This date marks the planned plant commissioning, which begins the production of LC3 cement and therefore the start of monitoring. | November 2027 |

1.1.5 Expected amount of greenhouse gas emissions/removal

The expected emission reductions per year are detailed in the calculation sheet provided in Annex 1.

Table 2 Expected emission reductions per year

| Year | Expected ITMOs (t CO2e) | Expected ITMOs (t CO2e) – withholding by DCCE 10% |
| --- | --- | --- |
| 2027 | 2,200 | 2,200 |
| 2028 | 127,300 | 127,300 |
| 2029 | 163,000 | 163,000 |
| 2030 | 163,000 | 117,410 |
| Total | 455,500 | 409,950 |

Thailand’s NDC Action Plan on Mitigation 2021-2030 sets a target of reducing GHG emissions by up to 3% through carbon credit management mechanisms under the international cooperation frameworks. This reduction is in addition to the unconditional target (of 30%) that Thailand aims to achieve through domestic efforts. Additionally, at least 10% of the total carbon credits accumulated throughout the transfer period will be retained. This retained portion will be deducted from the total amount of carbon credits to be transferred in the final year (the NDC target year) to facilitate corresponding adjustments, preventing double counting to ensure that this measure does not hinder the achievement of the Action Plan’s targets.

1.2 Project Boundary

1.2.1 Technology and key equipment

The LC3 cement production system is divided into three integrated processes: (A) Calcined clay production at the TL Plant, (B) Clinker production at the KW Plant, and (C) LC3 cement grinding and dispatching. The process is designed to maximize the use of existing plant assets while integrating calcined clay as a supplementary cementitious material in a controlled, industrially proven manner. The details of each process are also described as illustrated in Figure 4.

Process A: Calcined Clay production – TL plant

Calcined clay production at the TL plant converts raw clay into a highly reactive supplementary cementitious material through controlled calcination. The process consists of clay preparation, fuel preparation, calcination, cooling, and material handling.

A1. Clay feeding and preparation

Raw clay is delivered by trucks from the mine to the TL plant and unloaded into the existing receiving hopper. The material is initially reduced in size by the existing crusher located beneath the hopper. Crushed clay is then conveyed by belt to the mixed material storage yard, where the existing stacker distributes the crushed clay into stockpiles.

A reclaimer continuously extracts clay from the stockpile and feeds it onto a conveyor system passing through a roller crusher (to be installed), which further reduces the particle size to below 20 mm. The finely crushed clay is then stored in two intermediate bins (to be installed) to ensure stable kiln feeding.

From these bins, the clay is extracted via a weight feeder (to be installed) that precisely controls the feeding rate according to production demand. The clay is conveyed onward and transferred to the existing screw conveyor, where it is prepared for feeding into TL-6 kiln.

A2. Burning

The calcination process takes place in the rotary kiln, TL-6 kiln (to be modified), supported by an existing grate cooler. The clay, together with a controlled dosage of colour-treatment material, is fed via a screw conveyor directly into the kiln. Inside the kiln, calcination occurs at controlled temperatures in the range of 700–900°C, transforming raw clay into reactive calcined clay. To support market acceptance during the initial deployment phase, a colour control technique is implemented. This technique is based on an ash-assisted reduction technique (Dynamis/Loesche concept), which is an industrially proven solution compatible with rotary kiln technology. In this process, a small amount of fine ash from coal is mixed with the clay to prevent the formation of reddish or brown hues in the calcined product. This colour treatment process emits CO2 during the combustion phase which accounts for 35% of heat required for clay calcination. Detailed technical descriptions of the technique are provided in Annex 2 for further reference.

Following calcination, the calcined clay enters an early cooling zone where recirculated low-oxygen gas (approximately 5–8% O₂) combined with rapid water spraying creates fast quenching conditions. This two-step reduction and quenching mechanism effectively stabilizes the grey colour of the calcined clay. The calcined clay then exits the kiln and enters the grate cooler, where its temperature is further reduced using ambient air supplied by cooling fans and controlled water spraying. Recirculated kiln exhaust gas at approximately 200°C is also utilized to optimize thermal efficiency.

After cooling, the calcined clay temperature is reduced to approximately 100°C. The material is then conveyed by existing chain and pan conveyors to a calcined clay silo (to be installed), from which it is dispatched for cement production.

A3. Fuel feeding system

Thermal energy for clay calcination is generated using a biomass-based main burner (to be modified) at the TL-6 kiln inlet. The main burner is a modified unit designed to achieve the temperature and atmosphere conditions required for clay calcination, particularly moisture control and oxygen balance.

Biomass fuel, primarily rice husk, is sourced from the plant’s existing biomass storage. It is conveyed by the existing belt conveyor to a hopper and then transported pneumatically to a hopper (to be installed) located near the kiln. A weight feeder (to be installed) meters the biomass accurately before it is conveyed pneumatically into the kiln burner. The kiln burner converts the biomass into hot process gas, which is directed into the kiln to provide the required thermal input for calcination.

A4. Gas treatment

The calcined clay production process at the TL plant is equipped with a comprehensive gas treatment system to control atmospheric emissions and ensure compliance with applicable environmental regulations and ISO 14000–aligned environmental management standards.

Exhaust gases generated from the rotary kiln No 6 are first directed to the existing main bag filter, where particulate matter is effectively removed through high-efficiency fabric filtration. This system is designed to capture fine dust generated during calcination and material handling, thereby minimizing particulate emissions to the atmosphere. Following particulate removal, the cleaned gas stream is routed to a wet scrubber system (to be installed). The wet scrubber provides an additional stage of gas cleaning by removing residual dust, soluble gaseous components, and acid-forming compounds, as applicable. This process also contributes to temperature control of the exhaust gas prior to discharge.

The gas treatment system utilizes existing and proven technologies commonly applied in cement and calcination plants worldwide. This integrated emission control approach ensures that STL maintains adherence to national air quality mandates, ensuring all point-source emissions consistently fall below the thresholds established by the Factory Act of 1992 and international environmental management standard e.g. ISO 14000:2015.

Confidential

Figure 4 – LC3 production process

Process B: Clinker production – KW plant

Clinker production at the KW plant follows the conventional limestone-based cement manufacturing process, which is widely applied in cement plants worldwide. All equipment, operating conditions, and material handling systems are part of the plant’s normal operation and remain unchanged under the LC3 project.

B1. Raw material feeding

Raw material feeding for clinker production is carried out using the standard limestone-based process which is a normal cement manufacturing practice globally. Limestone, together with other conventional raw materials, is extracted from storage areas and conveyed through the plant’s crushing, conveying, and dosing systems.

The materials are proportioned according to the plant’s existing raw mix design and fed continuously into the preheater system. All material handling, storage, and feeding operations utilize existing equipment and control systems, ensuring stable and proven operation consistent with worldwide clinker production practice.

B2. Burning process

The burning process is conducted using an existing KW-1 kiln, utilizing globally established technology for limestone calcination and clinker production. The raw materials are preheated and then fed into the KW-1 kiln, where it undergoes calcination and clinkerisation at high temperatures typically used in conventional cement manufacturing.

Fuel combustion, heat transfer, and process control are managed through the KW plant’s KW-1 kiln, burner, and cooler systems. After leaving the KW-1 kiln, the clinker is cooled in the clinker cooler (grate cooler) and conveyed to clinker storage silos. The stored clinker is subsequently used in cement grinding operations, including limestone-based and LC3 cement production.

Process C: LC3 Cement production – grinding and dispatch

LC3 cement production integrates calcined clay into the existing cement grinding circuit, ensuring minimal disruption to the current operations. The process C occurs exclusively within the KW plant.

C1. Grinding and cement dispatch

Calcined clay is supplied from the TL plant and transported by trucks to the KW plant and stored in the calcined clay storage (to be installed). It is then conveyed through the existing plant conveying system to a calcined clay bin (to be installed) equipped with a dedicated weight feeder to ensure accurate control of the LC3 formulation to meet with quality and performance requirements. Clinkers and other additives are handled through the existing cement plant systems.

Clinker, calcined clay, limestone, and gypsum are ground together in the existing cement mill system. The ground material is classified by a separator to achieve the required fineness. The final product, LC3 cement, is conveyed pneumatically to cement storage silos (to be installed silos and existing silos). LC3 cement is dispatched using existing logistics systems through bulk loading, big bag filling, or paper bag packing, ensuring full compatibility with current distribution channels.

The main equipment to be installed and modified is listed in Table 3.

Table 3 List of main equipment to be installed and modified

|  | Process | Equipment to be modified or installed |
| --- | --- | --- |
| A | Calcined Clay production – TL plant | A1: Clay feeding and preparation Roller crusher – to be installed Intermediate bins – to be installed A2. Burning Weight feeder – to be installed Colour treatment system – to be installed Calcined clay silo – to be installed A3. Fuel feeding system Kiln burner for calcined clay calcination - to be modified Weight feeder – to be installed Hopper – to be installed A4. Gas treatment Wet scrubber – to be installed |
| B | Clinker production – KW plant | There is no modification or installation. |
| C | LC3 Cement production – grinding and dispatch | Calcined clay storage – to be installed Calcined clay bin – to be installed Weight feeder – to be installed LC3 cement storage (silo) – to be installed partially |

1.2.2 The legal entities involved

Siam Cement (Ta Luang) Company Limited, the owner of the project is the official entity for signing the Mitigation Outcome Purchase Agreement (MOPA) with the KliK Foundation. The initial financing for the project was provided by SCG. The project is located at the TL plant and KW plant Saraburi Province, Thailand, and its activities are carried out by The Siam Cement (Ta Luang) Company Limited at both the TL and KW plants. STL is also responsible for all monitoring and data collection related to the project.

The emission reduction program for this project is managed by CCME Co., Ltd., which has an agreement with STL to facilitate the generation of mitigation outcome units. Under the agreement, CCME receives operational data from the plant operator, STL. Using additional third-party data and approved methodology by TGO, CCME produces the documents that are required to request the issuance of mitigation outcomes and ITMO transfer recognition. Mitigation outcomes will be issued by DCCE and TGO into the registry account of the project owner (STL), from where they will be transferred to the Swiss Government, into the account of the acquiring entity (KliK Foundation).

An overview of the contractual arrangements among participants is described in Table 4 and Figure 5.

Table 4 An overview of the contractual arrangements among participants

| Item | Name | Roles |
| --- | --- | --- |
| Buyer of the ITMOs | KliK Foundation | The KliK Foundation will be the buyer of the ITMOs generated by the project. |
| Owner | The Siam Cement (Ta Luang) Company Limited (STL) | STL) will be the owner of the project and recipient of Internationally Transferred Mitigation Outcomes (ITMO). STL is also the owner of the mitigation outcomes, and it is also the registry account holder in which the project activity is registered to, and where the carbon credits will be credited towards. |
| Manager/MRV | The Siam Cement (Ta Luang) Company Limited (STL) | STL operates the project at TL plant and KW plant. The company’s role also includes monitoring and reporting activity data to the carbon consultant to facilitate ITMO origination process. |
| Technology Provider | To be confirmed | Technology providers are companies that deliver, install, and maintain technology solutions related to the modification of kilns and installation of silos and cement mixing machineries. |
| Main government agency | Host country: Department of Climate Change and Environment (DCCE) of Thailand Thailand Greenhouse Gas Management Organization (TGO) | The Department of Climate Change and Environment (DCCE) of Thailand, under the Ministry of Natural Resources and Environment (MoNRE), is responsible for the authorisation of project activities and international transfer of ITMOs under the Article 6 of the Paris Agreement. Thailand Greenhouse Gas Management Organization (TGO), the governance entity of the Premium T-VER Standard administers the Thai registry system and facilitates the international transfer of carbon credits by cancelling the carbon credits and recording the transfer data in the carbon credit registry system. |
|  | Buyer country: Federal Office for the Environment (FOEN), Switzerland | Responsible for the authorisation of project activities and international carbon credits transfers on behalf of the buyer country. |
| Investor | The Siam Cement Public Company Limited (SCG) | SCG will allocate the upfront financing needed to start the activities. |
| Carbon consultant | Carbon Coordinating Managing Entity Limited (CCME), fully owned subsidy of South Pole | Carbon Coordinating Managing Entity Limited (CCME) is a subsidiary of South Pole Holding AG, who is a professional service provider that helps businesses achieve net zero with climate solutions and environmental consulting. CCME collects data from STL and applies the applicable methodologies and processes as well as assisting STL during validation, verification. Additionally, CCME assists with the request for the authorisation and fulfilment process to achieve the international transfers of the mitigation outcomes. |

Table 4 Actors involved in the implementation of the Project

Figure 5 - The contractual arrangements diagram

1.3 Double Counting

The project is operationalized in a way to prevent double counting/use as follows:

Preventing double counting from request for credit issuance

According to the Implementing Agreement under Article 6.2 of the Paris Agreement between the Swiss Confederation and the Kingdom of Thailand, the project must not be registered with more than one crediting mechanism. This project is not registered under any carbon standard. A declaration on no double issuance of mitigation outcomes under other GHG programs is provided by STL as in Annex 3.

Additionally, under the Article 6.2 Thai-Swiss cooperation, TGO is responsible for the issuance and cancellation of MOs, facilitating their transfer as ITMOs to the KliK Foundation’s account in the Swiss Registry. The application of Corresponding Adjustments ensures robust accounting, preventing the double counting of credits between the host and acquiring nations.

Preventing double counting from reporting or declaring voluntary GHG emission reduction performance

Apart from a 10% contribution to Thailand NDC, STL commits that no ITMOs generated by this project will be reported toward STL’s and/or SCG’s corporate GHG inventory, Scope 1–3 targets, or environmental product claims. STL will disclose this project in its corporate sustainability reporting as ‘ITMO-transferred reductions with corresponding adjustment’, ensuring full transparency and avoidance of double claiming.

To prevent any attestations issued for emission reductions that have already been offset for added ecological value a guideline on LC3 cement, marketing campaign has been developed to avoid double claiming and is provided in the Annex 4. This guideline prohibits the use of vague or misleading green claims, emission reductions claims toward corporate climate targets, country-level NDCs, industrial targets (e.g., Saraburi Sandbox) and Scope 3 targets of customers. Instead, the communication will focus exclusively on product-level carbon performance (Carbon Footprint of Product (CFP)/Environmental Product Declaration (EPD)). Furthermore, STL commits not to increase product prices with a premium or market the product with misleading 'carbon neutral' labels. STL will also make sure to inform and prevent its stakeholders and customers from potentially claiming mitigation outcomes from the project.

Prevention double counting from Renewable Energy Certificate (REC) certification

The renewable energy used within the project boundary is from STL’s own facility and is not registered under I-Track Foundation for Renewable Energy Certificate (REC). STL will not register or claim RECs from part or all the activities within the project’s boundary during the crediting period of the project and shall prevent its stakeholders from doing so.

The project activity in this project used to register or in the process of registering other carbon other international climate mitigation mechanisms such as Development Mechanism (CDM), Voluntary Carbon Standard (VCS), Gold Standard etc. or Renewable Energy Certificates (REC)

☒   No

☐   Yes

Project Title

Scheme

Crediting period that is issued

  - Additionality

    - The regulations relevant with the project’s operation

Thailand’s climate policy framework operates as a strategic hierarchy designed to ensure long-term goals are met through immediate, technical action. The LT-LEDS 2022 establishes the long-term strategic anchor, committing the nation to Carbon Neutrality by 2050 and Net Zero Emissions by 2065. This vision is operationalized for the 2030 horizon through the Second Updated NDC (2022), which elevates the national mitigation target to 30–40% and formally integrates Article 6.2 market mechanisms to facilitate international cooperation. Finally, the NDC Action Plan (2021–2030) provides the technical granularity required for implementation, translating these international pledges into sectoral roadmaps and measurable KPIs—specifically identifying clinker substitution in the cement industry as a key lever for industrial decarbonization. The impact of the frameworks is demonstrated in Table 5-6.

Table 5 Thailand’s Long Term Low Greenhouse Gas Emission Development Strategy (LT-LEDS) 2022

| Policy/ law/ measure No. 1: Thailand’s Long Term Low Greenhouse Gas Emission Development Strategy (LT-LEDS) 2022 |  |
| --- | --- |
| Description | Thailand’s LT-LEDS 2022 sets out the country's long-term pathway toward carbon neutrality by 2050 and net-zero greenhouse gas emissions by 2065. The strategy emphasises the economy-wide mitigation across energy, industry, waste, agriculture, and forestry, supported by enabling policies, investment, and international cooperation. For the IPPU sector, key mitigation measures in the IPPU sector focus mainly on clinker substitution and substitution of high GWP refrigerants. |
| Type | National strategic plan |
| Impact | This strategy indicated that the DIW (Department of Industrial Works), under the Ministry of Industry, is driving the green economy by promoting business transition into Green Industry. Social responsibility and environmentally friendly operations within the industrial sector using technologies and innovations to maximize resource efficiency and transition to eco-friendly manufacturing is encouraged. In the IPPU sector, one of the main key mitigation measures is on clinker substitution. The Thai cement industry intends to mitigate greenhouse gas by continually working together with ambition towards carbon neutrality on clinker replacement measures by stimulating blended hydraulic cement consumption in accordance with the national plan. TCMA in collaboration with the Council of Engineers of Thailand, the Engineering Institute of Thailand under the Patronage of His Majesty the King, and the Thai Concrete Association (TCA), with the support of the Office of Natural Resources and Environmental Policy and Planning (ONEP), Ministry of Natural Resources and Environment, joined forces with 31 alliances from public, professional, industry and academic sectors to push forward carbon neutrality efforts by launching “MISSION 2023” to mitigate 1 MtCO2 within the year 2023 by encouraging all sectors to use environmentally friendly cement called ‘Hydraulic Cement’ in all types of construction projects in Thailand. Such collaboration is further enhanced by construction standard and production standard improvement, capacity building and product research and development. |
| Implementation plan/schedule | The plan was adopted in November 2022 and implemented toward 2065, supported by successive NDCs and sectoral action plans. |
| Assessment of impact on the proposed Project | As a key strategy in the LT-LEDS, clinker substitution actively contributes to the 2065 net-zero target. This project is therefore aligned with LT-LEDS’ goals. The MISSION 2023 only launched the introduction of blended hydraulic cement to replace OPC, but as explained under Section 3.1, this plan has not been accompanied by neither financial support nor by mandatory measures. The transition has therefore been slow. Furthermore, the LT-LEDS does not refer neither to LC3 nor to further GHG emission reductions from clinker substitution. This project specifically delivers surplus reductions that exceed the requirements of the current plan. |

Table 6 Thailand’s second updated NDC (November 2022) and NDC Action Plan on Mitigation 2021 – 2030

| Policy/ law/ measure No. 2: Thailand’s second updated NDC (November 2022) and NDC Action Plan on Mitigation 2021 – 2030 |  |
| --- | --- |
| Description | Thailand enhanced its first NDC at the 26th Conference of the Parties (COP26) to fulfil the long-term temperature goal set in Article 2 of the Paris Agreement. The National Greenhouse Gas Emission Reduction Action Plan of Thailand, B.E. 2564 – 2573 (2021 – 2030) was formulated to serve as a sectoral framework for the country's greenhouse gas emission reduction efforts. |
| Type | Policy and policy-linked action plan |
| Impact | Thailand’s 2nd Updated NDC establishes a national mitigation target of 30–40% reduction in greenhouse gas emissions below the BAU scenario by 2030. The NDC Action Plan on Mitigation 2021-2030 provides the implementation framework for responsible agencies and identifies sectoral mitigation contributions amounting to approximately 222 MtCO2e by 2030, comprising 184.8 MtCO2e (33.3%) from domestic actions (unconditional contribution) and 37.5 MtCO2e (6.7%) from international support (conditional contribution). In addition, Thailand foresees the use of Article 6 mechanisms to support enhanced mitigation ambition beyond the unconditional and conditional contributions, with the Article 6 component capped at 3% (approximately 16.65 MtCO2e), resulting in a total mitigation ambition of up to 43% by 2030. The reduction target for the cement sector amounts to 1 MtCO2eq in the unconditional target. It comprises of the following measures. ● Clinker substitution in production process of the hydraulic cement: 0.9 MtCO2 ● Clinker substitution in production process of the ready-mix concrete: 0.1 MtCO2 There are no measures for the cement industrial industry in the conditional target. Furthermore, Thailand will continue vigorous efforts in its challenge to meet the long-term goal of carbon neutrality by 2050 and net-zero greenhouse gas emission by 2065. The NDC also indicates Thailand’s openness to explore international market-based cooperation in the context of Article 6 of the Paris Agreement, that promotes development and transfer of advanced technologies and innovation and provides access to financial resources that support Thailand's achievement of NDC and LT-LEDS. |
| Implementation plan/schedule | Thailand’s second updated NDC was submitted on 02 November 2022, and the NDC Action Plan on Mitigation was adopted by the Cabinet in December 2024. The implementation timeframe is 2021 - 2030. |
| Assessment of impact on the proposed Project | The project is within the scope of Thailand's NDC for 2021-2030 i.e. emissions reduction from the project will contribute towards Thailand's NDC. Progress reports from December 2025 (Annex 5) confirm that clinker substitution in hydraulic cement has delivered 0.4 MtCO₂ in GHG reductions as of 2023. Considering the increase of blended hydraulic cement in Thailand since 2022, Thailand is expected to achieve its unconditional NDC target of 1 MtCO2 by 2030 for the IPPU sector. The emission reductions generated from this project will not be attributed to Thailand’s unconditional NDC target. This project is implemented under the bilateral agreement between Thailand and Switzerland. The resulting emission reductions will be transferred to Switzerland and will be subject to a Corresponding Adjustment (CA). The application of this CA prevents any double counting towards Thailand's domestic unconditional targets. Furthermore, the project has received an official confirmation letter from the Department of Industrial Works (DIW), which guarantees the project's additionality to the national NDC (see Annex 6). Crucially, the project incorporates LC3 technology, calcined clay to replace clinker using technology (S3), which provides a greater reduction in clinker factor compared to traditional hydraulic cement (S1), which provides an additional, surplus reduction beyond the standard measures defined in the action plan. This project therefore goes beyond the measures identified for the unconditional NDC target and does not impact achievement of the unconditional NDC target. |

The Notification of the Ministry of Industry No. 6819 (2022) serves as the primary regulatory enabler for the project. It provides the legal framework necessary to transition from theoretical technical feasibility to large-scale industrial application as described in Table 7.

Table 7 Notification of the Ministry of Industry No.6819 (2022) RE: Subjecting utilisation of the blended hydraulic cement as a raw material in the industrial concrete and cement products.

| Policy/ law/ measure No. 3: Notification of the Ministry of Industry No.6819 (2022) RE: Subjecting utilisation of the blended hydraulic cement as a raw material in the industrial concrete and cement products. |  |
| --- | --- |
| Description | Under the Notification, several industrial concrete and cement products e.g., building, sanitary pipe, foundation pile, etc. could be produced using blended hydraulic cement qualified under Thai Industrial Standard (TIS) 2594-2556. |
| Type | Notification of the Ministry |
| Impact | Previously, TIS for various concrete products, including those used in buildings, sanitary pipes, and foundation piles, exclusively permitted the use of Portland Cement (TIS 15). However, this notification expands the accepted materials to include blended hydraulic cement certified under TIS 2594. |
| Implementation plan/schedule | Effective 1 December 2022 |
| Assessment of impact on the proposed Mitigation Activity | This notification signals the intention to promote greener cement by allowing the introduction of hydraulic cement, including LC3. While the production of LC3 aligns perfectly with this regulation, the project goes beyond the level of adoption currently encouraged by the government. As TIS 2594 already specifies the critical physical properties required for construction, LC3 can be certified under this current standard without the need for additional legislation. |

Furthermore, the International Carbon Credit Guideline (2025) establishes the formal mechanism for the project  to facilitate international transfers, as detailed in Table 8.

Table 8 International Carbon Credit Guideline

| Policy/ law/ measure No. 4: International Carbon Credit Guideline (2025) |  |
| --- | --- |
| Description | The National Committee on Climate Change Policy prescribes the carbon credit management guidelines for an international objective. |
| Type | Implementation guideline |
| Impact | Thailand’s International Carbon Credit Guideline establishes the national framework for the use and international transfer of carbon credits under Article 6 of the Paris Agreement and other international mechanisms. The guideline sets out principles and procedures for Thailand’s authorization, corresponding adjustment, accounting, and avoidance of double counting, and clarifies institutional roles of the Department of Climate Change and Environment (DCCE) and the Thailand Greenhouse Gas Management Organization (TGO) in managing international carbon credit transactions. The guideline further specifies that mitigation activities intended for international transfer must deliver mitigation outcomes additional to those included in Thailand’s NDC Action Plan, support Thailand’s LT-LEDS, and generate mitigation outcomes that are measurable, reportable, and verifiable. It also defines eligible project types and requires the use of an approved registry system for issuance and international transfer of carbon credits. This project falls under the project type (6) of the article 7: “Improvement of production process or management of industrial waste such as improvement of the process or technology of chemical or petrochemical production, production of low carbon cement, production of methanol using carbon dioxide or green hydrogen, reduction of the use of fluorinated gases by avoiding use, substitution, recovery, reuse, reclamation, and environmentally friendly disposal”. |
| Implementation plan/schedule | The Cabinet resolution granting approval was passed on 26 August 2025. |
| Assessment of impact on the proposed Mitigation Activity | The proposed project is designed in full alignment with the International Carbon Credit Guideline. It targets mitigation outcomes beyond Thailand’s unconditional and conditional NDC contributions, follows the national authorisation process for use of carbon credits for an international objective, and applies corresponding adjustment and registry requirements as prescribed. The project therefore supports Thailand’s participation in Article 6.2 cooperative approaches while maintaining environmental integrity, transparency, and consistency with national climate policy. |

    - Additionality to NDC

In accordance with Thailand’s Second Updated NDC (November 2022) and the NDC Action Plan on Mitigation (2021–2030), this project is well-aligned with the national climate mitigation targets and strategy. The project supports the LT-LEDS strategy for clinker substitution, actively contributing to the national 2065 Net-Zero target.

By utilising LC3 technology for clinker substitution, the project generates surplus GHG reductions which go beyond the clinker substitution measures defined in the current sectoral action plans of the NDC Action Plan on Mitigation (2021–2030). In addition, the project satisfies the criteria for international transfer under the International Carbon Credit Guideline (2025) as a Type (6) project (Improvement of production processes for low-carbon cement). This alignment ensures the project’s eligibility for DCCE authorisation, and the subsequent cabinet approval required for international credits transfer.

Finally, in initial technical feedback from DCCE (refers to Annex 7), (letter No. 0804/3395, dated 14 March 2025), DCCE confirmed that the proposed mitigation activity is within the scope of Thailand's GHG inventory and requested that STL seeks further confirmation regarding NDC additionality from DIW. FOEN also requested to obtain a “written confirmation by the host country government on the NDC alignment” (letter CH-3003 Bern (Annex 8). To clarify the additionality of the project and its alignment with Thailand’s NDC target within the IPPU sector, STL has therefore engaged in consultations and sought clarification from DIW in which STL received a Letter of Support dated 27 August 2025, that “the project is not in the scope of the measures under the NDC Action Plan”, as provided in Annex 6. Therefore, this MA has obtained confirmation from both DCCE as the DNA and DIW as the relevant sectoral agency, that the project is not included in the unconditional measures identified for the unconditional NDC target.

    - Financial additionality

      - Business model and role of carbon

The business model for this project involves modification of the existing equipment and production lines for the new LC3 cement production of approximately 2,010,000 tons annually. The new cement will be sold by STL to a broad customer base within Thailand’s domestic construction market including both private and public sectors. Majority of the product is sold in big volume to    large construction companies, infrastructure developers, cement distributors and state-owned enterprises (Business-to-Business (B2B)) and customers. The smaller part is bagged and sold to particularly small contractors and individual consumers for residential and small-scale construction (Business-to-Consumer (B2C)). The main financial flows among the participants are demonstrated in Annex 9 and described as follows.

- Upfront financing and investment in the technology: Provided by SCG’s internal funds

- Operational revenue: Derived from the sale of LC3 cement to customers in the green building and infrastructure sectors and operation savings.

- ITMO revenue: Generated from the sale of ITMOs, which is used to subsidise investment costs and maintain competitive pricing.

The upfront financing and investment in the technology or the capital expenditure (CAPEX) of USD 31,970,210 is used in modifying the existing production line and additional equipment specifically designed for calcined clay production, such as a calcined clay kiln burner, storage silos and conveyors. Additionally, the CAPEX is used for modification of the cement blending process, intergrinding equipment (see Annex 10). These costs are from the proposals received from the selected suppliers and contractors including the contingency and the interest during construction (IDC).

For the operational revenue, the market price of the traditional blended hydraulic cement is approximately THB 1,800/ton (approximately USD 56). For the new LC3 cement, despite its unique value proposition appealing to environmentally conscious consumers in the construction industry, market acceptance remains a challenge due to the limited willingness to pay a premium price for low-carbon or green products for big volume purchase and a concern on the reliability of the new product. With these reasons, together with the request from DCCE, the selling price of the LC3 cement remains the same as the blended hydraulic cement making the sales revenue of the project the same as the baseline scenario. On the other hand, the project has some revenue from reduced operation expenditure (OPEX), where, even though the costs for electricity and other maintenance costs increase, the project has some savings from reduced costs in raw materials, fuel, and third-party contractors, resulting in the overall saving operational cost per ton of LC3 produced compared to hydraulic cement reduced by USD 1.49/ton. These costs are retrieved from market data i.e. Newcastle Price Index and energy rate from Energy Regulatory Commission and the company 3-5-year historical operation cost with inflation rate and other conservative assumptions. For the previous reasons, the only operational revenue of the project is from the operational cost saving, which, compared to the CAPEX, is not sufficient to make the project financially feasible.

The funds are provided by SCG, in accordance with SCG group’s internal cash management policy, which identifies four primary sources of funds: cash flow from operations, capital increase from shareholders, intercompany loans, and bank loans. Among these, for subsidiaries in Thailand, intercompany loans are the most commonly used source of funding due to their flexibility, speed of execution, ability to optimize liquidity within the group.

Despites that, the interest rate is applied to intercompany loans to ensure the following:

- Compliance with transfer pricing regulations (arm’s length principle)

- Fair compensation to the lending entity

- Proper financial discipline and performance measurement across entities

The intercompany loan’s interest rate is typically benchmark-referenced (e.g. based on market rates such as bank lending rates (Annex 11, Section A) or cost of funds (Annex 11, Section B and C)) or adjusted to reflect the borrower’s risk profile, loan tenor, and internal policy. As a result, the rate is generally aligned with market conditions but may be slightly lower or more favourable than external bank loan rates due to lower transaction costs and internal risk considerations. For this project, the interest rate is 3.4 % which is lower than the MLR from major banks of 6.3%-6.825%.To become financially feasible, carbon finance, through support funding from the KliK Foundation, is necessary to improve the internal rate of return (IRR) and reduce the payback period of the project. The carbon revenue will be contractually accrued to STL, as the project proponent and the primary implementing entity of the project. This revenue will be used to subsidise the sales revenue, keeping its selling price comparable to traditional cement and improving the cashflow of the project, and resulting in higher IRR and less payback period. The allocation and financial impact of these carbon revenues will be tracked and substantiated through the monitoring procedures detailed in Section 5.6 of this MADD. Under these terms, STL remains the sole legal beneficiary of the ITMOs proceeds, which are strategically ring-fenced to support the ongoing operation of the project and to fulfil debt service obligations including principal and interest to SCG. This structure ensures that the project maintains financial viability at the subsidiary level while enabling the parent company to recover its investment through a transparent repayment schedule.

The contribution of the KliK Foundation under Article 6 of the Paris Agreement plays a pivotal role in enabling the implementation and long-term viability of the project. Its support directly addresses and abates the barriers that would otherwise prevent the market deployment of LC3 cement in Thailand. The use of ITMO revenue is always directly or indirectly linked to barriers to which the relevant actors play a specific role in its abatement. In summary, the KliK Foundation’s contribution under Article 6 enables SCG’s investment that will transform an otherwise non-bankable climate activity into a viable business case, while simultaneously addressing systemic barriers in market acceptance, policy alignment, and technical readiness.

      - Investment analysis

This assessment follows the methodologies outlined in UNFCCC’s standard “A6.4-SBM015-A11 Demonstration of Additionality in Mechanism Methodologies (Version 01.0) which aligned with the financial analysis framework provided by FOEN and CDM Tool 27 ("Methodological Tool: Investment Analysis" Version 14.0). The “A6.4-SBM015-A11 Demonstration of Additionality in Mechanism Methodologies (Version 01.0)” provides three analysis methods: (a) Simple Cost Analysis, (b) Benchmark Analysis and (c) Investment Comparison Analysis.

(a) Simple Cost Analysis is only applicable when the mitigation activity and its alternatives generate no financial or economic benefits beyond carbon finance income. As this project generates income from cost savings, it has revenues beyond ITMO sales, making (a) Simple Cost Analysis unsuitable.

With the remaining option (b) and (c), the project uses the CDM Methodology Tool 27 to further identify the approach for financial additionality demonstration. Based on Tool 27, “local commercial lending rates or the weighted average cost of capital (WACC) are appropriate benchmarks for a project IRR” (paragraph 16). As there is no other technology that can provide a similar outcome as the project, this project therefore uses (b) Benchmark Analysis option, using WACC.

WACC calculation

The WACC is determined based on the information provided in Table 9 using CDM Tool 27. The details of the assessment are demonstrated in the Annex 9.

Table 9 The Operating expenditure

| Description | Value | Reference |
| --- | --- | --- |
| Amount of equity in the project | USD 19,182,126 | Investment Analysis (Annex 9) |
| Amount of debt in the project | USD 12,788,084 | Investment Analysis (Annex 9) |
| Cost of equity | 8.23% | CDM Tool 27 |
| Cost of debt | 3.4% | Internal loan |
| Corporate tax | 20% | Investment Analysis (Annex 9) |
| Value of total investment (CAPEX) | USD 31,970,210 | Investment Analysis (Annex 9) |
| WACC | 6.63% | Calculation according to CDM Tool 27 |

The WACC has been calculated to be 6.63% based on the aforementioned data.

IRR estimation without ITMOs

The financial viability of this project was evaluated using the project IRR compared to WACC. The assessment period is set at 20 years, consistent with the anticipated operational lifespan of the equipment involved in the project. The calculation of the project IRR incorporates factors such as CAPEX, OPEX, sales revenue, cost savings, and taxes. Comprehensive financial details are presented in the Investment Analysis (Annex 9), which has been derived from available data and forecasts, acknowledging potential influences from the economic and market environment. As the project operation will utilize the existing facilities and operational structure, it has minimal changes in operating personnel, new permanent operational functions, additional routine operational activities and new maintenance systems beyond normal plant practice. As a result, the project has small operational savings of USD 1.49/ton LC3 cement produced.

Table 10 The Capital Expenditure

| Description | Value (USD) | Reference |
| --- | --- | --- |
| Kiln Restoration | 3,802,813 | Investment Analysis (Annex 9) |
| Clay Calcination | 21,799,367 | Investment Analysis (Annex 9) |
| KW LC3 Mixing - Inter grinding | 6,368,031 | Investment Analysis (Annex 9) |
| Depreciation | 1,598,510 USD/year | Investment Analysis (Annex 9) |

From the above information, the project IRR is calculated as 2.45%, which is below the established 6.63% WACC benchmark. Consequently, it can be affirmed that the project is not economically or financially feasible without the revenue generated from ITMOs and qualifies as financially additional under UNFCCC’s standard “A6.4-SBM015-A11 Demonstration of Additionality in Mechanism Methodologies (Version 01.0)”, CDM Tools 01.

IRR estimation with ITMOs

From Table 10, with ITMOs revenue, the IRR of the project will be improved from 2.45% to 6.70% which overcome WACC benchmarks making it become financially feasible and attractive for investment. This proves the need of carbon finance to enable the project investment.

Table 11 Benchmark analysis: Carbon Revenue Impact Component Activity Level

|  | Without ITMO Revenue | With ITMO Revenue |
| --- | --- | --- |
| Additional Revenue from sale of product and savings (USD) | 0 | 0 |
| Program Cost (USD) | 34,305,995 | 39,274,715 |
| Free cashflow (unlevered) (% of program cost) (20 years) | 9,234,355 | 22,584,595 |
| Variable cost savings (per year) | 2,287,380 | 2,287,380 |
| Project IRR | 2.45% | 6.70% |
| Payback Time (year) | 16.56 | 10.40 |
| Share of costs covered by carbon finance (% of program cost) | 0% | 46.26% |

Sensitivity analysis

The CAPEX for this project is divided into kiln restoration, the clay calcination, and KW Plant LC3 mixing – intergrinding. These costs include engineering work, machinery, construction/erection and installation, as well as the contingency and interest during construction. The OPEX for this project is divided into raw materials, fuel, power, maintenance, and third-party contractors.

For this PDD, the business model is run based on the difference in costs and revenues between the baseline activity and the project. As there the operation cost per ton LC3 is cheaper than blended hydraulic cement, there is no OPEX and the savings from OPEX is considered revenue.

Among these costs, CAPEX is the most sensitive while in the OPEX, the fuel cost is the most sensitive given it represents 40% of the OPEX. In case of any changes in fuel cost, the project has 5% contingency to remain financially viable.

Table 12 Economic Feasibility – Sensitivity Analysis – Project Level

| Sensitivity | % | Project scenario | CAPEX | CAPEX | OPEX | OPEX | CAPEX & OPEX | CAPEX & OPEX |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0% | 10% | -10% | 10% | -10% | 10% | -10% |
| Revenue from Sale of Product (% of Program Cost) | % | 127% | 116% | 140% | 126% | 128% | 115% | 141% |
| ITMO Revenue (% of Program Cost) | % | 0% | 0% | 0% | 0% | 0% | 0% | 0% |
| Program Cost (% of project scenario) | % | 100% | 109% | 91% | 101% | 99% | 110% | 90% |
| Project CF (%) | % | 27% | 16% | 40% | 26% | 28% | 15% | 41% |
| Project IRR (without ITMOs revenue) | % | 2.45% | 1.50% | 3.54% | 2.40% | 2.50% | 1.45% | 3.59% |

It is therefore clear from above that variation the critical parameters could give a maximum project IRR of 3.59%, which is still below the WACC benchmark.

    - Barrier analysis

Despite its technical viability, the adoption of LC3 cement faces significant market barriers;

- Significant Market Barriers: The adoption of LC3 cement in Thailand is hindered by a risk-averse construction sector that strongly prefers familiar, conventional materials over new, low-carbon alternatives.

- Aesthetic Misconceptions: The addition of calcined clay alters the cement's natural colour, shifting it away from traditional grey. This causes end-users to incorrectly assume the structural quality is compromised, further slowing market acceptance.

- Financial Feasibility Challenges: For the project to be commercially viable, operational revenue must be increased to equal 58.83% of the total program costs (CAPEX + OPEX). For that reason, carbon finance from Klik Foundation plays a vital role in subsidising the project, enabling the project to offer competitive price parity with traditional cement.

- Targeted Marketing Strategy: To counter the risk of slow adoption for this new product, STL is maintaining competitive selling prices while actively working to increase market acceptance.

- Building Consumer Trust: STL's strategy includes publishing technical performance data from pilot projects and highlighting compliance with TIS 2594-2567, ensuring customers have the same level of confidence in LC3 as they do in traditional cement.

    - Common Practice analysis

The project represents a pioneering technological shift that deviates significantly from the standard industrial operations in Thailand. Currently, the "common practice" within the Thai cement sector is the production of traditional Ordinary Portland Cement (OPC) or Blended Hydraulic Cement (TIS 2594), which relies on mature mineral components such as limestone powder, fly ash, or slag. In contrast, the large-scale integration of Limestone Calcined Clay Cement (LC3) remains a first-of-its-kind endeavour in the country

1.5 Crediting Period

Project start date: Referring to T-VER-P-METH-08-01, the start date of the project is date on which the Project Owner and the Contractor jointly signed the construction or installation agreement for the greenhouse gas reduction project to be developed as a T-VER project. The contract signing date for kiln modification is xx/05/2026 (TBC).

☒ 5 years

☐ 15 years

☐ others          years

1.6 Document or Certificate of Land Use Rights (For Reduction, absorption and removal of greenhouse gases from the forestry and agriculture sectors)

The project is classified under the industrial sector. The project does not involve land-use change, biomass cultivation, afforestation/reforestation, or any activity related to agricultural land management.

# Part 2 T-VER Methodology

2.1 T-VER Methodology and Tools

| No. | Methodology Code | Version | Title of methodology/tool |
| --- | --- | --- | --- |
| 1 | T-VER-P-METH-08-01 | 1 | Cement Production from Alternative Materials |
| 2 | T-VER-P-TOOL-02-01 | 1 | Tool to calculate project or leakage CO2 emissions from fossil fuel combustion |
| 3 | T-VER-P-TOOL-02-02 | 1 | Tool to Calculation for Project Emission and Leakage Emissions from Biomass |

2.2 Project Conditions

| Code: T-VER-P-METH-08-01 |  |
| --- | --- |
| Version: 1 |  |
| Methodology/tool: Cement Production from Alternative Materials |  |
| Project Conditions | Justification/Explanation |
| - Using alternative materials will not increase clinker production capacity or extend equipment lifespan. | The production capacity for this MA stays within the limits of existing production licenses: 8,000 tons of clinker per day at the TL plant and 11,000 tons of clinker per day overall. The planned production of 2 million tons of LC3 cement is similar to the baseline production volume for normal operation, ensuring it won't increase overall clinker production capacity. |
| - Using alternative raw materials that don't contain carbonate in clinker production. | The clay used as an alternative raw material is Terra Rossa which consists of kaolinite and iron oxides. The chemical formula of Kaolinite is Al₂Si₂O₅(OH)₄. This means it's composed of aluminum (Al), silicon (Si), oxygen (O), and hydrogen (H) in a specific crystal structure. This composition ensures that the alternative raw materials used in the MA do not contain carbonate. |
| - The increased use of alternative raw materials that do not contain carbonate must not compromise cement quality, as per Thai Industrial Standard (TIS) 2594. | LC3 cement will be certified under TIS 2594, ensuring its quality is equivalent to that of hydraulic cement. |
| - The alternative raw materials have not been used in the project developer's clinker production prior to the project activities. | Prior to the implementation of the project, SCG Hybrid Cement's composition included clinker, gypsum, calcium components, and strength-enhancing additives. Calcined clay had not been incorporated. |
| - If the project developer uses fuel-switching measures, these will not be included in the greenhouse gas emission reduction calculations for this methodology. | Coal, RDF, and biomass are the fuel types utilized in both pre- and post-project development; consequently, no fuel switching is involved. |
| - This methodology is not applicable to the production of cement intended for international export. | The LC3 cement will be marketed only in Thailand. |

2.3 Relevant information for calculating greenhouse gas emissions

Relevant greenhouse gas sources for calculation

| Source |  | Gas | Included? [yes/no] | Justification/Explanation |
| --- | --- | --- | --- | --- |
| Reference scenario | Calcination of raw material in the clinker kiln | CO2 | Yes | Direct emission from clinker kiln |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  | Use of fuels (coal, biomass, RDF) in the kiln including burner | CO2 | Yes | Direct emission from clinker kiln Direct emission from transportation of biomass |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  | Use of electricity for the preparation of fuels and raw materials for clinker, and for the operation of equipment related to kiln | CO2 | Yes | Indirect emissions from grid power consumption, including electricity used for the feeding system, material preparation, and the kiln, are the primary source of emissions. Emissions from self-generated solar power are negligible. |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  | Use of electricity for the preparation of additives and for grinding cement types | CO2 | Yes | Indirect emission from grid power consumption for crushing and grinding additives and grinding cement. Emissions from self-generated solar power are negligible. |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
| Mitigation Activity | Calcination of raw materials in the clinker kiln | CO2 | Yes | Direct emission from clinker kiln |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  |  | Other | No | Emissions negligible, excluded for simplification |
|  | Use of fuel in the clinker kiln and calcined clay kiln including burner | CO2 | Yes | Direct emission from clinker kiln and calcined clay kiln Direct emission from transportation of biomass |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  | Use of electricity for the preparation of fuels and raw materials for clinker, and for the operation of equipment related to kiln | CO2 | Yes | Indirect emissions from grid power consumption, including electricity used for the feeding system, material preparation, and the kiln, are the primary source of emissions. Emissions from self-generated solar power are negligible. |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  | Use of electricity for the preparation of additives and for grinding cement types | CO2 | Yes | Indirect emission from grid power consumption for crushing and grinding additives and grinding cement. Emissions from self-generated solar power are negligible. |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  | Transportation of calcined clay to the grinding facility | CO2 | Yes | Direct emission from transportation of additives (Calcined Clay) within the plant |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
| Other activities (Leakage) | Transportation of raw clay to the factory | CO2 | Yes | Indirect emission from transportation and freight of the additional additives being used by the activity |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |
|  | Rerouting of existing raw materials and alternative additives | CO2 | Yes | Direct emission from the existing uses of diverted additives |
|  |  | CH4 | No | Emissions negligible, excluded for simplification |
|  |  | N2O | No | Emissions negligible, excluded for simplification |

# Part 3 Calculation of greenhouse gas reduction

3.1 Calculation of baseline sequestration/emission

Greenhouse gas emissions for the baseline scenario consider only carbon dioxide emissions from the volume of cement produced and sold domestically, as well as from the production of clinker and cement. Two specific cases have been defined:

Case 1

If

BEClinker,BSL×BBlend,y+BEele,ADD,BC+BEFC,ADD,BC   < 871 kgCO2/ton of blended cement,

BEClinker,BSL×BBlend,y+BEele,ADD,BC+BEFC,ADD,BC   is equal to the calculated result

Case 2

If

BEClinker,BSL×BBlend,y+BEele,ADD,BC+BEFC,ADD,BC   ≥ 871 kgCO2/ton of blended cement,

BEClinker,BSL×BBlend,y+BEele,ADD,BC+BEFC,ADD,BC   is equal to 871 kgCO2/ton of blended cement

Step 1 Determination of BEy

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEy=BCy×BEClinker,BSL×BBlend,y+BEele,ADD,BC+BEFC,ADD,BC Equation (1) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEy | Reference scenario emission in year y | Calculation | 15,67,167 | tCO2/year |
| BCy | Blended cement produced and sold in the domestic market in year y (LC3) | Capacity of the LC3 production | 2,010,000 | tBC |
| BEClinker,BSL | Reference scenario emissions per tonne of clinker in year y | Calculation (Step 1.1) | 0.92 | tCO2/tClinker |
| BBlend,y | Reference scenario share of clinker per tonne of blended cement updated for year y | Calculation (Step 1.2) | 0.84 | tClinker/t BC |
| BEele,ADD,BC | Reference scenario electricity emissions for BC grinding and preparation of additives | Calculation (Step 1.3) | 0.01 | tCO2/tBC |
| BEFC,ADD,BC | Reference scenario fossil fuel combustion emissions for BC grinding and preparation of additives | Calculation (Step 1.4) | 0 | tCO2/tBC |

Step 1.1 Determination of BEClinker,BSL

Reference scenario emissions per ton of clinker in base year are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEclinker+BEcalcin+BEFC+BEele,grid,CLNK+BEele,sg,CLNK+BEDust+BEFCFry+BETR Equation (2) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEClinker,BSL | Reference scenario CO2 emissions per tonne of clinker | Calculation | 0.92 | tCO2/tClinker |
| BEcalcin | Reference scenario emissions per tonne of clinker due to calcinations of calcium carbonate and magnesium carbonate | Calculation (Step 1.1.1) | 0.54 | tCO2/tClinker |
| BEFC | Reference scenario emissions per tonne of clinker due to combustion of fuels for clinker | Calculation (Step 1.1.2) | 0.36 | tCO2/tClinker |
| BEele,grid,CLINK | Reference scenario grid electricity emissions for clinker production per tonne of clinker | Calculation (Step 1.1.3) | 0.019 | tCO2/tClinker |
| BEele,sg,CLINK | Reference scenario self-generated electricity emissions for clinker production per tonne of clinker | Calculation (Step 1.1.4) | 0 | tCO2/tClinker |
| BEdust | Reference scenario emissions due to dust discarded through bypass and dedusting units (CKD) | Calculation (Step 1.1.5) | 0 | tCO2/tClinker |
| BEFC_Dry | Reference scenario emissions due to fuel consumption for preparation of raw materials or fuels | Calculation (Step 1.1.6) | 0 | tCO2/tClinker |
| BETR | Reference scenario emissions due to fuel consumption for preparation of raw materials or fuels | Calculation (Step 1.1.7) | 0 | tCO2/tClinker |

Step 1.1.1 Determination of BEcalcin

Reference scenario emissions per ton of clinker due to calcination of calcium carbonate and magnesium carbonate are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEcalcin =0.785×(OutCaO-InCaO)+1.092×(OutMgO-InMgO)CLINKBL Equation (3) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEcalcin | Reference scenario emissions per tonne of clinker due to calcinations of calcium carbonate and magnesium carbonate | Calculation | 0.54 | tCO2/tClinker |
| 0.785 | Stoichiometric emission factor for CaO | Fixed ex-ante parameter | 0.785 | tCO2/tCaO |
| 1.092 | Stoichiometric emission factor for MgO | Fixed ex-ante parameter | 1.092 | tCO2/tMgO |
| InCaO | Reference scenario non-carbonated CaO content in the raw material | Dynamic ex-post parameter | - | tCaO |
| OutCaO | Reference scenario CaO content in clinker produced | Dynamic ex-post parameter | 1,133,386 | tCaO |
| InMgO | Reference scenario non-carbonated MgO content in the raw material | Dynamic ex-post parameter | - | tMgO |
| OutMgO | Reference scenario MgO content in the clinker produced | Dynamic ex-post parameter | 21,234 | tMgO |
| CLNKBSL | Annual production of clinker in the base year y | Dynamic ex-post parameter | 1,688,400 | tClinker/year |

Step 1.1.2 Determination of BEFC

Reference scenario emissions per ton of clinker due to combustion of fuels for clinker production are calculated include emissions from biomass utilization as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEFC,i,BSL=BEFC,FF,i,BSL+BEFC,RDF,i,BSL+BEFC,biomass,t,BSL Equation (4) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEFC,i,BSL | Reference scenario emissions per tonne of clinker due to combustion of fuels for clinker production | Calculation | 0.36 | tCO2/tClinker |
| BEFC,FF,i,BSL | Reference scenario emissions per tonne of clinker due to combustion of fossil fuels for clinker production | Calculation (Step 1.1.2.1) | 0.22 | tCO2/tClinker |
| BE FC,RDF,i,BSL | Reference scenario emissions per tonne of clinker due to combustion of RDF for clinker production | Calculation (Step 1.1.2.2) | 0.02 | tCO2/tClinker |
| BEFC,biomass,i,BSL | Reference scenario emissions per tonne of clinker due to combustion of biomass for clinker production | Calculation (Step 1.1.2.3) | 0.03 | tCO2/tClinker |

Step 1.1.2.1 Determination of BEFC,FF,i,BSL

Reference scenario emissions per ton of clinker due to combustion of fossil fuel in clinker kilns are calculated based on historical fuel consumption data for each type of fossil fuel used in the existing kiln.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEFC,FF,i,BSL =ΣFCi,BSL×NCVi×EFCO2,i10-6CLNKBSL Equation (5) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEFC,FF,i,BSL | Reference scenario emissions per tonne of clinker due to combustion of fossil fuels for clinker production | Calculation | 0.22 | tCO2/tClinker |
| FCcoal,BSL | Coal consumed for clinker production in the base year | Dynamic ex-post parameter | 224,548 | tCoal |
| NCVcoal | Net calorific value of coal | Fixed ex-ante parameter | 0.01716 | TJ/tCoal |
| EFCO2,coal | Emission factor for coal | Fixed ex-ante parameter | 94.6 | tCO2/TJ |
| CLNKBSL | Annual production of clinker in the reference scenario | Dynamic ex-post parameter | 1,688,400 | tClinker/year |

Step 1.1.2.2 Determination of BEFC,RDF,i,BSL

Reference scenario emissions per ton of clinker due to combustion of waste-derived fuels in clinker kilns are calculated based on option 2 equation (unsorted waste) as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEFC,RDF,i,BSL=i44/12×EFFCOM×Qwaste×FFCwasteCLNKBSL Equation (6) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEFC,RDF,i,BSL | Baseline emissions per tonne of clinker due to combustion of RDF for clinker production | Calculation | 0.02 | tCO2/tClinker |
| EFFCOM,y | Combustion efficiency of combustor | Fixed ex-ante parameter | 0.99 | - |
| Qwaste,BSL | Quantity of waste j fed into combustor the in base year | Dynamic ex-post parameter | 20,336 | tWaste |
| FFCwaste | Fraction of total carbon content in waste plastic | Dynamic ex-post parameter | 0.55 | - |
| 44/12 | Conversion factor | Fixed ex-ante parameter | 44/12 | tCO2/tC |
| CLNKBSL | Annual production of clinker in the base year | Dynamic ex-post parameter | 1,688,400 | tClinker/year |

Step 1.1.2.3  Determination of BEFC,biomass,i,BSL

Reference scenario emissions per ton of clinker due to combustion of biomass fuel in clinker kilns are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEbiomass,i,BSL=BEBcom,BSL+BEBC,BSL+BEBRT,BSL+BEBP,BSLCLNKBSL Equation (7) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEbiomass,i,BSL | Reference scenario emissions per tonne of clinker due to combustion of biomass for clinker production | Calculation | 0.118 | tCO2/tClinker |
| BEBCom,BSL | Reference scenario emissions of clinker due to biomass combustion reaction for clinker production in base year | Calculation (Step 1.1.2.3.1) | 160,348 | tCO2 |
| BEBC,BSL | Reference scenario emissions of clinker due to biomass cultivation for clinker production. The value is o as there is no biomass cultivation. | Excluded | 0 | tCO2 |
| BEBT,BSL | Reference scenario emissions of clinker due to biomass transportation for clinker production. The value is o as there is no biomass transportation. | Excluded | 0 | tCO2 |
| BEBRT,BSL | Reference scenario emissions of clinker due to residual biomass transportation for clinker production | Calculation (Step 1.1.2.3.2) | 38,877 | tCO2 |
| BEBP,BSL | Reference scenario emissions of clinker due to biomass production for clinker production. The value is o as there is no biomass production. | Excluded | 0 | tCO2 |
| CLNKBSL | Annual production of clinker in the base year | Dynamic ex-post parameter | 1,688,400 | tClinker/year |

Step 1.1.2.3.1  Determination of BEFC,biomass,i,BSL

Reference scenario emissions per ton of clinker due to combustion reaction of biomass fuel in clinker kilns are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEBcomp,BSL=Qi,BSL×EFi Equation (8) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEBCom,BSL | Reference scenario emissions of clinker due to biomass combustion reaction for clinker production in base year | Calculation | 160,348 | tCO2 |
| Qbiomass,BSL | Quantity of biomass fed into combustor in base year | Dynamic ex-post parameter | 145,353 | tBiomass |
| EFbiomass | Emission factor for biomass | Fixed ex-ante parameter | 0.1032 | tCO2/tBiomass |

Step 1.1.2.3.2  Determination of BEBT,BSL

Reference scenario emissions of clinker due to residual biomass transportation for clinker production are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEBRT,BSL=Di,BSL×FRi,BSL×EFCO2,i×10-6 Equation (9) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEBRT,BSL | Reference scenario emissions of clinker due to residual biomass transportation for clinker production | Calculation | 38,877 | tCO2 |
| Dbiomass,BSL | Distance between origin and destination for the transportation of residual biomass during base year | Dynamic ex-post parameter | 356 | km |
| FRbiomassBSL | Total residual biomass transported transportation in base year | Dynamic ex-post parameter | 145,353 | t |
| EFCO2,biomass | Emissions factor of the transportation of biomass | Fixed ex-ante parameter | 0.7513 | kgCO2/tkm |

Step 1.1.3 Determination of BEele,grid,CLNK

Reference scenario grid electricity emissions for clinker production per ton of clinker are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEele,grid,CLNK =BELEgrid,CLNK×EFElec,BSLCLNKBSL Equation (10) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEele,grid,CLNK | Reference scenario grid electricity emissions for clinker production per tonne of clinker | Calculation | 0.019 | tCO2/tClinker |
| BELEgrid,CLNK | Grid electricity consumed for clinker production in base year | Dynamic ex-post parameter | 71,076 | MWh |
| EFElec,BSL | Baseline grid emission factor | Fixed ex-ante parameter | 0.4401 | tCO2/MWh |
| CLNKBSL | Annual production of clinker in the base year | Dynamic ex-post parameter | 1,688,400 | tClinker/year |

Step 1.1.4 Determination of BEele,sg,CLNK

Baseline emissions from self-generated electricity for clinker production per ton of clinker are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEele,sg,CLNK =BELEsg,CLNK×EFsg,BSLCLNKBSL Equation (11) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEele,sg,CLINK | Baseline self-generated electricity emissions for clinker production per tonne of clinker | Calculation | 0 | tCO2/tClinker |
| BELEsg,CLNK | Self-generation of electricity for clinker production in the base year | Dynamic ex-post parameter | 44,875 | MWh |
| EFsg,BSL | Emission factor for self-generated electricity in the base year | Fixed ex-ante parameter | 0 | t CO2/MWh |
| CLNKBSL | Annual production of clinker in the base year | Dynamic ex-post parameter | 1,688,400 | tClinker/year |

Step 1.1.4.1 Determination of EFsg,BSL

Emission factor for self-generated electricity in the base year is calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: EFsg,BSL=FCm,n.BSL×COEFmGENn,BSL Equation (12) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| EFsg,BSL | Emission factor for self-generated electricity in the base year | Calculation | 0 | tCO2/MWh |
| FCm,n,BSL | Quantity of fuel m consumed by relevant power sources b in base year | Dynamic ex-post parameter | 0 | mass or volume unit |
| COEFm | CO2 emission coefficient of fuel m | Fixed ex-ante parameter | 0 | tCO2/mass or volume unit |
| GENn,BSL | Electricity generated by the source j in the base year | Dynamic ex-post parameter | 44,875 | MWh |

Step 1.1.5 Determination of BEdust

Reference scenario emissions due to dust discarded through bypass and dedusting units (CKD) are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEdust=CBSL×BypassBSL+CBSL×dBSLCBSL×1-dBSL+1×CKDBSLCLNKBSL Equation (13) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEdust | Reference scenario emissions due to dust discarded through bypass and dedusting units (CKD) | Calculation | 0 | tCO2/tClinker |
| CBSL | Reference scenario calcination emissions factor due to both de-carbonization reaction and fuel consumption in clinker production | Calculation (Step 1.1.5.1) | 0.9 | tCO2/tClinker |
| BypassBSL | Annual production of bypass dust leaving kiln system | Dynamic ex-post parameter | 0 | tDust |
| CKDBSL | Annual production of CKD dust leaving kiln system in the baseline | Dynamic ex-post parameter | 0 | tDust |
| dBSL | CKD calcination rate (released CO2 expressed as a fraction of the total carbonate CO2 in the raw materials) | Dynamic ex-post parameter | 1 | - |
| CLNKBSL | Annual production of clinker in the base year | Dynamic ex-post parameter | 1,688,400 | tClinker/year |

Step 1.1.5.1 Determination of CBSLt

Reference scenario calcination emissions factor due to both de-carbonization reaction and fuel consumption in clinker production are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: CBSL=BEcalcin+BEFC Equation (14) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| CBSL | Reference scenario calcination emissions factor due to both de-carbonization reaction and fuel consumption in clinker production | Calculation | 0.9 | tCO2/tClinker |
| BEcalcin | Reference scenario CO2 emissions from calcination of calcium carbonate and magnesium carbonate | Calculation (Step 1.1.1) | 0.54 | tCO2/tClinker |
| BEFC | Reference scenario CO2 emissions from fuel consumption in clinker production | Calculation (Step 1.1.2) | 0.36 | tCO2/t clinker |

Step 1.1.6 Determination of BEFC_Dry

Reference scenario emissions due to fuel consumption for preparation of raw materials or fuels are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEFCDry=FCdry,i×NCVi×EFCO2,i×10-6CLBKBSL Equation (15) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEFC_Dry | Reference scenario emissions due to fuel consumption for preparation of raw materials or fuels | Calculation | 0 | tCO2/tClinker |
| FCdry,coal,BSL | Quantity of coal consumed for preparation of raw materials or fuels in the baseline | Dynamic ex-post parameter | 0 | tCoal |
| NCVcoal | Net calorific value of coal | Fixed ex-ante parameter | - | TJ/tCoal |
| EFCO2,coal | CO2 emission factor for coal | Fixed ex-ante parameter | - | tCO2/TJ |
| CLNKBSL | Annual production of clinker in the base year | Dynamic ex-post parameter | - | tClinker/year |

Step 1.1.7 Determination of BETR

Reference scenario emissions due to transportation of clinker to cement production facility are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BETR=iFCTR,i×NCVi×EFCO2,i×10-6CLNKBSL Equation (16) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BETR | Reference scenario emissions due to transportation of clinker to cement production facility | Calculation | 0 | tCO2/tClinker |
| FCTR,i | Quantity of fossil fuel i consumed for transportation of clinker to cement production facility | Dynamic ex-post parameter | 0 | fossil fuel unit |
| NCVi | Net calorific value of the fuel type i | Fixed ex-ante parameter | - | GJ/fossil fuel unit |
| EFCO2,i | CO2 emission factor for fuel type i | Fixed ex-ante parameter | - | t CO2/GJ |
| CLNKBSL | Annual production of clinker in the base year | Dynamic ex-post parameter | - | tClinker/year |

Step 1.2 Determination of BBlend

There are two approaches for determining the BBlend,y value, with the following details:

Option 1: Use data on the clinker-to-cement ratio from the project developer for at least three years prior to the start of the project activity. This should be evaluated as an average (excluding years with unusual situations).

Option 2: In cases where clinker-to-cement ratio data of three years prior to the start of project activities is unavailable, the project developer shall use data with comparable operating conditions and statistical significance, or data obtained from relevant units. The priority of data sources shall be ranked as follows:

(1) Clinker-to-cement ratio from other companies with comparable operations conducted concurrently with the project;

(2) National statistics on clinker-to-cement ratios;

(3) International data on clinker-to-cement ratios.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| Alternative 1 | Use data on the clinker-to-cement ratio from the project developer for at least 3 years prior to the start of project activities and evaluate as an average (excluding years with unusual operation) | - | - | - |
| BBlend | Baseline share of clinker per tonne of BC updated for year y | - | 84.00% | t clinker/t BC |

Step 1.3 Determination of BEele,grid,CLNK

Reference scenario electricity emissions for cement grinding and preparation of additives are calculated based on emissions from grid electricity and self-generated electricity used for cement grinding and additive preparation, as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEele,ADD,BC=BEele,grid,BC+BEele,SG,BC+BEele,grid,ADD+BEele,SG,ADD Equation (17) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| Bele,ADD,BC | Reference scenario electricity emissions for BC grinding and preparation of additives | Calculation | 0.01 | tCO2/tBC |
| BEele,grid,BC | Reference scenario grid electricity emissions for BC grinding (t CO2/t BC) | Calculation (Step 1.3.1) | 0.01 | tCO2/tBC |
| BEele,SG,BC | Reference scenario self-generated electricity emissions for BC grinding | Calculation (Step 1.3.2) | 0 | tCO2/tBC |
| BEele,grid,ADD | Reference scenario grid electricity emissions for additive preparation | Calculation (Step 1.3.3) | 0 | tCO2/tBC |
| BEele,SG,ADD | Reference scenario self-generated electricity emissions for additive preparation | Calculation (Step 1.3.4) | 0 | tCO2/tBC |

Step 1.3.1 Determination of BEele,grid,BC

Reference scenario electricity emissions for cement grinding and preparation of additives are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEele,grid,BC =BELEgrid,BC×EFElec,BSLBCBSL Equation (18) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEele,grid,BC | Reference scenario grid electricity emissions for BC grinding | Calculation | 0.01 | tCO2/tBC |
| BELEgrid,BC | Reference scenario grid electricity for grinding BC | Dynamic ex-post parameter | 42,371 | MWh |
| EF lec,BSL | Reference scenario grid emission factor | Fixed ex-ante parameter | 0.4401 | tCO2/MWh |
| BCBSL | Annual production of blended cement in the base year | Dynamic ex-post parameter | 2,010,000 | tBC/year |

Step 1.3.2 Determination of BEele,SG,BC

Reference scenario grid electricity emissions for additives preparation per tonne of blended cement are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEele,grid,ADD =BELEgrid,ADD×EFElec,BSLBCBSL Equation (19) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEele,SG,BC | Reference scenario self-generated electricity emissions for cement blending | Calculation | 0 | tCO2/tBC |
| BELESG,BC | Reference scenario self-generation electricity for cement blending | Dynamic ex-post parameter | 30,924 | MWh |
| EFSG,BSL | Emission factor for self-generated electricity in the base year | Fixed ex-ante parameter | 0 | tCO2/MWh |
| BCBSL | Annual production of blended cement in the base year | Dynamic ex-post parameter | 2,010,000 | tBC/year |

Step 1.3.3 Determination of BEele,grid,ADD

Reference scenario grid electricity emissions for additives preparation per tonne of blended cement are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEele,SG,ADD =BELESG,ADD×EFElec,BSLBCBSL Equation (20) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEele,grid,ADD | Reference scenario grid electricity emissions for additives preparation per tonne of blended cement | Calculation | 0 | tCO2/tBC |
| BELEgrid,ADD | Grid electricity consumed for cement blending in base year | Dynamic ex-post parameter | 0 | MWh |
| EFElec,BSL | Reference scenario grid emission factor | Fixed ex-ante parameter | 0.4401 | tCO2/MWh |
| BCBSL | Annual production of blended cement in the base year | Dynamic ex-post parameter | 2,010,000 | tBC/year |

Step 1.3.4 Determination of BEele,SG,BC

Reference scenario self-generated electricity emissions for  additives preparation  per tonne of blended cement can be calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEele,SG,ADD =BELESG,ADD×EFElec,BSLBCBSL Equation (21) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEele,SG,ADD | Reference scenario self-generated electricity emissions for additives preparation per tonne of blended cement | Calculation | 0 | tCO2/tBC |
| BELESG,ADD | Self-generated electricity consumed for additives preparation in base year | Dynamic ex-post parameter | 0 | MWh |
| EFsg,BSL | Reference scenario self-generated emission factor | Fixed ex-ante parameter | 0 | t CO2/MWh |
| BCBSL | Annual production of BC in the base year | Dynamic ex-post parameter | 2,010,000 | tBC/year |

Step 1.4 Determination of BEFC,ADD,BC

Reference scenario fossil fuel combustion emissions for BC grinding and preparation of additives are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: BEFC,ADD,BC=FCadd,BC,i×NCVi×EFCO2,i×10-6BCBSL Equation (22) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| BEFC,ADD,BC | Reference scenario fossil fuel combustion emissions for BC grinding and preparation of additives | Calculation | 0 | tCO2/tBC |
| FCadd,bc,i | Quantity of fossil fuel i consumed for cement grinding and additives preparation | Dynamic ex-post parameter | 0 | unit |
| NCVi | Net calorific value of the fuel type i | Fixed ex-ante parameter | 0.01716 | TJ/tCoal |
| EFCO2,i | CO2 emission factor for fuel type i | Fixed ex-ante parameter | 94.6 | tCO2/ TJ |
| BCBSL | Annual production of BC in the base year | Dynamic ex-post parameter | 2,010,000 | tBC/year |

3.2 Calculation of project sequestration/emission

Step 2 Determination of PEy

The calculation in this step include emissions from fossil fuels and electricity for calcined clay (alternative material) production and processing of raw material. Project emissions per ton of clinker in the project activity plant are calculated as follow.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEy=BCy×PEclinker,y×Pblend,y+PEAM,y×PEblend,AM,y+PETR,CC,i,y+PEele,ADD,BC,y+PEFC,ADD,BC,y Equation (23) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| PEy | Project emissions in year y | Calculation | 1,400,449 | tCO2/year |
| BCy | Blended cement produced and sold in domestic markets in year y | Capacity of the LC3 production | 2,010,000 | t BC |
| PEclinker,y | CO2 emissions per tonne of clinker in the project plant in year y | Calculation (Step 2.1) | 0.966 | tCO2/tClinker |
| PBlend,y | Share of clinker per tonne of BC in year y | Calculation (Step 2.2) | 0.68 | tClinker/tBC |
| PEAM,y | Project emissions from alternative material preparation in the project activity plant in year y | Calculation (Step 2.3) | 0.257 | tCO2/tAM |
| Pblend,AM,y | Share of alternative material per tonne of blended cement in year y | Dynamic ex-post parameter | 0.12 | tAM |
| PETR,LCC,i,y | Determination of project emissions due to the transportation of calcined clay | Calculation (Step 2.4) | 0.00569 | tCO2/tBC |
| PEele,ADD,BC,y | Electricity emissions for preparation of additives in year y | Calculation (Step 2.5) | 0.0032 | tCO2/tBC |
| PE FC,ADD,BC,y | Electricity emissions for cement blending in year y | Calculation (Step 2.6) | 0 | tCO2/tBC |

Step 2.1 Determination of PEClinker,BSL

Project emissions per tonne of clinker in the project activity plant in year y are calculated using the same approach with equation (2) – (16) of step 1.1.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEclinker,y =PEcalcin,i,y+PEFC,i,y+PEele,grid,CLNK,i,y+PEele,sg,CLNK,i,y+PEDust+PEFCDry,i,y+PETR,i,y×PCLNK,i,y Equation (24) |  |  |  |  |
| Parameter | Meaning | Ref | Value | Unit |
| PEclinker,y | CO2 emissions per tonne of clinker in the project plant in year y | Calculated | 0.866 | tCO2/tClinker |
| PEcalcin,i,y | Project emissions per tonne of clinker i due to calcination of calcium carbonate and magnesium carbonate in year y | Calculation (step 2.1.1) | 0.54 | tCO2/tClinker |
| PEFC,i,y | Project emissions per tonne of clinker due to combustion of fossil fuels for clinker | Calculation (step 2.1.2) | 0.32 | tCO2/tClinker |
| PEele,grid,CLNK,i,y | Programme grid electricity emissions for clinker production per tonne of clinker | Calculation (step 2.1.3) | 0.01 | tCO2/tClinker |
| PEele,sg,CLNK,i,y | Programme self-generated electricity emissions for clinker production per tonne of clinker | Calculation (step 2.1.4) | 0 | tCO2/tClinker |
| PEDust,i,y | Project emissions due to dust discarded through bypass and dedusting units (CKD) | Calculation (step 2.1.5) | 0 | tCO2/tClinker |
| PEFC_Dry,i,y | Project emissions due to fuel consumption for preparation of raw materials or fuels | Calculation (step 2.1.6) | 0 | tCO2/tClinker |
| PETR,i,y | Project emissions due to transportation of clinker to cement production facility | Calculation (step 2.1.7) | 0 | tCO2/tClinker |
| PCLNK,i,y | Ratio of clinker i for total production of clinker in year y | Calculation (step 2.1.8) | 1 | tClinker/tBC |

Step 2.2 Determination of Pblend,i,y

Ratio of clinker for total production of blended cement is calculated as follow.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: Pblend,i,y=CLNKi,yBCy Equation (25) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| Pblend,i,y | Ratio of clinker i for total production of clinker in year y | Calculation | 0.68 | tClinker/tBC |
| CLNKi,y | Annual production of clinker in the year y (CLK) | Technical Data | 1,366,800 | tClinker |
| BCBSL | Annual production of BC in year y | Dynamic ex-post parameter | 2,010,000 | tBC/year |

Step 2.3 Determination of PEAM,y

Project emissions from alternative material preparation in the project activity plant in year y are calculated as follows. The calculation in this step is deviated from T-VER-P-METH-08-01 to include emission from alternative material preparation.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEAM,y=PEFC,AM,y+PEele,grid,AM,y+PEele,sg,AM,y Equation (26) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEAM,y | Project emissions from alternative material preparation in the project activity plant in year y | Calculation | 0.257 | tCO2/tAM |
| PEFC,AM,y | Project emissions from fuel consumption for alternative material preparation in the project activity plant in year y | Calculation (Step 2.3.1) | 0.248 | tCO2/tAM |
| PEelec,grid,AM,i,y | Emissions from grid electricity per tonne of alternative material i production in year y | Calculation (Step 2.3.2) | 0.0086 | tCO2/tAM |
| PEelec,sg,AM,i,y | Emissions from self-generated electricity per tonne of alternative material i production in year y | Calculation (Step 2.3.3) | 0 | tCO2/tAM |

Step 2.3.1 Determination of PEAM,y

Project emissions from fuel consumption for alternative material preparation in the project activity plant in year y are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEAM,y=PEFC,AM,y+PEele,grid,AM,y+PEele,sg,AM,y Equation (27) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEFC,AM,y | Project emissions from fuel consumption for alternative material preparation in the project activity plant in year y | Calculation | 0.248 | tCO2/tAM |
| PEFC,AM,FF,i,y | Project emissions per tonne of BC due to combustion of fossil fuels for alternative material production | Calculation (Step 2.3.1.1) | 0.086 | tCO2/tAM |
| PEAM,biomass,i,y | Project emissions per tonne of BC due to combustion of biomass for alternative material production | Calculation (Step 2.3.1.2) | 0.16 | tCO2/tAM |

Step 2.3.1.1 Determination of PEFC,AM,FF,i,y

Project emissions per tonne of BC due to combustion of fossil fuels for alternative material production are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEFC,AM,FF,i,y=FCAM,y×NCVi×EFCO2,i×10-6AMy Equation (28) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEFC,AM,FF,i,y | Project emissions per tonne of BC due to combustion of fossil fuels for alternative material production | Calculation | 0.086 | tCO2/tAM |
| FCcoal,AM,y | Quantity of coal consumed for alternative material production in the year y | Dynamic ex-post parameter | 12,785 | tCoal |
| NCVcoal | Net calorific value of coal | Fixed ex-ante parameter | 0.01716 | TJ/tCoal |
| EFCO2,coal | CO2 emission factor for coal | Fixed ex-ante parameter | 94.6 | tCO2/TJ |
| AMy | Alternative material produced in year y | Dynamic ex-post parameter | 241,200 | tAM/year |

Step 2.3.1.1.1 Determination of PEAM,biomass,i,y

Project emissions of clinker due to combustion of biomass for alternative material production are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEAM,biomass,i,y=PEBcom,AM,y+BEBC,AM,y+BEBRT,AM,y+BEBP,AM,yAMy Equation (29) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEAM,biomass,i,y | Project emissions of clinker due to combustion of biomass for alternative material production | Calculation | 0.1621 | tCO2/tAM |
| PEBCom,AM,y | Project emissions of clinker due to biomass combustion for alternative material production | Calculation (Step 2.3.1.1.1) | 31,472 | tCO2 |
| PEBC,AM,y | Project emissions of clinker due to biomass combustion reaction for clinker production in base year | Excluded | 0 | tCO2 |
| PEBT,AM,y | Project emissions of clinker due to biomass cultivation for clinker production. The value is o as there is no biomass cultivation. | Excluded | 0 | tCO2 |
| PEBRT,AM,y | Project emissions of clinker due to biomass transportation for clinker production. The value is o as there is no biomass transportation. | Calculation (Step 2.3.1.1.2) | 7,630 | tCO2 |
| PEBP,AM,y | Project emissions of clinker due to residual biomass transportation for clinker production | Excluded | 0 | tCO2 |
| AMy | Project emissions of clinker due to biomass production for clinker production. The value is o as there is no biomass production. | Dynamic ex-post parameter | 241,200 | tAM/year |

Step 2.3.1.1.1  Determination of PEBCom,AM,y

Project emissions of clinker due to biomass combustion for alternative material production are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEBcomp,AM,y=Qi,y×EFi Equation (30) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEBCom,AM,y | Project emissions of clinker due to biomass combustion for alternative material production | Calculation | 31,472 | tCO2 |
| QbiomassAM,y | Quantity of biomass fed into combustor for alternative material production in year y | Dynamic ex-post parameter | 28,529 | tBiomass |
| EFbiomass | Emission factor for biomass | Fixed ex-ante parameter | 1.1032 | tCO2/tBiomass |

Step 2.3.1.1.2 Determination of PEBT,AM,y

Project emissions of alternative material due to residual biomass transportation are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEBRT,AM,y=Df,m×FRf,m×EFCO2,f×10-6 Equation (31) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEBRT,AM,y | Programme emissions of alternative material due to residual biomass transportation | Calculation | 7,630 | tCO2 |
| DbiomassBSL | Distance between origin and destination for the transportation of residual biomass during base year | Dynamic ex-post parameter | 356 | km |
| FRbiomass,BSL | Total residual biomass transported transportation for alternative material production in year y | Dynamic ex-post parameter | 28,529 | t |
| EFCO2,biomass | Emissions from the transportation of biomass | Fixed ex-ante parameter | 0.7513 | kgCO2/tkm |

Step 2.3.2 Determination of PEele,grid,AM,y

Project emissions from grid electricity per tonne of alternative material i production in year y are calculated as follow.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEele,grid,AM,i,y =PELEgrid,AMi,y×EFElec,i,yAMy Equation (32) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEelec,grid,AM,i,y | Project emissions from grid electricity per tonne of alternative material i production in year y | Calculation | 0.0086 | tCO2/tAM |
| PELEgrid,AM,y | Grid electricity consumed for clinker production in year y | Dynamic ex-post parameter | 4,736 | MWh |
| EFElec,i,y | Programme grid emission factor | Fixed ex-ante parameter | 0.4401 | tCO2MWh |
| AMi,y | Annual production of alternative material in the year y | Dynamic ex-post parameter | 241,200 | tAM/year |

Step 2.3.3 Determination of PEele,sg,AM,y

Project emissions from self-generated electricity per tonne of alternative material i production in year y are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEele,sg,AM,i,y =PELEsg,AM,i,y×EFsg,i,yAMi,y Equation (33) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEelec,sg,AM,i,y | Project emissions from self-generated electricity per tonne of alternative material i production in year y | Calculation | 0 | tCO2/tAM |
| PELEsg,AM | Self-generation of electricity for alternative material production in the year y | Dynamic ex-post parameter | 3,485 | MWh |
| EFsg,y | Emission factor for self-generated electricity in the year y | Fixed ex-ante parameter | 0 | tCO2MWh |
| AMi,y | Annual production of alternative material in the year y | Dynamic ex-post parameter | 241,200 | tAM/year |

Step 2.4 Determination of PETR,LC2,i,y

Project emissions due to the transportation of alternative material are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PETR,AM,i,y=DLC2,y×QLC2,y×EFCO2,AM×10-6BCy Equation (34) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PETR,i,y | Project emissions due to transportation of clinker to cement production facility | Calculation (step 2.1.7) | 0.00569 | tCO2/tClinker |
| DAM,y | Round-trip distance between TL and KW in year y | Dynamic ex-post parameter | 40 | km |
| QAM,i,y | Quantity of transported alternative material | Dynamic ex-post parameter | 241,200 | t AM |
| EFCO2,AM | Default CO2 emission factor for freight transportation activity for alternative material | Fixed ex-ante parameter | 1.1855 | kgCO2/tkm |
| BCy | Blended cement produced and sold in the domestic market in year y | Capacity of the LC3 production | 2,010,000 | tBC |

Step 2.5 Determination of PEele,ADD,BC,y

Programme fossil fuel combustion emissions for BC grinding and preparation of additives are calculated using the same approach with equation (17) – (21) of step 1.3.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEele,ADD,BC,y=PEele,grid,BC,y+PEele,SG,BC,y+PEele,grid,ADD,y+PEele,SG,ADD,y Equation (35) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PEele,ADD,BC,y | Electricity emissions for preparation of additives in year y | Calculated | 0.0032 | tCO2/tBC |
| PEele,grid,BC,y | Programme grid electricity emissions for cement blending (t CO2/t BC) | Result of the calculation step 2.5.1 | 0.0032 | tCO2/tBC |
| PEele,SG,BC,y | Programme self-generated electricity emissions for cement blending | Result of the calculation step 2.5.2 | 0 | tCO2/tBC |
| PEele,grid,ADD,y | Programme grid electricity emissions for alternative raw material preparation | Result of the calculation step 2.5.3 | 0 | tCO2/tBC |
| PEele,SG,ADD,y | Programme self-generated electricity emissions for alternative raw material preparation | Result of the calculation step 2.5.4 | 0 | tCO2/tBC |

Step 2.6 Determination of PEFC,ADD,BC,y

Programme fossil fuel combustion emissions for BC grinding and preparation of additives are calculated using the same approach with equation (22) of step 1.4.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: PEFC,ADD,BC=FCADD,BC,i,y×NCVcoal×EFCO2,coal×10-6BCy Equation (36) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| PE FC,ADD,BC,y | Electricity emissions for cement blending in year y | Calculation (step 2.5) | 0 | t CO2/t of BC |
| FCadd,bc,i | Quantity of fossil fuel i consumed for cement grinding and additives preparation in year y | Dynamic ex-post parameter | 0 | t fossil fuel |
| NCVi | Net calorific value of the fuel type i | Fixed ex-ante parameter | - | GJ/fossil fuel unit |
| EFCO2,i | CO2 emission factor for fuel type i | Fixed ex-ante parameter | - | t CO2/GJ |
| BCy | Annual production of BC in year y | Capacity of the LC3 production | 2,010,000 | t BC |

3.3 Calculation of Leakage Emission

The project calculates leakage emissions from 3 sources as identified in T-VER-P-METH-08-01 from the following.

- the transportation of raw materials and alternative additives,

- the rerouting of existing raw materials and alternative additives, and

- the use of biomass and/or residual biomass.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: LEy =LETR,y+LEADD,y+ LEbiomass,y Equation (37) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| LEy | Leakage emissions in year y | Calculation | 2,537 | tCO2 |
| LETR,y | Leakage emissions due to transport of alternative raw material and additional additives in year y | Calculation (Step 3.1) | 2,537 | tCO2 |
| LEADD,y | Leakage emissions due to the diversion of alternative raw material and additives from existing uses in year y | Calculation (Step 3.2) | 0 | tCO2 |
| LEbiomass,y | Leakage emissions due to the diversion of biomass or residual biomass outside project boundary in year y | Calculation (Step 3.3) | 0 | tCO2 |

Step 3.1 Determination of LETR,y

Leakage emissions from the transportation of alternative materials from mine are calculated as follow.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: LETR,y=Df,y×QADD,y×EFCO2,f×10-6 Equation (38) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| LETR,y | Leakage emissions due to transport of alternative raw material and additional additives in year y | Calculation | 2537 | tCO2 |
| Df,y | Round-trip distance between origin and destination for the transportation of raw materials and alternative additives of type f in year y | Dynamic ex-post parameter | 140 | km |
| QADD,y | Quantity of alternative additives transported in year y | Fixed ex-ante parameter | 241,200 | tAdditives |
| EFCO2,f | Emissions from the use of fossil fuels for transporting alternative additives of type f | Dynamic ex-post parameter | 0.7513 | gCO2/tkm |

Step 3.1.1 Determination of QADD,y

Quantity of alternative additives transported in year y is calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: QADD,y=Apj,Blend,y-ABSL,Blend,y×BCy Equation (39) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| QADD,y | Quantity of alternative additives transported in year y | Calculation | 241,200 | tAdditives |
| Apj,Blend,y | Proportion of additives in cement from project operations in year y | Dynamic ex-post parameter | 12% | - |
| A BSL,Blend,y | Proportion of additives in cement from project operations in base year | Dynamic ex-post parameter | 0 | - |
| BCy | Blended cement produced and sold in the domestic market in year y | Capacity of the LC3 production | 2,010,000 | tBC |

Step 3.2 Determination of LEADDy

Leakage emissions due to the diversion of alternative raw material and additives from existing uses in year y are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: LEADD,y=BEy-PEy×αy Equation (40) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| LEADD,y | Leakage emissions due to the diversion of alternative raw material and additives from existing uses in year y | Calculation | 0 | tCO2 |
| BEy | Baseline Emission in year y | Calculation (Step 1) | 1,567,167 | tCO2/year |
| PEy | Project emissions in year y | Calculation (Step 2) | 1,400,449 | tCO2/year |
| αy | Leakage penalty factor in year y | Calculation (Step 3.2.1) | 0 | - |

Step 3.2.1 Determination of αy

Leakage penalty factors in year y are calculated as follows.

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: αy=ADDNS,yADDy Equation (41) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| αy | Leakage penalty factor in year y | Calculation | 0 | - |
| ADDNS,y | Amount of alternative raw material and/or additives used for BC production in project plant for which the project participants could not substantiate that they are surplus in year y | Dynamic ex-post parameter | 0 | t additives |
| ADDy | Amount of alternative raw material and additives used for BC production in project plant in year y | Dynamic ex-post parameter | 241,200 | t additives |

Step 3.3 Determination of LEbiomass,y

There are no leakage emissions due to the diversion of biomass or residual biomass outside project boundary in year y

The following leakage emissions are considered minor and not included in the ex-ante emissions qualification.

- Leakage emissions from raw clay production: For consideration of the leakage emissions from the increased demand for raw clay production, a comparison of emission factors reveals that limestone production (including crushing and washing) generates 0.0025 kgCO2e/kg, whereas clay production has an emission factor of 0.0004 kgCO2e/kg referring to Ecoinvent 2.2, IPCC 2007 GWP 100a . Apart from this, the environmental mitigation and preventative measures are also in place at the Pang Asok Clay Mine to minimize potential environmental impacts as defined in the environmental impact monitoring reports submitted to the Department of Primary Industries and Mines (DPIM) every 6 months, as stipulated in the Announcement of the DPIM dated August 23, B.E. 2556 (2013), concerning the Environmental Prevention and Mitigation Measures for Industrial Clay Mining Projects (Cement Clay Type) . This indicates that the shift in raw material usage resulting from this MA is not anticipated to cause leakage emissions.

- Leakage emissions from diversion of fossil fuel: The diversion of coal resulting from the project transition to LC3 technology is deemed negligible. As demonstrated in Annex 12, the reduction in coal demand represents less than 0.1% of the total national coal supply, according to the EPPO 2024 Energy Statistics. Given the high elasticity of the global coal market and the insignificant volume of this reduction relative to regional capacity, the project does not influence market prices or trigger secondary emissions elsewhere. Consequently, the leakage associated with fossil fuel diversion is determined to be zero.

- Leakage emissions from the displacement of biomass: The biomass energy demand for the project accounts for only 5.85% of the total regional potential as detailed in Annex 13. The assessment confirms a substantial surplus of available rice husk. This significant margin ensures that the mobilization of these resources does not deprive existing users or result in indirect fossil fuel consumption elsewhere. To further mitigate leakage risks, the MA owner will utilize long-term procurement contracts to secure a stable supply without disruptive market impacts, while maintaining an ongoing strategy to integrate additional renewable energy sources over the project's lifetime.

- Leakage emissions associated with the supply of clinker to external facilities: The project activity is substituting clinker with calcined clay which results in a net reduction of total clinker output within the project boundary compared to the reference scenario. Consequently, the project does not increase the availability of clinker for other plants, thereby ruling out any indirect emission increases outside the project boundary.

3.4 Summary of greenhouse gas emissions reduction

| Code: T-VER-P-METH-08-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Version: 01 |  |  |  |  |
| Methodology/tools: Cement Production from Alternative Materials |  |  |  |  |
| Equation: ERy =BEy-PEy-LEy Equation (42) |  |  |  |  |
| Parameter | Meaning | Ref. | Value | Unit |
| ERy | Emission reduction in year y | Calculation (rounded) | 163,000 | tCO2/year |
| BEy | Reference scenario emission in year y | Calculation (rounded) | 1,567,000 | tCO2/year |
| PEy | Project emissions in year y | Calculation (rounded) | 1,401,000 | tCO2/year |
| LEy | Leakage emissions in year y | Calculation (rounded) | 3,000 | tCO2 |

3.5 Summary of estimated greenhouse gas emission reductions

| Year | d/m/y – d/m/y | Baseline Emission (rounded) | Project Emission (rounded) | Leakage Emission (rounded) | Emission Reduction (rounded) | Emission Reduction (10% withholding by DCCE in 2030) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | November 2027- 31 December 2027 | 23,300 | 21,000 | 100 | 2,200 | 2,200 |
| 2 | 1 January 2028 – 31 December 2028 | 1,216,300 | 1,087,000 | 2,000 | 127,300 | 127,300 |
| 3 | 1 January 2029 – 31 December 2029 | 1,567,000 | 1,401,000 | 3,000 | 163,000 | 163,000 |
| 4 | 1 January 2030 – 31 December 2030 | 1,567,000 | 1,401,000 | 3,000 | 163,000 | 117,450 |
| Total (tCO2eq) |  |  |  |  | 455,500 | 409,950 |
| Average (tCO2e/y) |  |  |  |  | 113,875 | 102,487 |

# Part 4 Monitoring of project

4.1 Monitoring Plan

The operational data is monitored at the monitoring points defined in Figure 6.

Figure 6 - The monitoring points of the project

All cement plant staff will be trained by technology provider companies prior to full commissioning of the LC3 production unit.

Figure 7 Structure of the LC3 unit operation in TL and KW

The overall responsibility regarding the proper operation, maintenance and project monitoring lies with the heads of the Operation Team #1 and #2. They will be supported by the Process Operation Teams who will undertake the actual measurements. Further, the staff will follow monitoring procedures and GHG emission reduction standard requirements.

Figure 8 Operational data flow

The teams mentioned above will ensure proper and timely calibration (in accordance with the manufacturer specifications) of systems, data acquisition and storage. The site manager of each plant will also undertake regular follow-ups to make sure the data measured is consistent. All monitoring data will be stored in the logbooks and in electronic format. The monitoring records shall be archived for a period of the crediting period plus 2 years.

The following table outlines the key monitored data and the associated reports for this project.

Table 13 Key monitoring parameters and the associated reports

| Key monitoring parameters | Associated reports | Frequency | Responsible team |
| --- | --- | --- | --- |
| Products quantity (BCy) | G-TK001/DCS production dashboard | Daily | Process operation team |
| Clinker and calcined clay production (CLNKi,y and AMy) | G-TK001/DCS production dashboard | Daily | Process operation team |
| Chemical properties (InCaO, OutCaO, InMgO and OutMgO) | QIS (quality information system) | Daily | Laboratory team |
| Fuel consumption | G-TK001/DCS production dashboard | Daily | Process operation team |
| Electricity consumption | G-TK001/DCS production dashboard | Daily | Process operation team |

4.2 Parameters not monitored

| Parameter | EFCO2,coal |
| --- | --- |
| Value applied | 94.6 |
| Unit | tCO2/TJ |
| Meaning | CO2 emission factor for coal used in heat generation process |
| Source of data | Table 2.2 of 2006 IPCC Guidelines for National GHG Inventories |

| Parameter | EFFbiomass |
| --- | --- |
| Value applied | 1.1032 |
| Unit | tCO2/ tBiomass |
| Meaning | Emission factor for rice husks |
| Source of data | Page 62, appendix titled "Energy Content of Fuel (Net Calorific Value), Thailand Alternative Energy Situation" report 2023, Department of Alternative Energy Development and Efficiency (DEDE) TABLE 2.2 DEFAULT EMISSION FACTORS FOR STATIONARY COMBUSTION IN THE ENERGY INDUSTRIES, 2006 IPCC Guidelines for National Greenhouse Gas Inventories |

| Parameter | EFFCOM,y |
| --- | --- |
| Value applied | 0.8 |
| Unit | - |
| Meaning | Combustion efficiency of combustor |
| Source of data | Department of Alternative Energy Development and Efficiency |

| Parameter | EFCO2,ricehusk |
| --- | --- |
| Value applied | 0.7513 |
| Unit | kg CO2/t km |
| Meaning | Emissions factor from the freight transportation of rice husks |
| Source of data | TGO’s announcement on Emission Factor (CFP) Based on the assumption of transportation via 10-wheel truck 0% loading, harsh conditions (Row 142). |

| Parameter | EFCO2,clay |
| --- | --- |
| Value applied | 0.7513 |
| Unit | kg CO2/t km |
| Meaning | Emissions factor from the freight transportation of clay |
| Source of data | TGO’s announcement on Emission Factor (CFP) Based on the assumption of transportation via 10-wheel truck 0% loading, harsh conditions (Row 142). |

| Parameter | EFCO2,AM |
| --- | --- |
| Value applied | 1.1855 |
| Unit | kg CO2/t km |
| Meaning | Emissions factor from the freight transportation of clay |
| Source of data | TGO’s announcement on Emission Factor (CFP) Based on the conservative assumption of transportation via cement truck 0% loading, harsh conditions (Row 222). |

| Parameter | dBSL |
| --- | --- |
| Value applied | 100 |
| Unit | % |
| Meaning | CKD burning rate |
| Source of data | Direct Greenhouse Gases Emission from Cement Clinker Production, Thai Cement Manufacturers Association |

4.3 Monitored Parameters

The parameters monitored and determined ex post for mitigation outcomes quantification are identified as follows.

| Data / Parameter 1: | BCy |
| --- | --- |
| Data unit: | t BC |
| Description: | Blended cement produced and sold in the domestic market in year y (t BC) |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 2: | Pblend,y |
| --- | --- |
| Data unit: | t clinker/t BC |
| Description: | Share of clinker per tonne of BC in year y |
| Measurement procedures (if any): | Calculation using BCy and CLNKi,y |
| Monitoring frequency: | Recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | - |
| Any comment: | - |

| Data / Parameter 3: | InCaOi,y |
| --- | --- |
| Data unit: | t CaO |
| Description: | Quantity of non-carbonated CaO content in the raw material for clinker type i in year y |
| Measurement procedures (if any): | This parameter is calculated as the CaO content (%) of the raw material in year y times the raw material quantity used in year y. This will be calculated and measured as part of normal operations. The sample will be taken at the sampling point to measure the composition of the input limestone using the X-ray fluorescence (XRF) machine in the internal laboratory. |
| Monitoring frequency: | Recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 4: | OutCaOi,y |
| --- | --- |
| Data unit: | t CaO |
| Description: | Quantity of CaO content in the produced clinker type i in year y |
| Measurement procedures (if any): | This parameter is calculated as the CaO content (%) of the clinker in year y times clinker produced in year y [CLNKi,y]. This will be calculated and measured as part of normal operations. The sample will be taken at the sampling point to measure the composition of the input limestone using the XRF machine in the internal laboratory. |
| Monitoring frequency: | Recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 5: | InMgOi,y |
| --- | --- |
| Data unit: | t MgO |
| Description: | Quantity of non-carbonated MgO content in the raw material for clinker type i in year y |
| Measurement procedures (if any): | This parameter is calculated as the MgO content (%) of the raw material in year y times the raw material quantity in year y [CLNKi,y]. This will be calculated and measured as part of normal operations. The sample will be taken at the sampling point to measure the composition of the input limestone using the XRF machine in the internal laboratory. |
| Monitoring frequency: | Recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 6: | OutMgOi,y |
| --- | --- |
| Data unit: | t MgO |
| Description: | Quantity of MgO content in the produced clinker type i in year y |
| Measurement procedures (if any): | This parameter is calculated as the MgO content (%) of the clinker in year y times clinker produced in year y [CLNKi,y]. This will be calculated and measured as part of normal operations. The sample will be taken at the sampling point to measure the composition of the input limestone using the XRF machine in the internal laboratory. |
| Monitoring frequency: | Recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 7: | Qrm,y |
| --- | --- |
| Data unit: | t raw materials |
| Description: | Quantity of raw material used for clinker production in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and Recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | Parameter required to calculate InCaOi,y and InMgOi,y |

| Data / Parameter 8: | CLNKi,y |
| --- | --- |
| Data unit: | t clinker |
| Description: | Quantity of clinker production in year y |
| Measurement procedures (if any): | Calculation using OutCaOi,y , OutMgOi,y and Qrm,y |
| Monitoring frequency: | Measured by a weight feeder which connects to the digital control system and recorded at least monthly. |
| Source of data: | Monitoring continuously and Recorded at least monthly. |
| QA/QC procedures | The quantity will be cross-checked with a level sensor in the storage silo. |
| Any comment: | - |

| Data / Parameter 9: | FCi,y |
| --- | --- |
| Data unit: | unit |
| Description: | Amount of fossil fuel type i consumption for clinker production in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 10: | PELEgrid,CLNK,i,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of grid electricity consumption for clinker production type i in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 11: | PELEsg,CLNK,i,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of self-generated electricity consumption for clinker production type i in year y |
| Measurement procedures (if any): | Measured by an electricity meter which connects to the digital control system and recorded at least monthly. |
| Monitoring frequency: | Monitoring continuously and recorded daily |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 12: | ADDy |
| --- | --- |
| Data unit: | t additives |
| Description: | Amount of alternative raw material and additives used for BC production in project plant in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 13: | ADDNS,y |
| --- | --- |
| Data unit: | t additives |
| Description: | Amount of alternative raw material and additives used for BC production in project plant for which the project participants could not substantiate that they are surplus in year y |
| Measurement procedures (if any): | - |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | National data or on-site measurements in plant records |
| QA/QC procedures | - |
| Any comment: | - |

| Data / Parameter 14: | PELEgrid,BC,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of grid electricity consumption for BC grinding in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 15: | PELEsg,BC,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of self-generated electricity consumption for BC grinding in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 16: | PELEgrid,ADD,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of grid electricity consumption for additives grinding in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 17: | PELEsg,ADD,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of self-generated electricity consumption for additives grinding in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 18: | FCk,j,y |
| --- | --- |
| Data unit: | unit |
| Description: | Amount of fossil fuel consumption type k by relevant power source j in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 19: | NCVk |
| --- | --- |
| Data unit: | GJ/mass or volume unit |
| Description: | Net calorific value per mass or volume unit of a fuel k |
| Measurement procedures (if any): | Measured by weight or volume meter |
| Monitoring frequency: | For (a) and (b): The NCV will be obtained for each fuel delivery, from which weighted average annual values will be calculated. For (c): Review appropriateness of the values annually. For (d): Any future revision of the IPCC Guidelines will be taken into account. |
| Source of data: | The following data sources may be used if the relevant conditions apply: Data source; Conditions for using the data source. (a) Values provided by the fuel supplier in invoices; This is the preferred source if the carbon fraction of the fuel is not provided (Option A). (b) Measurements by the project participants; If (a) is not available. (c) Regional or national default values; If (b) is not available. These sources can only be used for liquid fuels and should be based on well documented, reliable sources (such as national energy balances). (d) IPCC default values at the upper limit of the uncertainty at a 95% confidence interval as provided in Table 1.2 of Chapter 1 of Vol. 2 (Energy) of the 2006 IPCC Guidelines on National GHG Inventories; If (c) is not available. |
| QA/QC procedures | Verify if the values under (a), (b) and (c) are within the uncertainty range of the IPCC default values as provided in Table 1.2, Vol. 2 of the 2006 IPCC Guidelines. If the values fall below this range collect additional information from the testing laboratory to justify the outcome or conduct additional measurements. The laboratories in (a), (b) or (c) should have ISO17025 accreditation or justify that they can comply with similar quality standards |
| Any comment: | - |

| Data / Parameter 20: | GENj,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of electricity consumption from source j for self generation electricity in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 21: | APJ,blend,y |
| --- | --- |
| Data unit: | t additives/t BC |
| Description: | Share of additives per tonne of BC in year y |
| Measurement procedures (if any): | Calculation using ADDy and BCy |
| Monitoring frequency: | Annually |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | - |
| Any comment: | - |

| Data / Parameter 22: | TEMPCLNK,i |
| --- | --- |
| Data unit: | °C |
| Description: | Temperature from calcination process at combustor for clinker production |
| Measurement procedures (if any): | Measured by temperature sensor which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | - |
| Any comment: | Cross check for reduction of fossil fuel consumption due to lower calcination temperature |

| Data / Parameter 23: | ByPassy |
| --- | --- |
| Data unit: | t |
| Description: | Annual production of bypass dust leaving kiln system in year y |
| Measurement procedures (if any): | Measured by weight meter or weight bridge |
| Monitoring frequency: | Monitoring continuously |
| Source of data: | It will be measured as part of normal operations |
| QA/QC procedures | These data will be collected as part of normal plant level operations. QA/QC requirements according to ISO 9000 or similar quality systems |
| Any comment: | - |

| Data / Parameter 24: | CKDy |
| --- | --- |
| Data unit: | ton |
| Description: | Annual production of CKD dust leaving kiln system |
| Measurement procedures (if any): | Measured by weight meter or weight bridge |
| Monitoring frequency: | Monitoring continuously |
| Source of data: | It will be measured as part of normal operations |
| QA/QC procedures | These data will be collected as part of normal plant level operations. QA/QC requirements according to ISO 9000 or similar quality systems |
| Any comment: | - |

| Data / Parameter 25: | dy |
| --- | --- |
| Data unit: | % |
| Description: | CKD calcination rate (released CO2 expressed as a fraction of the total carbonate CO2 in the raw materials) |
| Measurement procedures (if any): | Sampling method |
| Monitoring frequency: | Monthly |
| Source of data: | It will be measured as part of normal operations |
| QA/QC procedures | These data will be collected as part of normal plant level operations. QA/QC requirements according to ISO 9000 or similar quality systems |
| Any comment: | - |

| Data / Parameter 26: | EFgrid,y |
| --- | --- |
| Data unit: | tCO2/MWh |
| Description: | CO2 emission factor for electricity generation/ consumption in year y |
| Measurement procedures (if any): | For preparation of project design document (MADD/PDD) Use the value from latest announcement from TGO For monitoring of project activity Use the value for each issuance year that matched with the report. If no value is reported on that year, the latest year shall be used. |
| Monitoring frequency: | Annually |
| Source of data: | Emission factor from electricity generation and heat generation for greenhouse gas emission reduction project report by TGO |
| QA/QC procedures | - |
| Any comment: | - |

| Data / Parameter 29: | EFFCOM,y |
| --- | --- |
| Data unit: | % |
| Description: | Efficiency of clinker combustor in year y |
| Measurement procedures (if any): | - |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | 1) Value from project’s information 2) Value from country’s information 3) Default value from IPCC |
| QA/QC procedures | The efficiency test will be conducted annually. |
| Any comment: | - |

| Data / Parameter 30: | FCi,AM,y |
| --- | --- |
| Data unit: | unit |
| Description: | Amount of fossil fuel type i consumption for alternative material production in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 31: | Qwaste,c,y |
| --- | --- |
| Data unit: | t |
| Description: | Quantity of fresh waste or RDF/SB fed into combustor c in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 32: | FFCwaste,,y |
| --- | --- |
| Data unit: | % |
| Description: | Fraction of fossil carbon in total carbon content of waste or RDF/SB fed into combustor c in year y |
| Measurement procedures (if any): | Measure by using the following standards, or similar national or international standards: ASTM D6866: “Standard Test Methods for Determining the Biobased Content of Solid, Liquid, and Gaseous Samples Using Radiocarbon Analysis”; ASTM D7459: “Standard Practice for Collection of Integrated Samples for the Speciation of Biomass (Biogenic) and Fossil Carbon Dioxide Emitted from Stationary Emissions Sources”. |
| Monitoring frequency: | The frequency of measurement shall be as a minimum four times in year y with the mean value valid for year y |
| Source of data: | Monitoring by project participants |
| QA/QC procedures | - |
| Any comment: | - |

| Data / Parameter 33: | Qbiomass,y |
| --- | --- |
| Data unit: | t |
| Description: | Quantity of biomass fed into combustor for clinker production in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 34: | Qbiomass,AM,y |
| --- | --- |
| Data unit: | t |
| Description: | Quantity of biomass fed into combustor for alternative material production in year y |
| Measurement procedures (if any): | Measured by weight feeder which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 35: | DBiomass,CLNK,y |
| --- | --- |
| Data unit: | km |
| Description: | Distance between origin and destination for the transportation of residual biomass for clinker production in year y |
| Measurement procedures (if any): | Using digital records and transportation routes. |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | Monitoring by project participants |
| QA/QC procedures | - |
| Any comment: | - |

| Data / Parameter 36: | DBiomass,AM,y |
| --- | --- |
| Data unit: | km |
| Description: | Distance between origin and destination for the transportation of residual biomass for alternative material production in year y |
| Measurement procedures (if any): | Using digital records and transportation routes. |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | Monitoring by project participants |
| QA/QC procedures | - |
| Any comment: | - |

| Data / Parameter 37: | PELEgrid,AM,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of grid electricity consumption for alternative material production in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 38: | PELEsg,AM,y |
| --- | --- |
| Data unit: | MWh |
| Description: | Amount of self-generated electricity consumption for alternative material production in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

| Data / Parameter 39: | DCC,y |
| --- | --- |
| Data unit: | km |
| Description: | Distance between origin and destination for the transportation of residual biomass for alternative material production in year y |
| Measurement procedures (if any): | Measured by electricity meter which connects to the digital control system |
| Monitoring frequency: | Monitoring continuously and recorded at least monthly. |
| Source of data: | On-site measurements in plant records |
| QA/QC procedures | The calibration of the measuring equipment will be conducted as prescribed in ISO procedure by the manufacturer or certified third party. |
| Any comment: | - |

The sustainable development and good governance parameters monitored and determined ex post are identified as follows.

| Data / Parameter 40: | Renewable Electricity Consumption |
| --- | --- |
| Data unit: | MWh |
| Description: | Total quantity of renewable electricity consumed across the clinker, calcined clay, and cement blending processes. |
| Measurement procedures (if any): | Measured using digital power meters installed at the point of generation (solar farm) and at the main distribution boards of the production lines. Data is logged automatically. |
| Monitoring frequency: | Recorded in real-time; aggregated into monthly and annual reports. |
| Source of data: | Monitoring by project participants |
| QA/QC procedures | Meters are calibrated every 2 years or according to manufacturer specifications. |
| Any comment: | This parameter tracks progress toward SDG 7.2 by measuring the increase in the share of renewable energy within the project boundary. |

| Data / Parameter 41: | Limestone Input |
| --- | --- |
| Data unit: | Tons |
| Description: | The total mass of raw limestone utilized for clinker production |
| Measurement procedures (if any): | Measured by automated weight feeders on the belt conveyors leading to the cement grinding/blending mills. |
| Monitoring frequency: | Continuously recorded and aggregated monthly. |
| Source of data: | Monitoring by project participants |
| QA/QC procedures | Weigh scales are calibrated annually by the quality control department. |
| Any comment: | Monitors SDG 9.4 by demonstrating resource-use efficiency. Reduced limestone usage directly corresponds to a reduction in carbon-intensive cement. |

| Data / Parameter 42: | Quantity of Coal Consumed |
| --- | --- |
| Data unit: | Tons |
| Description: | Total quantity of coal used for thermal energy in the kiln/calciner systems in year y |
| Measurement procedures (if any): | Measured using weighing scales at the fuel feeding system and cross-referenced with fuel procurement and inventory data. |
| Monitoring frequency: | Continuously recorded daily; aggregated into monthly and annual consumption reports. |
| Source of data: | Monitoring by project participants |
| QA/QC procedures | Monthly physical stocktakes are compared against digital consumption records. |
| Any comment: | Directly monitors SDG 12.a (Sustainable Production). A declining trend in this parameter verifies the transition away from fossil fuel dependence in cement manufacturing. |

| Data / Parameter 43: | Human Rights Violations |
| --- | --- |
| Data unit: | Number of confirmed incidents |
| Description: | The number of confirmed human rights violations resulting directly from the operations of the MA in year y. This covers severe occupational health and safety (OHS) negligence, child labor, forced labor, discrimination, unfair compensation, and severe negative impacts on the local community's right to health and a safe environment. |
| Measurement procedures (if any): | Continuously monitor and aggregate all complaints flagged as potential human rights or severe safety violations through the established grievance mechanisms. |
| Monitoring frequency: | Annually |
| Source of data: | Internal grievance and whistle-blower logs |

# Part 5 Local Stakeholder Consultation

5.1 Stakeholders consultation process

The process of collecting stakeholder feedback was carried out in accordance with the guidelines set out in the “Guidelines for Stakeholder Engagement and Public Consultation for Projects Seeking Registration under the Premium Thailand Voluntary Emission Reduction Program (T-VER)” (Thailand Greenhouse Gas Management Organization, 2023). This approach aims to ensure transparency, disclosure of information, and to foster mutual understanding between the project developer and the local community. The full LSC report as required by TGO is available in Annex 14.

5.1.1 Preparation Prior to the Meeting

Step 1: Identify the scope of the affected area

The target area for stakeholder engagement activities of the project is defined as a 3-kilometer radius from the project boundary. This includes two facilities: Tha Luang Cement Plant, located in Village No. 9, Ban Khrua Subdistrict, Ban Mo District, Saraburi Province; and Khao Wong Cement Plant, located in Village No. 4, Khao Wong Subdistrict, Phra Phutthabat District, Saraburi Province

Step 2: Identify stakeholders and select project participants

The identification of all stakeholders and the selection of participants for the engagement process were carried out in consultation with landowners and the Subdistrict Administrative Organization to compile a list of stakeholders involved in the project. These include local residents, community leaders, government agencies, the private sector, and civil society organizations, among others.

Step 3: Plan and prepare for the meeting

A plan was developed and preparations were made for the stakeholder consultation meeting, with an emphasis on ensuring participatory implementation and alignment with the local community context. Initial preparation began with individual consultations with key target groups, including local administrative organizations, community leaders, and farmer groups, to gather relevant feedback and suggestions, as well as to disseminate preliminary project information. These consultations were conducted as part of the planning and preparation process.

Step 4: Prepare project information, publicize the meeting, and invite participants

The project notified stakeholders at least 10 days in advance, in accordance with the Thailand Voluntary Emission Reduction (T-VER) 2023 manual. Invitations, project brochures, and RSVP forms were sent out between May 7-8, 2025. Public relations signs were also posted in public areas within the communities 12 days prior to the meetings.

5.2 Summary of comments from stakeholders

The Objectives of the Meetings are listed as below

- To explain the project details.

- To listen to feedback from stakeholders.

- To build mutual understanding and mitigate potential impacts.

5.2.2 Date, time and place of public hearing

The stakeholder consultation meetings, held from May 21-23, 2025, had a total of 347 registered attendees (not including the project owner and consulting company) across the three days (excluding project owners and consultants).

- Citizens: 265 people

- Government Agencies/Organizations: 48 people

- Schools: 18 people

- Temples: 16 people

The meetings for the LC3 Cement Production Project (Premium T-VER) were divided into three forums:

- Forum 1: May 21, 2025 (09:00 - 12:00 hrs.) at the Ta Luang Cement Plant Training Center. Target group: Residents of Saraburi Province (Ban Khrua and Bang Khamot sub-districts, Ban Mo District; Roeng Rang sub-district, Sao Hai District).

- Forum 2: May 22, 2025 (09:00 - 12:00 hrs.) at the Ta Luang Cement Plant Training Center. Target group: Residents of Phra Nakhon Si Ayutthaya Province (Tha Luang and Champa sub-districts, Tha Ruea District).

- Forum 3: May 23, 2025 (16:00 - 18:00 hrs.) at the Wat Sub Bon Dome. Target group: Residents of Saraburi Province (Khao Wong, Huai Pa Wai, Khun Khlon sub-districts, Phra Phutthabat District; Hua Pluak sub-district, Sao Hai District; Na Phra Lan and Phu Khae sub-districts, Chaloem Phra Kiat District).

5.2.3 Summarize opinions received from questions or information that need to be heard.

During the meetings, several key issues were raised. The main topics included: the difference between traditional cement and LC3, the control of dust and air pollution (especially PM2.5), safety concerns regarding raw material transportation and its impact on the community, and the use of clay instead of traditional materials, as well as the sourcing of these raw materials. Employees from STL addressed these questions and concerns, providing detailed explanations and clarifying measures to alleviate the anxieties of the meeting attendees.

Based on the post-meeting evaluations, 91.6% of the feedback indicated that the project would be beneficial. These benefits included reducing GHG emissions, promoting economic growth and employment, and increasing learning resources on the environment. While 71.6% of attendees expressed no concerns, some still had worries about air pollution and its health impacts, as well as potential traffic and safety issues from transportation. To address these concerns, participants suggested increasing the frequency of environmental quality monitoring. In addition, attendees provided feedback on their awareness of the project. Most participants stated they were already familiar with the project and identified community leaders as the primary channel for receiving information. Cement plant staff and local government agencies were the next most common contact channel. This feedback highlights the crucial role that local individuals and organizations play in communicating and raising awareness about the project.

  - Corrective actions for issues identified in section 5.2

Following the consultation, the project considered the feedback and adjusted its operational approach in several key areas:

- Environmental impact: The operational plan now includes enhanced environmental mitigation measures, such as a systematic approach to air pollution management.

- Transportation: The project will consider adjusting transportation schedules to better align with the daily lives of residents, such as avoiding rush hour.

- Information dissemination: The format for distributing information will be revised to be more accessible to the public, using local media and clear, simple language.

- Communication: Additional locations for public relations signs will be designated within the community.

These revised approaches will be integrated into the project's operational plan and will serve as a framework for future monitoring and evaluation.

Additionally, stakeholders suggested that the project should create new channels for the community to access information and report problems. STL will utilise a comprehensive grievance mechanism to monitor stakeholder satisfaction and manage concerns. Dedicated channels, managed by relevant departments, provide stakeholders with direct access to report issues. The system will be supported by defined operational scopes and performance metrics, ensuring that the grievance management process is both efficient and transparent.

# Appendix 1 Supporting documents/evidence

| Annex 1 | Ex-ante mitigation outcomes calculation |
| --- | --- |
| Annex 2 | Colour Control Treatment Technology |
| Annex 3 | No double issuance declaration |
| Annex 4 | Draft of SCG Guideline to avoid premium claim |
| Annex 5 | The progress of greenhouse gas (GHG) emission reductions under Thailand’s NDC 2.0 |
| Annex 6 | Confirmation letter from DIW |
| Annex 7 | Feedback to MAIN_SCG Ta Luang Plant Cement LC3 Project DCCE |
| Annex 8 | Feedback to MAIN_SCG Ta Luang Plant Cement LC3 Project FOEN |
| Annex 9 | Investment analysis |
| Annex 10 | List of main equipment to be installed and modified |
| Annex 11 | Financial Cost |
| Annex 12 | Fossil Fuel Leakage Assessment |
| Annex 13 | Biomass Leakage Assessment |
| Annex 14 | LSC report of SCG Ta Luang Plant Cement LC3 Project Thailand |

https://www.bot.or.th/en/statistics/exchange-rate.html : 27 March 2026

Kiln No.1-4 were replace in accordance with the operational lifetime.

https://www.thaicma.or.th/en/environmentally/environment/environmentally_friendly_cement_products#:~:text=Hydraulic%20Cement%20is%20aimed%20to,to%20Paris%20Agreement%20(Thai%20Version)&text=hydraulic%20cement%20has%20quality%20controlling,properties%20have%20not%20been%20set.

https://www.ratchakitcha.soc.go.th/DATA/PDF/2556/E/065/5.PDF

https://ghgreduction.tgo.or.th/en/premium-t-ver-download/174-premium-t-ver-download-publication/3436-t-ver.html

https://www.dcce.go.th/wp-content/uploads/2025/08/English_NDC-Action-Plan-on-Mitigation_clean.pdf

https://faolex.fao.org/docs/pdf/tha209657.pdf

https://www.iso.org/standard/60857.html

https://www.dcce.go.th/wp-content/uploads/2025/08/International-Carbon-Credit-Guideline-1.pdf

https://www.bafu.admin.ch/en/bilateral-climate-agreements

https://unfccc.int/sites/default/files/resource/Thailand%20LT-LEDS%20(Revised%20Version)_08Nov2022.pdf

https://unfccc.int/sites/default/files/NDC/2022-11/Thailand%202nd%20Updated%20NDC.pdf

https://www.dcce.go.th/3317/#:~:text=%E0%B8%84%E0%B8%93%E0%B8%B0%E0%B8%A3%E0%B8%B1%E0%B8%90%E0%B8%A1%E0%B8%99%E0%B8%95%E0%B8%A3%E0%B8%B5%20%E0%B8%A1%E0%B8%B5%E0%B8%A1%E0%B8%95%E0%B8%B4%E0%B9%80%E0%B8%A1%E0%B8%B7%E0%B9%88%E0%B8%AD%E0%B8%A7%E0%B8%B1%E0%B8%99,%E0%B8%A8.%202573%20%E0%B8%8B%E0%B8%B6%E0%B9%88%E0%B8%87%E0%B9%81%E0%B8%9A%E0%B9%88%E0%B8%87%E0%B9%80%E0%B8%9B%E0%B9%87%E0%B8%99%E2%80%A6

https://www.tisi.go.th/data/news/139_282_1110166.pdf

https://www.dcce.go.th/wp-content/uploads/2025/08/International-Carbon-Credit-Guideline-1.pdf

https://tradingeconomics.com/commodity/coal

https://www.erc.or.th/th/automatic/

https://www.rd.go.th/english/14860.html, 5 May 2026

https://www.sciencedirect.com/topics/earth-and-planetary-sciences/terra-rossa

https://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/2_Volume2/V2_2_Ch2_Stationary_Combustion.pdf

https://www.dede.go.th/uploads/2566_forweb_Edit1_73a25948e1.pdf?updated_at=2025-04-04T06:40:58.778Z

https://www.ipcc-nggip.iges.or.jp/public/2006gl/pdf/2_Volume2/V2_1_Ch1_Introduction.pdf

https://oldwww.dede.go.th/ewt_news.php?nid=509&filename=index

https://thaicarbonlabel.tgo.or.th/index.php?lang=TH&mod=Y0hKdlpIVmpkSE5mWlcxcGMzTnBiMjQ9

https://thaitca.or.th/thaitca_journal/uploads/TCA_M-020104_การปลดปล่อยแก๊ซเรือนกระจก.pdf

https://ghgreduction.tgo.or.th/en/download/120-tver-gwp-emission-factor.html

https://ghgreduction.tgo.or.th/en/premium-t-ver-download/174-premium-t-ver-download-publication/3436-t-ver.html